3GPP TSG-SA WG2 Meeting #133	S2-1906158

13 - 17 May 2019, Reno, USA	rev. of 6106

Source:	ORANGE, Huawei

Title:	UE Communication Analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes additions for NWDAF “UE Communication Analytics” for analytics on UE groups, limitation of results, and optional detection of periodic communication

# Discussion

This discussion addresses the following topics:

- Addition of general input parameters in the §General

- communication analytics for UE groups.

It is difficult to provide detailed per-UE analytics results in the case of a large group (on the contrary it is easy to provide detailed analytics per-UE for a small group by applying multiple analytics requests, one per each UE). As a consequence, several modifications are introduced in order to facilitate cumulative values, with the idea that UE-group analytics shall be requested only for large groups.

As a consequence to the introduction of analytics for UE groups, is has been necessary to redefine the structure of results (with little impact in the case of a single UE).

- Limitation of results

For large UE groups or for UEs moving on large areas, it may be necessary to limit the size of results. This can be done by requesting results on a dedicated area, or by limiting the number of returned entries.

- Optional request on detection on periodic communication

The detection of periodic communication may be useful in some use cases, but it requires additional computation. Therefore it is proposed to offer it as an optional parameter. The possibility to request subsets of analytics in the Analytics Filter Information is stated in §6.1.3.

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

### 6.1.3	Contents of Analytics Exposure

The NFs/AFs/ OAM subscribing to (using Nnwdaf_ AnalyticsSubscription) or requesting (using Nnwdaf_AnalyticsInfo) analytics to NWDAF may provide:

-	Analytics ID: one or multiple Analytics ID(s)

NOTE `1:	Analytics ID(s) are also used by AFs subscribing to analytics information via NEF. NEF controls the mapping of Analytics IDs allowed to be exposed to AFs. Details in clauses 6.1.1.2 and 6.1.2.2.

-	Analytics Filter Information. This set of parameter types and values enables to select which type of analytics information is requested (e.g. subset of all available analytics produced by NWDAF for the given Analytics ID value) and may in addition indicate a maximum number of results expected.

-	Target of Analytics Reporting: the object targeted by analytics. The target of analytics reporting indicates entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	Parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	Observation period: time interval [start..end], either in the past or in the future, expressed as positive or negative offsets to the present. An interval in the past is a request for statistics. An interval in the future is a request for predictions. A default value means statistics in the recent past, with a time range defined by the NWDAF.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	Time when analytics are needed (if applicable).

NOTE 2:	The feasibility of the parameter "Time when analytics are needed" will be checked by stage 3.

The NWDAF provides analytics feedback to NFs/AFs/ OAM by notifying them (using Nnwdaf_AnalyticsSubscription service) or by responding to analytics requests (using Nnwdaf_AnalyticsInfo service) to analytics with following information:

-	The Notification Correlation Information provided in the subscription request.

-	The Analytics ID(s),

-	Related list of analytics on the requested observation period;

-	Optional additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: level of certainty, degree of confidence in statistics/prediction.

* * * Second change * * * *

## 6.11	UE Communication Analytics

### 6.11.1	General

In order to support some optimized operations, e.g. customized mobility management or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF and SMF that corresponds to this application ID.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytic Filter Information containing optional list of analytics subset(s)

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high)

### 6.11.2	Input Data

The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.11.2-1.

Table 6.11.2-1: Service Data from AF related to UE communication

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Correlation information | Identifying the application generating the traffic |
| Application ID | Identifying the application providing this information |
| Communication pattern parameters | Same as communication pattern specified in TS 23.502 |
| UE communication (1..max) | Communication description per application |
| >Communication start | The time stamp that this communication starts |
| >Communication stop | The time stamp that this communication stops |
| >UL data rate | UL data rate of this communication expected by the application |
| >DL data rate | DL data rate of this communication expected by the application |

The correlation information may be used to correlate the communication information collected from an AF with the communication information collected from SMF(s). Other correlations can be made within the NWDAF using UE communication time pattern.

Editor's Note:	 the exact nature of correlation information (e.g. IP address 5-tuple) is FFS.

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE communication information).

### 6.11.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF includes the UE communication statistics or predictions.

UE communication statistics are defined in the Table 6.11.3-1.

Table 6.11.3-1: UE Communication Statistics

| Information |  | Description |
| --- | --- | --- |
| UE group ID or UE ID |  | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. |
| UE communications (1..max) |  | List of communication time slots. |
| > Periodic communication indicator |  | Identifies whether the UE communicates periodically or not. |
| > Periodic time |  | Interval Time of periodic communication (average and variance) if periodic. Example: every hour. |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication (average and variance). Example: Time: 13:00-20:00, Day: Monday. |
| > Start time | Start time observed (average and variance) |
| > Duration time |  | Duration interval time of communication (average and variance). |
| > Traffic characterization |  | DNN, ports, other useful information. |
| > Traffic volume |  | Volume UL/DL (average and variance). |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

UE communication predictions are defined in the Table 6.11.3-2.

Table 6.11.3-2: UE Communication Predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (average and variance) if periodic. Example: every hour. |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication. Example: Time: 13:00-20:00, Day: Monday. |
| > Start time | Start time predicted (average and variance). |
| > Communication duration time | Duration interval time of communication . |
| > Traffic characterization | DNN, ports, other useful information. |
| > Traffic volume | Volume UL/DL (average and variance). |
| > Confidence | Confidence of the prediction. |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group for a given communication  at a given time and duration.

The number of UE communication entries (1..max) is limited by the maximum number of results provided as input parameter. The communications shall be provided by order of time, possibly overlapping.

Depending on the list size limitation, the least probable communications on a given observation period may not be provided

The analytics subset Periodic communication information shall be provided only if the parameter Periodicity Detection was requested.

* * * End of changes * * * *