3GPP TSG-SA WG2 Meeting #133	S2-1906794

May 13 – 17, 2019, Reno, Nevada, US 	(e-mail revision 1 of S2-1906156)

Source:	Huawei, HiSilicon,

Title:	Adding Information for enabling proper triggering of NWDAF Data Collection

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes the definition of the information associated with the network map for data collection, configured at NWDAF and how it is used to trigger the data collection from different NF sources.

# Discussion

Observation 1: TS 23.288 defines in Clause 6.2.2.4 the following:

“The information provided by the NRF to the NWDAF with the Nnrf_NFDiscovery_Request and the Nnrf_NFManagement_NFStatusNotify operations are the NFProfile and the NFService. Such information can be used to set-up and maintain a consistent network map for data collection and also, depending on use cases, in order to perform estimations:”

Problem: It is not defined in TS 23.288 which information qualifies the network map for data collection and how this network map for data collection is used by NWDAF.

Proposed Solution:

- Definition of the information associated with the network map for data collection, configured at NWDAF and how it is used to trigger the data collection from different NF sources.

Observation 2: TS 23.288 defines in Clause 6.6.4 the following FFS:

Editor's note:	It is FFS based on which information the NWDAF knows which NF to ask NRF.

Proposed Changes in TS 23.288:

- Update Clause 6.2.1 with the information related to the network map for data collection and how NWDAF uses such information for triggering data collection.

# Proposal

It is proposed to update Clause 6.2.1 with the information related to the network map for data collection and how NWDAF uses such information for triggering data collection.

**** First Change ****

## 6.2	Procedures for Data Collection

### 6.2.1	General

The Data Collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF), as a basis of the computation of network analytics.

All available data encompass:

-	OAM global NF data,

-	behaviour data related to individual UEs or UE groups (e.g. UE reachability), and pre-computed metrics covering UE populations (e.g. number of UEs present in a geographical area), per spatial and temporal dimensions (e.g. per region for a period of time),

-	other NF data available in the 5GC (e.g. NRF)

The NWDAF shall use at least one of the following services:

-	the Generic management services as defined in TS 28.532 [6] offered by OAM in order to collect OAM global NF data.

-	the Exposure services offered by NFs/AFs in order to retrieve behaviour data and other non-OAM pre-computed metrics.

-	Other NF services in order to collect NF data (e.g. NRF)

The NWDAF shall obtain the proper information to perform data collection for a UE or group of UEs:

-	For an analytics ID, NWDAF is configured with the corresponding NF Type(s) and/or event ID(s) and/or OAM measurement types.

-	NWDAF shall determine which NF instance(s) of the relevant NF type(s) are serving the UE or group of UEs, optionally taking into account the S-NSSAI(s) and area of interest as defined in clause 7.1.3, TS 23.501 [2].

-	NWDAF invokes Nnf_EventSubscribe services to collect data from the determined NF instance(s), and/or triggers the procedure in clause 6.2.3.3 to subscribe to OAM services to collect the OAM measurement.

The NWDAF shall be able to discover the metrics supported by a NF/AF.

Data collection procedures enables the NWDAF to efficiently obtain the appropriate data with the appropriate granularity.

When a request or subscription for statistics or predictions is received, the NWDAF may not possess the necessary data to perform the service.

-	Data on the monitoring period in the past matching the observation period is necessary for the provision of statistics and predictions.

-	Data on longer monitoring periods in the past is necessary for model training.

Therefore, in order to optimize the service quality, the NWDAF may undertake the following actions:

-	The NWDAF may return a probability assertion as stated in 6.1.3 expressing the confidence in the analytics produced. With zero confidence, no analytics shall be returned. This confidence is likely to grow in the case of subscriptions.

-	The value of the confidence depends on the level or urgency expressed by the time deadline as stated in 6.1.3, the level of accuracy, the availability of data. If no sufficient data is collected to provide an estimation for the requested level of accuracy before the time deadline, the service shall return a zero confidence. Otherwise, the NWDAF may wait until enough data is collected before providing a response or a first notification.

-	In order to be prepared for future requests on statistics from NFs/AFs/OAM, the NWDAF, upon operator configuration, may collect data on its own initiative, e.g. on samples of UEs (e.g. mobility), and retain the data collected in the data storage.

The volume and maximum duration of data storage is also subject of operator configuration.

The NWDAF may decide to reduce the amount of data collection in case of high signalling load, by either prioritizing requests, reducing the duration of data collection, or the sampling ratios.

*************** End of changes ***************