3GPP TSG-SA2 Meeting #133	S2-1904937

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1283 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interaction between MICO mode with active time and eDRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to further optimize the power consumption during the active time for MICO mode is running, the UE may also request to use eDRX, same as PSM with active time and eDRX in LTE as specified in subclause 4.5.13.1 of TS23.682 and subclause 5.3.13 of TS24.301. When the UE requests the use of both MICO mode with active time and eDRX during a registration procedure, it is up to the network to decide to enable none, one of them or both. If the network accepts the use of both MICO mode with active time and extended idle mode DRX, the extended idle mode DRX cycle should be set in order to have multiple paging occasions while the active timer is running. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Interaction between active time for MICO mode and eDRX. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No guidance on network behavoir when the UE requests MICO mode with active time and eDRX during a registration procedure. when the network accepts the use of both MICO mode with active time and extended idle mode DRX, if the extended DRX parameters IE provided to the UE allows no paging occasions before the active timer expires, the UE will get no paging from the network while the active time is running. The active time is negotiated and started in vain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

5.31.7.1	General

To enable UE power saving and to enhance MT reachability while using MICO mode, e.g. for CIoT, the following features are specified in the following clauses:

-	Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE; and

-	MICO mode with Extended Connected Time; and

-	MICO mode with Active Time; and

-	MICO mode and Periodic Registration Timer Control.

If a UE requests via NAS both to enable MICO mode with Active Time and extended idle mode DRX, the AMF may decide to:

1.	Enable only MICO mode with Active Time, i.e. not accept the request for extended idle mode DRX;

2.	Enable only extended idle mode DRX, i.e. not accept the request for an active time; or

3.	Enable both MICO mode with Active Time and extended idle mode DRX.

The decision between the three above, and which active time, periodic registration update timer and/or extended idle mode DRX cycle value to provide to the UE, are implementation dependent, based on local configuration, and possibly other information available in the AMF. The method selected is then used until the next Registration procedure is initiated, when a new decision may be made. If both extended idle mode DRX and MICO mode with Active Time are enabled, the extended idle mode DRX cycle should be set in order to have multiple paging occasions while the active timer is running.

***** Next change *****