3GPP TSG-SA2 Meeting #133	S2-1904938

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1284 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on service gap control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are several conflicting requirements for service gap control, specifically speaking, whether the UE can send MO user data and MO signaling in CM-CONNECTED state. There are some requirements specifying the service gap control does not impact UE Mobile Originated user data transmission or Mobile Originated signaling in CM-CONNECTED state, for example: A1: The Service Gap control is applied in CM-IDLE state only and does not impact UE Mobile Originated user data transmission or Mobile Originated signaling in CM-CONNECTED state. A2: The following procedures are impacted by Service Gap Control: - Registration Procedure, see TS 23.502 [3] clause 4.2.2.2; - UE Triggered Service Request, see TS 23.502 [3] clause 4.2.3.2; NOTE 5: Since UE triggered Service Request is prevented by Service Gap timer, this implicitly prevents the UE from initiating UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation (see TS 23.502 [3] clause 4.24.1), NEF Anchored Mobile Originated Data Transport (see TS 23.502 [3] clause 4.25.4) and MO SMS over NAS in CM-IDLE (see TS 23.502 [3] clause 4.13.3.3). There are also requirements implying the service gap control will impact UE Mobile Originated user data transmission or Mobile Originated signaling in CM-CONNECTED state, for example: B1: The UE shall not initiate PDU Session Establishment Requests when a Service Gap timer is running, unless it is for Emergency services which are allowed. (Please note that the UE can only send PDU Session Establishment Requests in CM-CONNECTED state) B2: The AMF may enforce the Service Gap timer by not allowing SM signalling after Initial Registration requests when a Service Gap timer is running unless access to the network for regulatory prioritized services like Emergency services is required which are allowed. (Please note SM signalling can only be send in CM-CONNECTED state) B3: When rejecting the connection requests and the SM signalling after Initial Registration Requests while the Service Gap timer is running, the AMF may include a Mobility Management back-off timer corresponding to the time left of the current Service Gap timer. (Please note SM signalling can only be send in CM-CONNECTED state) In order to correct the above misalignment, this contribution proposes to correct the requirements in above B1–B3, to align with the requirement that The Service Gap control is applied in CM-IDLE state only and does not impact UE Mobile Originated user data transmission or Mobile Originated signaling in CM-CONNECTED state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct misalingment of requirement of service gap control, including: 1. change “PDU Session Establishment Requests” to ‘Connection request’. 2. change “SM signaling” to ‘Connection request’ in two places. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conflicting requirements on service gap control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

5.31.16	Service Gap Control

Service Gap Control is an optional feature intended for CIoT UEs to control the frequency at which these UEs can access the network. That is, to ensure a minimum time gap between consecutive Mobile Originated data communications initiated by the UE. This helps reducing peak load situations when there are a large number of these UEs in an operator network. Service Gap Control is intended to be used for "small data allowance plans" for MTC/CIoT UEs where the applications are tolerant to service latency.

NOTE 1:	Time critical applications, such as regulatory prioritized services like Emergency services can suffer from the latency caused by the Service Gap Control feature. Therefore Service Gap Control feature is not recommended for subscriptions with such applications and services.

Service Gap Time is a subscription parameter used to set the Service Gap timer and is enforced in the UE and in the AMF on a per UE level (i.e. the same Service Gap Timer applies for all PDU Sessions that the UE has). The UE indicates its capability of support for Service Gap Control in the Registration Request message to the AMF. The AMF passes the Service Gap Time to the UE in the Registration Accept message for UE that has indicated its support of the Service Gap Control. The Service Gap Control shall be applied in a UE when a Service Gap Time is stored in the UE context and applied in the AMF when the Service Gap Time is stored in the UE Context in AMF.

Service Gap Control requires the UE to stay in CM-IDLE mode for at least the whole duration of the Service Gap timer before triggering Mobile Originated user data transmission, except for procedures that are exempted (see TS 24.501 [47]). The Service Gap timer shall be started each time a UE moves from CM-CONNECTED to CM-IDLE, unless the connection request was initiated by the paging of a Mobile Terminated event, or after a Mobility or Periodic Registration procedure without Uplink data status, which shall not trigger a new or extended Service Gap interval. When a Service Gap timer expires, the UE is allowed to send a connection request again. If the UE does so, the Service Gap timer will be restarted at the next CM-CONNECTED to CM-IDLE transition.

The Service Gap control is applied in CM-IDLE state only and does not impact UE Mobile Originated user data transmission or Mobile Originated signaling in CM-CONNECTED state. The Service Gap timer is not stopped upon CM-IDLE state to CM-CONNECTED state transition. The UE shall not initiate connection requests for MO user plane data, MO control plane data, or MO SMS when a Service Gap timer is running. The UE shall not initiate connection requests when a Service Gap timer is running, unless it is for Emergency services which are allowed. CM-CONNECTED with RRC_INACTIVE is not used for UEs that have a Service Gap Time configured.

NOTE 2:	As a consequence of allowing Initial Registration Request procedure, the UE with a running Service Gap timer does not initiate further MO signalling, except for Mobility Registration procedure, until the UE receives MT signalling or after the UE has moved to CM-IDLE state and the Service Gap Timer is not running.

NOTE 3:	Implementations need to make sure that latest and up-to-date data are always sent when a Service Gap timer expires.

The AMF may enforce the Service Gap timer by rejecting connection request for MO user plane data, MO control plane data, or MO SMS when a Service Gap timer is running. The AMF may enforce the Service Gap timer by not allowing connection requests after Initial Registration requests when a Service Gap timer is running unless access to the network for regulatory prioritized services like Emergency services is required which are allowed. When rejecting the connection requests after Initial Registration Requests while the Service Gap timer is running, the AMF may include a Mobility Management back-off timer corresponding to the time left of the current Service Gap timer. For the UEs that does not support Service Gap Control (e.g. pre-release-16 UEs), Service Gap Control may be enforced using "General NAS level congestion control" as defined in clause 5.19.7.2.

When the AMF starts the Service Gap timer, the AMF should invoke the Service Gap timer with a value that is slightly shorter than the Service Gap Time value provided to the UE in the subscription information received from the UDM.

NOTE 4:	This ensures that the AMF doesn't reject any UE requests just before the Service Gap timer expires e.g. because of slightly unsynchronized timers between UE and AMF.

A UE which transitions from a MICO mode or eDRX power saving state shall apply Service Gap Control when it wakes up if the Service Gap timer is still running.

Additional aspects of Service Gap Control:

-	Service Gap Control applies in all PLMNs.

-	When the Service Gap timer is running and the UE receives paging, the UE shall respond as normal.

-	Service Gap Control does not apply to exception reporting for NB-IoT.

-	Access to the network for regulatory prioritized services like Emergency services are allowed when a Service Gap timer is running.

-	Service Gap Control shall be effective also for UEs performing de-registration and re-registration unless access to the network for regulatory prioritized services like Emergency services is required.

-	If the Service Gap timer is running, the Service Gap is applied at PLMN selection as follows:

a)	Re-registration to the registered PLMN: The remaining Service Gap timer value survives.

b)	Registration to a different PLMN: The remaining Service Gap timer value survives.

c)	USIM swap: The Service Gap timer is no longer running and the Service Gap feature does not apply, unless re-instantiated by the serving PLMN.

-	Multiple uplink packets and downlink packets are allowed during one RRC connection for UE operating within its Rate Control limits.

The following procedures are impacted by Service Gap Control:

-	Registration Procedure, see TS 23.502 [3] clause 4.2.2.2;

-	UE Triggered Service Request, see TS 23.502 [3] clause 4.2.3.2;

NOTE 5:	Since UE triggered Service Request is prevented by Service Gap timer, this implicitly prevents the UE from initiating UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation (see TS 23.502 [3] clause 4.24.1), NEF Anchored Mobile Originated Data Transport (see TS 23.502 [3] clause 4.25.4) and MO SMS over NAS in CM-IDLE (see TS 23.502 [3] clause 4.13.3.3).

***** Next change *****