SA WG2 Meeting #133 	S2-1905087

# Reno, Nevada, USA, 13-17 May 2019												(rev of )

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1333 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Emergency session handling at EPS to 5GC/N3IWF handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-05-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The handling of emergency session during the following handover scenarios is not specified: - EPC/ePDG to 5GS handover - EPS to 5GC/N3IWF handover - EPS to 5GS mobility without N26 The following are pproposed for tp address the above handover scenarios: [Proposal-1] Add Emergency Information in UE Subscription data type “UE context in SMF data” [Proposal-2] UDM notifies also the Emergency-Information containing FQDN of PGW-C to the AMF [Proposal-3] When the emergency session is handed over to 5GC (i.e. PDU Session establishment with request type “Existing Emergency PDU Session” is received in the AMF), the AMF uses Emergency Info (provided by UDM) to locate the PGW-C+SMF. Discussion paper S2-1905083 may be referenced for more information. This CR addresses scenario of EPS to 5GC/N3IWF handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.11.3.1 Add description how AMF locate the PGW-C+SMF for emergency session handover from EPS to 5GC/N3IWF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPS to 5GC/N3IWF handover failure for emergeny session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on the update in 23.502CR1331 (S2-1905085) for the update in clauses 5.2.3.3.1 and 5.2.3.3.3. |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.11.3.1	Handover from EPS to 5GC-N3IWF

Figure 4.11.3.1-1: Handover from EPS to 5GC-N3IWF

0.	Initial status: one or more PDN connections have been established in EPC between the 5G capable UE and the PGW via E-UTRAN.

1.	The UE initiates Registration procedure on untrusted non-3GPP access via N3IWF (with 5G-GUTI is available or SUCI if not) per clause 4.12.2.

2.	The UE initiates a UE requested PDU Session Establishment with Existing PDU Session indication in 5GC via Untrusted non-3GPP Access via N3IWF per clause 4.12.5.

If the Request Type indicates "Existing Emergency PDU Session", the AMF shall use the Emergency Information received from the HSS+UDM which contains PGW-C+SMF FQDN for S5/S8 interface for the emergency PDN connection established in EPS.

The combined PGW+SMF/UPF initiates a PDN GW initiated bearer deactivation as described in TS 23.401 [13] clause 5.4.4.1 to release the EPC and E-UTRAN resources.

END OF CHANGES