3GPP TSG-SA WG2 Meeting #133	                                                                                               S2-1906105

13 – 17 May Reno, Nevada , US

Source:                         Ericsson, CATT, Huawei

Title:                              Combine clause 6.9 MICO Mode Parameters related analytics with clause 6.11 UE Communication Analytics

Document for:              Approval

Agenda Item:                6.11

Work Item / Release:    eNA / Rel-16

Abstract of the contribution: This contribution argues that MICO mode analytics is already covered by more general analytics. And therefore suggests to combine clause 6.9 MICO Mode Parameters related analytics with clause 6.11 UE Communication Analytics.

# Discussion

In last SA2 meeting the work of consolidation of UE analytics started. All analytics involving UE behaviour shall reference to chapter 6.10	 UE mobility analytics and 6.11	UE Communication Analytics. This was done in the approved tdoc S2-1903977 Expected UE behavioural parameters related network data analytics.

The exact output analytics for MICO mode is still FFS. But the current version of 23.501 suggests that if NWDAF is deployed, the AMF may also use analytics on expected UE behaviour and/or UE communication pattern generated by NWDAF (see TS 23.288 [86]) to decide MICO mode parameters. However, the current version of 23.288 has concluded (in clause 6.7) that the NWDAF consumers get the Expected UE Behaviour by requesting an Analytics ID of UE Mobility analytics and/or UE Communication analytics. It is, therefore, expected that the output of the UE Mobility analytics includes the parameters defined in clause 6.10.3; similarly, the output of UE Communication analytics should include the parameters defined in clause 6.11.3. It seems reasonable to conclude that the MICO mode analytics is effectively a subset of the UE Communication analytics. As such, there is no need to define a new Analytics ID for requesting this type of analytics.

The following table shows the data collection for MICO Mode Parameters related analytics

Table 6.9.2-1: Network data collection for the  MICO Mode Parameters related analytics

| Information | Source | Description |
| --- | --- | --- |
| Location Info | AMF | The location information of the UE. |
| DNN | SMF or PCF | To identify the DNN |
| S-NSSAI | SMF or AMF | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| UE ID | AMF | To identify UE |
| Internal Group ID | SMF | To identify UE group if available |
| Traffic usage report | UPF or SMF | To extract traffic characteristics such as flow length, size, and inter packet arrival time. |
| Expected UE Behaviour parameters | UDR/UDM | To extract the Expected UE Behaviour parameters. |
| Service experience | AF | User or service provide feedback on service experience such as MOS. |

From the above list Location Info is covered by the input described in clause 6.10

Table 6.10.2-1: UE location information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |

From the above list traffic usage and Expected UE behavior are covered by the input described in clause 6.11

Table 6.11.2-1: Service Data from AF related to UE communication

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Correlation information | Identifying the application generating the traffic |
| Application ID | Identifying the application providing this information |
| Communication pattern parameters | Same as communication pattern specified in TS 23.502 |
| UE communication (1..max) | Communication description per application |
| >Communication start | The time stamp that this communication starts |
| >Communication stop | The time stamp that this communication stops |
| >UL data rate | UL data rate of this communication expected by the application |
| >DL data rate | DL data rate of this communication expected by the application |

And the communication pattern are further described in 23.502 according to below table.

Table 4.15.6.3-1: Description of Expected UE Behaviour parameters

| Expected UE Behaviour parameter | Description |
| --- | --- |
| Expected UE Moving Trajectory | Identifies the UE's expected geographical movement Example: A planned path of movement |
| Stationary Indication | Identifies whether the UE is stationary or mobile [optional] |
| Communication Duration Time | Indicates for how long the UE will normally stay in CM-Connected for data transmission. Example: 5 minutes. [optional] |
| Periodic Time | Interval Time of periodic communication Example: every hour. [optional] |
| Scheduled Communication Time | Time and day of the week when the UE is available for communication. Example: Time: 13:00-20:00, Day: Monday. [optional] |
| Battery Indication | Identifies power consumption criticality for the UE: if the UE is battery powered with not rechargeable/not replaceable battery, battery powered with rechargeable/replaceable battery, or not battery powered. [optional] |
| Traffic Profile | Identifies the type of data transmission: single packet transmission (UL or DL), dual packet transmission (UL with subsequent DL or DL with subsequent UL), multiple packets transmission [optional] |
| Scheduled Communication Type | Indicates that the Scheduled Communication Type is Downlink only or Uplink only or Bi-directional [To be used together with Scheduled Communication Time] Example: <Scheduled Communication Time>, DL only. [optional] |

So, in summary Location Info, traffic usage and Expected UE behavior and are covered already as input data for Expected UE behavior.

The rest of the input parameters are brought into table Table 6.7.2-2: UE behavioural information collected from 5GC NF(s).

Observe if Service experience is used for MICO mode decisions this can be retrieved by AMF via clause 6.4 Observed Service experience related network data analytics.

In conclusion, the MICO mode analytics can utilize more general, already available analytics. Therefore, clause 6.9 MICO Mode Parameters related analytics can be combined with clause 6.11.

The Editor’s Note in the Output chapter is also moved the Output chapter in 6.11

# Proposal

Combine clause 6.9 MICO Mode Parameters related analytics with clause 6.7.

**** First Change ****

**** End First Change ****

**** Second Change ****

## 6.11	UE Communication Analytics

### 6.11.1	General

In order to support some optimized operations, e.g. customized mobility management or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF and SMF that corresponds to this application ID.

### 6.11.2	Input Data

The NWDAF supporting data analytic on UE communication shall be able to collect communication information for the UE from AFs or SMFs. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.11.2-1.

Table 6.11.2-1: Service Data from AF related to UE communication

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Internal Group ID | To identify UE group if available |
| Correlation information | Identifying the application generating the traffic |
| Application ID | Identifying the application providing this information |
| Communication pattern parameters | Same as communication pattern specified in TS 23.502 |
| UE communication (1..max) | Communication description per application |
| >Communication start | The time stamp that this communication starts |
| >Communication stop | The time stamp that this communication stops |
| >UL data rate | UL data rate of this communication expected by the application |
| >DL data rate | DL data rate of this communication expected by the application |

The correlation information may be used to correlate the communication information collected from an AF with the communication information collected from SMF(s). Other correlations can be made within the NWDAF using UE communication time pattern.

Editor's Note:	the exact nature of correlation information (e.g. IP address 5-tuple) is FFS.

### 6.11.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF includes the UE communication statistics or predictions as defined in the Table 6.11.3-1.

Table 6.11.3-1: UE Communication Analytics

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID, i.e., SUPI |
| UE communications (1..max) |  |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (may be used together with 1). Example: every hour |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication. Example: Time: 13:00-20:00, Day: Monday |
| > Communication duration time | Duration interval time of periodic communication or the scheduled communication (may be used together with 1). Example: 5 minutes |
| > Traffic characterization | Estimated UL/DL traffic volume |
| > Traffic volume | Volume UL/DL |
| > Confidence | Confidence of the prediction if this is a communication pattern prediction |

**** End Second Change ****