SA WG2 Meeting #133	S2-1906244

May 13 – 17, 2019, Reno, Nevada, US	(revision of S2-1905002)

Source:	Ericsson

Title:	Move of 5G-RG procedures

Document for:	Approval

Agenda Item:	6.7

Work Item / Release:	5WWC/Rel-16

Abstract of the contribution: This CR proposes to move some procedures in 23.316.

# Introduction

This contribution proposes to move the “5G-RG Context Release procedure in the W-AGF” to “AN release” clause. It also corrects terminology for FCP vs W-CP/UP (changes-on-changes used). Furthermore, it proposes to move clause “CN-initiated selective deactivation of UP connection of an existing PDU Session associated with W-5GAN Access” to 7.3 session management procedures.

# Proposal

It is proposed to update TS 23.316 as follows:

**** First Change ****

**** Second Change ****

### 7.2.5	AN Release

Editor’s note: AN Release procedure for FN-RG is FFS

The AN Release procedure via W-5GAN access is used by the W-5GAN or the AMF to release the logical NG-AP signalling connection and the associated N3 User Plane connections between the W-5GAN and the 5GC.

The procedure will move the 5G-RG from CM-CONNECTED to CM-IDLE in AMF, and all 5G-RG related context information is deleted in the W-AGF.

Both W-AGF initiated and AMF-initiated AN release in the W-AGF procedures are shown in Figure 7.2.5-1.

Figure 7.2.5-1: 5G-RG AN release in the W-AGF

1.	The 5G-RG has already registered in the 5GC and may have established one or multiple PDU Sessions.

2.	The W-AGF detects that the 5G-RG is not reachable.

3.	The W-AGF sends a N2 UE Context Release Request message to the AMF This step is equivalent to step 1b of Figure 4.2.6-1.

NOTE:	AN Release procedure can also be triggered by an AMF internal event and in that case step 2 and step 3 do not take place.

4.	AMF to W-AGF: If the AMF receives the N2 UE Context Release Request from W-AGF or if due to an internal AMF event the AMF wants to release N2 signalling, the AMF sends an N2 UE Context Release Command (Cause) to the W-AGF. The cause indicated is cause from step 3 or a cause due to internal AMF event. This step is equivalent to step 2 of Figure 4.2.6-1.

5.	If the W-CP signalling connection and W-UP resources has not been released yet, the W-AGF releases the W-CP connection and W-UP resources with a procedure out of scope of 3GPP. The W-AGF sends to the 5G-RG the indication of the release reason if received in step 4.

7.	W-AGF to AMF: The W-AGF confirms the release of the 5G-RG-associated N2-logical connection by returning N2 UE Release Complete (list of PDU Session ID(s) with active N3 user plane) to the AMF as in step 4 defined in clause 4.2.6. The AMF marks the 5G-RG as CM-IDLE state in non-3GPP access.

8.	For each of the PDU Sessions in the N2 UE Context Release Complete, the steps 5 to 7 in TS 23.502 [3] clause 4.2.6 are performed (PDU Session Update SM Context). After the AMF receives the Nsmf_PDUSession_UpdateSMContext Response as in step 7 of TS 23.502 [3] clause 4.2.6, the AMF considers the N3 connection as released. If list of PDU Session ID(s) with active N3 user plane is included in step 3, then this step is performed before step 4.

**** Third Change ****

#### 7.3.X	CN-initiated selective deactivation of UP connection of an existing PDU Session associated with W-5GAN Access

The procedure described in TS 23.502 [3] clause 4.3.7 (CN-initiated selective deactivation of UP connection of an existing PDU Session) is used for CN-initiated selective deactivation of UP connection for an established PDU Session associated with W-5GAN Access of a 5G-RG in CM-CONNECTED state, with the following exceptions:

-	The NG-RAN corresponds to a W-AGF.

-	The user plane resource between the UE and W-AGF, is released not with RRC signalling but with procedure outside scope of 3GPP.

**** End of Changes ****