3GPP TSG-SA2 Meeting #134	S2-1907262

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1541 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing CIoT services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  |  | Date: |  | 2019-06-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The services specified in TS 23.502 as part of 5G_CIoT are missing in TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds the following services to clause 7.2 with references to TS 23.502: - Nsmf_NIDD - Nudm_ NIDDAuthorisation - Nnef_MSISDN-less_MO_SMS - Nnef_NIDDConfiguration - Nnef_NIDD - Nnef_SMContext - Nnef_NetworkStatus - Ninef_EventExposure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and misaligned specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3, 7.2.5, 7.2.8, 7.2.8x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

### 7.2.3	SMF Services

The following NF services are specified for SMF:

Table 7.2.3-1: NF Services provided by SMF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nsmf_PDUSession | This service manages the PDU Sessions and uses the policy and charging rules received from the PCF. The service operations exposed by this NF service allows the consumer NFs to handle the PDU Sessions. | 5.2.8.2 |
| Nsmf_EventExposure | This service exposes the events happening on the PDU Sessions to the consumer NFs. | 5.2.8.3 |
| Nsmf_NIDD | This service is used for NIDD transfer between SMF and another NF. | 5.2.8.4 |

Next Change

### 7.2.5	UDM Services

The following NF services are specified for UDM:

Table 7.2.5-1: NF Services provided by UDM

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nudm_UECM | 1. Provide the NF consumer of the information related to UE's transaction information, e.g. UE's serving NF identifier, UE status, etc. 2. Allow the NF consumer to register and deregister its information for the serving UE in the UDM. 3. Allow the NF consumer to update some UE context information in the UDM. | 5.2.3.2 |
| Nudm_SDM | 1. Allow NF consumer to retrieve user subscription data when necessary. 2. Provide updated user subscriber data to the subscribed NF consumer. | 5.2.3.3 |
| Nudm_UEAuthentication | 1. Provide updated authentication related subscriber data to the subscribed NF consumer. 2. For AKA based authentication, this operation can be also used to recover from security context synchronization failure situations. 3. Used for being informed about the result of an authentication procedure with a UE. | 5.2.3.4 |
| Nudm_EventExposure | 1. Allow NF consumer to subscribe to receive an event. 2. Provide monitoring indication of the event to the subscribed NF consumer. | 5.2.3.5 |
| Nudm_ParameterProvision | 1. To provision information which can be used for the UE in 5GS. | 5.2.3.6 |
| Nudm_ NIDDAuthorisation | 1. To authorise an NIDD configuration request for the received External Group Identifier or GPSI. | 5.2.3.7 |

Next Change

### 7.2.8	NEF Services

The following NF services are specified for NEF:

Table 7.2.8-1: NF Services provided by NEF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nnef_EventExposure | Provides support for event exposure. | 5.2.6.2 |
| Nnef_PFDManagement | Provides support for PFDs management. | 5.2.6.3 |
| Nnef_ParameterProvision | Provides support to provision information which can be used for the UE in 5GS. | 5.2.6.4 |
| Nnef_Trigger | Provides support for device triggering | 5.2.6.5 |
| Nnef_BDTPNegotiation | Provides support for background data transfer policy negotiation and optionally notification for the renegotiation. | 5.2.6.6 |
| Nnef_TrafficInfluence | Provide the ability to influence traffic routing. | 5.2.6.7 |
| Nnef_ChargeableParty | Requests to become the chargeable party for a data session for a UE. | 5.2.6.8 |
| Nnef_AFsessionWithQoS | Requests the network to provide a specific QoS for an AS session. | 5.2.6.9 |
| Nnef_MSISDN-less_MO_SMS | Used by the NEF to send MSISDN-less MO SM to the AF. | 5.2.6.10 |
| Nnef_ServiceParameter | Provides support to provision service specific information. | 5.2.6.11 |
| Nnef_APISupportCapability | Provides support for awareness on availability or expected level of a service API. | 5.2.6.12 |
| Nnef_NIDDConfiguration | Used for configuring necessary information for data delivery via the NIDD API. | 5.2.6.13 |
| Nnef_NIDD | Used for NEF anchored MO and MT unstructured data transport. | 5.2.6.14 |
| Nnef_SMContext | Provides the capability to create, update or release the SMF-NEF Connection. | 5.2.6.15 |
| Nnef_AnalyticsExposure | Provides support for exposure of network analytics. | 5.2.6.16 |
| Nnef_NetworkStatus | Editor's note: Detailed description for Nnef_NetworkStatus service in TS 23.502 is FFS. |  |
| Nnef_UCMFProvisioning | Provides the ability to configure the UCMF with dictionary entries consisting of UE manufacturer-assigned UE Radio Capability IDs and the corresponding UE radio capability via the NEF. Also used for deletion of dictionary entries in the UCMF corresponding to specific UE manufacturer-assigned UE Radio Capability IDs. | 5.2.6.17 |
| Nnef_ECRestriction | Provides support for queuing status of enhanced coverage restriction, or enable/disable enhanced coverage restriction per individual UEs. | 5.2.6.18 |
| Nnef_ApplyPolicy | Provides the capability to apply a previously negotiated Background Data Transfer Policy to a UE or a group of UEs. | 5.2.6.19 |

Next Change

### 7.2.8x	I-NEF Services

The following NF services are specified for I-NEF:

Table 7.2.8x-1: NF Services provided by I-NEF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Ninef_EventExposure | Provides support for event exposure. | 5.2.6A.2 |

End of changes