3GPP TSG-SA WG2 Meeting #134	S2-1907755

# Sapporo, Japan, June 24 – 28, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1653 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing a new Event ID in 23.502 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS23.288, one of the Editor’s Notes left in Abnormal behaviour related network data analytics is “Whether new event ID(s) or Event Filters needs to be defined for reporting UE behaviour deviating from its expected UE behaviour is FFS.” In this contribution, we suggested a new Event ID for Namf_EventExposure service, which will help for the abnormal behaviour analysis in the following case. The change of signal strength from adjacent cells seen by the UE is one of the reasons that trigger the handover. In most of the case, for a given time length, the average number of the handover request from a UE is below a certain number. However, due to the cell coverage degradation, which could be caused by unexpected accident at the cell site, and due to the unexpected situation of the UE equipment, e.g. the external antenna is damaged or covered unexpectedly or the UE has been hiden on purpose, the number of the UE handover requests may increase. No matter whether the result of the handover is successful or not, the un expected increament of handover request number can be helpful for the NWDAF and other consumers to analyse and detect the abnormal behaviour of the UE and the network. Therefore, we proposed to add a new event ID to the Namf_EventExposure service related to the averge number of the handover required of a UE for a predefined length of time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Event ID for Namf_EventExposure service is introduced in clause 5.2.3.3.1 which related to number of the handover request of a UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes are not removed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

5.2.2.3	Namf_EventExposure service

5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about an Event ID.

Following UE access and mobility information event are considered (Event ID is defined in clause 4.15.1):

-	Location changes (TAI, Cell ID, N3IWF/TNGF node, UE local IP address and optionally UDP source port number, Area Of Interest);

-	UE moving in or out of a subscribed "Area Of Interest" as described in clause 5.6.11 in TS 23.501 [2];

-	Number of UEs served by the AMF and located in "Area Of Interest";

-	Average number of Handover Required (N2 Path Switch Request for Xn based handover and Forward Relocation Request for EPS to 5GS handover) of a UE over a predefined length of time;

-	Time zone changes (UE Time zone);

-	Access Type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status;

-	UE indication of switching off SMS over NAS service;

-	Subscription Correlation ID change (implicit subscription); and

-	Subscription Correlation ID addition (implicit subscription).

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for AMF exposure.

Table 5.2.2.3.1-1: Example of Event Filters for AMF exposure events

| Event ID | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Area of Interest | <Parameter Type = TAI, Value = TA1> <Parameter Type = PRA ID, Value = PRA ID value> |
| Access Type | <Parameter Type=AN Type, Value=3GPP Access"> |
| Location | <Parameter Type=TAI, Value=wildcard> (to report any TAI change) |

The following service operations are defined for the Namf_EventExposure service:

-	Namf_EventExposure_Subscribe.

-	Namf_EventExposure_UnSubscribe.

-	Namf_EventExposure_Notify.

* * * * End of Change * * * *