3GPP TSG-WG SA2 Meeting #135 	S2-1908925

# Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1722 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multicast forwarding for Ethernet type PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SM |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the case of Ethernet traffic the frame with a broadcast destination needs to be send over all interface connected in the same LAN. IEEE has defined that the 24-bit MAC address prefix 01-00-5E is reserved for layer 2 multicast. It should be noted that IP Multicast Address are mapped to the Ethernet multicast address. The multicast traffic is managed as broadcast traffic and send toward all ports. In general in Ethernet switches the Ethernet frame are forwarded based on the MAC address table which identified which MAC address is present on which port/VLANs . The MAC address table may be build in several way, via configuration, from ARP messages, via IGMP snooping, etc. The forwarding have to take into account both the MAC and the VLAN tagging if present, since the traffic have not be send on the wrong ports/VLAN. The general rules for multicast and broadcast traffic can be defined as follow: 1) If the destination MAC address is a broadcast or multicast address, then the frame is sent to all ports with the same VLAN ID, excluding the received port. 2) If the destination MAC address comes from the same port on which it was received, then there is no need to forward it, and it is discarded. 3) If the destination MAC address comes from another port, then the frame is sent to the identified port for transmission matching the VLAN ID (i.e. the source and destination port shall be on same VLAN ) 4) If the destination MAC address is not known, i.e. is not in the MAC address table, then the frame needs to be flooded to all ports with matching VLAN ID, except for the port through which it arrived. In case of UPF the port can be identified on the N3 interface by the PDU session and on the port toward the related DNN on N6 interface. Furthermore the rules for forwarding the traffic are based on the PDR/FAR provided to UPF by SMF. The current R16 specification defines briefly how to manage the multicast traffic in clause 5.8.2.5 on Control of user forwarding , but the following aspects are not correct: 1) the ethernet broadcast and multicast traffic is not identified by the Source MAC address but by the usage of MAC address FF:FF:FF:FF:FF (broadcast) or 24-bit MAC address prefix 01-00-5E (Multicast). 2) how VLAN is considered in forwarding is missing 3) unclear usage of MAC address without distiction when it is used as source or as destination which cause erroneous interpretation of the specification. The Note 2 indicates that the SMF/UPF can remove the MAC address from forwarding rules is has been not detected, but this is behaviour should be a specification requirement otherwise the traffic is send toward the wrong PDU session cause an incorrect behaviour of the services supported by Ethernet PDU session |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the specification related the support of Ethernet multicast and broadcast traffic in UL and DL. Clarify the usage of VLAN in forwarding rules The note 2 is changed to normative text and clarified |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non correct specification of support of Ethernet multicast and broadcast for PDU Session of Ethernet type |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.8.2.5	Control of User Plane Forwarding

The SMF controls user-plane packet forwarding for traffic detected by a PDR by providing a FAR with instructions to the UPF, including:

-	Forwarding operation information;

-	Forwarding target information.

The details of the forwarding target and operation will depend on the scenario and is described below. The following forwarding functionality is required by the UPF:

-	Apply N3 /N9 tunnel related handling, i.e. encapsulation.

-	Forward the traffic to/from the SMF, e.g. as described in Table 5.8.2.5-1.

-	Forward the SM PDU DN Request Container from SMF to DN-AAA server

-	Forward the traffic according to locally configured policy for traffic steering.

-	Forward the traffic according to N4 rules of a 5G VN group for 5G VN group communication.

Data forwarding between the SMF and UPF is transmitted on the user plane tunnel established on N4 interface, defined in TS 29.244 [65].

Scenarios for data forwarding between the SMF and UPF are defined as below:

Table 5.8.2.5-1: Scenarios for data forwarding between the SMF and UPF

|  | Scenario description | Data forwarding direction |
| --- | --- | --- |
| 1 | Forwarding of user-plane packets between the UE and the SMF e.g. DHCP signalling. | UPF to SMF SMF to UPF |
| 2 | Forwarding of packets between the SMF and the external DN e.g. with DN-AAA server | UPF to SMF SMF to UPF |
| 3 | Forwarding of packets subject to buffering in the SMF. | UPF to SMF SMF to UPF |
| 4 | Forwarding of End Marker Packets constructed by the SMF to a downstream node. | SMF to UPF |
| 5 | Forwarding of user data using Control Plane CIoT 5GS Optimisation | UPF to SMF SMF to UPF |

When configuring an UPF acting as PSA for an Ethernet PDU Session Type, the SMF may instruct the UPF to route the traffic via PDR and FAR which are construct considering thefollowing rules:

-	for DL traffic received by UPF on the N6 Network Instance if the destination MAC address is a broadcast or multicast address shall forward the traffic to every DL PDU Session (corresponding to any N4 Session) associated with this Network Instance, unless the SMF knows that on a specifc PDU session there is no destination interested on the multicast traffic (e.g. based on AF influence of traffic, configuration, etc).

-	for uplink traffic received by UPF over a PDU session on a N3/N9 interface if the destination MAC address is a broadcast or multicast address , the UPF shall  forward the traffic to the N6 interface and downlink to every PDU session (except toward the one of the incoming traffic) associated with the same N6 Network Instance unless the SMF knows that on a specifc PDU session there is no destination interested on the multicast traffic (e.g. based on AF influence of traffic, configuration, etc).

-	for uplink and downlink traffic received by UPF if the destination MAC is not know, even in case of  unicast traffic, the frame shall be forwarded to every PDU session associated with the same N6 Network Instance and toward the N6 interface except toward the one of the incoming traffic.

-	if the traffic is received with VLAN tag, the above criteria applies only towards the N6 interface or PDU session matching the same VLAN ID, unless the UPF is instructed to remove the VLAN ID.

-	if the destination MAC address of traffic comes from the same N6 interface or PDU session on which it has received , fhe frame should be dropped.

-	traffic steering information received from AF via NEF as described in clause 5.6.7 or received by AAA at PDU session secondary authentication as described in clause 5.6.6

When the Ethernet PDU Session Information is set in the PDR, the UPF drops such DL traffic based on the MAC address(es) used for the UL traffic. Alternatively, in case Ethernet PDU Session Information is not set in the PDR, the SMF may installs explicit downlink filters in UPF for this purpose.

NOTE 1:	This is done in order to avoid the formation of loops in case of Ethernet topology changes, e.g., due to topology changes in the Data Network and/or topology changes in local networks behind the UE and/or topology changes due to UPF relocation.

In order to handle scenarios where a device behind a UE is moved from one UE to another UE, the SMF/UPF can remove a MAC address from the forwarding rules in case that MAC address has not been detected as Source MAC address in UL traffic for a period of time or it has been detected under a different UE.

For ARP/IPv6 Neighbour Solicitation traffic, a SMF's request to respond to ARP/IPv6 Neighbour Solicitation based on local cache information or to redirect such traffic from the UPF to the SMF overrules the traffic forwarding rules described above.

NOTE 3:	Local policies in UPF associated with the Network Instance can prevent local traffic switching in the UPF between PDU Sessions either for unicast traffic only or for any traffic. In the case where UPF policies prevent local traffic switching for any traffic (thus for broadcast/multicast traffic) some mechanism such as responding to ARP/ND based on local cache information or local multicast group handling is needed to ensure that upper layer protocol can run on the Ethernet PDU sessions.

If allowed by local UPF (acting as PSA) policies, for Ethernet traffic with unknown unicast destination MAC addresses, the UPF should forward the traffic in the same manner as described above for the broadcast and/or multicast traffic handling.

The SMF may ask to get notified with the source MAC addresses used by the UE and provide UPF with corresponding forwarding rules related with these MAC addresses.

# * * * * End of changes * * * *