3GPP TSG SA2 Meeting #135	S2-1909308

# Split, Croatia 14 - 16 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0290 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BDT renegotiation upon expected network performance change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 18-10-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing solution allows the MNO to notify the ASP that a BDT policy needs to be renegotiated if the ASP wants to improve the quality of the BDT traffic. The problem is that: - The negotiated BDT policy remains active. In particular, the charging and rate limit of the BDT policy still applies. The MNO has no mechanism to improve the network conditions, for example, by applying a more restrictive rate limitation or by applying a different charging to the traffic that will be routed in the network area that may already experience heavy load. - If the ASP wants to improve the quality of the BDT traffic, it needs to start the negotiation of new BDT policies. The problem is that the existing BDT policy still applies until the negotiation of new BDT policies is completed. This causes extra signaling between the MNO and the ASP and delays the application of new BDT policies according to the new network conditions. Furthermore, if a background data transfer session is already ongoing, the decision of the time when to send BDT is left to the ASP only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that the PCF includes as part of the notification: - The new conditions (charging rate and maximum aggregated bitrate) that will apply to the previously negotiated BDT policy . - The new list of candidate BDT policies for the AF to select. If a previously negotiated BDT transfer policy PCF was changed, the PCF updates the BDT policy for the corresponsing BDT reference ID in UDR. The PCF may also need to update the Validity Conditions in the URSP rules provided to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No mechanism defined to allow the MNO to update the conditions provided to the ASP for Backgound Data Transfer when the NW conditions change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 1477 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

#### 6.1.2.4	Negotiation for future background data transfer

The AF may contact the PCF via the NEF (and Npcf_BDTPolicyControl_Create service operation) to request a time window and related conditions for future background data transfer.

NOTE 1:	The NEF may contact any PCF in the operator network.

The AF request shall contain an ASP identifier, the volume of data to be transferred per UE, the expected amount of UEs, the desired time window, the External Group Identifier and optionally, Network Area Information, request for notification. The AF provides as Network Area Information either a geographical area (e.g. a civic address or shapes), or an area of interest that includes a list of TAs or list of NG-RAN nodes and/or a list of cell identifiers. When the AF provides a geographical area, then the NEF maps it based on local configuration into of a short list of TAs and/or NG-RAN nodes and/or cells identifiers that is provided to the PCF. The request for notification is an indication that BDT warning notification should be sent to the AF. The BDT warning notification indicates to the ASP that the BDT policy needs to be re-negotiated.

NOTE 2:	A 3rd party application server is typically not able to provide any specific network area information and if so, the AF request is for the whole operator network.

The PCF shall first retrieve all existing background transfer policies stored for any ASP from the UDR. The PCF may retrieve analytics on "Network Performance" from NWDAF in the area where the UEs of this ASP are expected to be located at a certain time period. The "Network Performance" Analytics includes the tuple-(expected load in the area of interest, expected number of UEs of this ASP in the Area of Interest) following the procedure and services described in TS 23.288 [24]. Afterwards, the PCF shall determine, based on the information provided by the AF, the analytics on "Network Performance" if available and other available information (e.g. network policy and existing background transfer policies) one or more background transfer policies. The PCF may be configured to map the ASP identifier into a target DNN and slicing information (i.e. S-NSSAI).

A background transfer policy consists of a recommended time window for the background data transfer, a reference to a charging rate for this time window, network area information and optionally a maximum aggregated bitrate (indicating that the charging according to the referenced charging rate is only applicable for the aggregated traffic of all involved UEs that stays below this value). Finally, the PCF shall provide the candidate list of background transfer policies or the selected background transfer policy to the AF via NEF together with the Background Data Transfer reference ID. If the AF received more than one background transfer policy, the AF shall select one of them and inform the PCF about the selected background transfer policy.

NOTE 3:	The maximum aggregated bitrate (optionally provided in a background transfer policy) is not enforced in the network. The operator may apply offline CDRs processing (e.g. combining the accounted volume of the involved UEs for the time window) to determine whether the maximum aggregated bitrate for the set of UEs was exceeded by the ASP and charge the excess traffic differently.

NOTE 4:	It is assumed that the 3rd party application server is configured to understand the reference to a charging rate based on the agreement with the operator.

The selected background transfer policy is finally stored by the PCF in the UDR together with the Background Data Transfer reference ID, the volume of data to be transferred per UE, the expected amount of UEs and the information of request for notification, if available. The same or a different PCF can retrieve this background transfer policy and corresponding related information from the UDR and take them into account for future decisions about background transfer policies for background data related to the same or other ASPs.

When the AF wants to apply the Background Data Transfer Policy to an existing session, then the AF will, at the time the background data transfer is about to start, provide, for each UE, the Background Data Transfer reference ID together with the AF session information to the PCF (via the N5 interface). The PCF retrieves the corresponding background transfer policy from the UDR and derives the PCC rules and PDU session related policy information (i.e. Time Window and Location Criteria) for the background data transfer according to this transfer policy.

When new PDU session setup is requested, PCF may reject corresponding SM Policy Association, as described in clause 4.16.4 of TS 23.502 [3], if Validation condition is not satisfied. And based on this feedback, SMF will reject the PDU session setup.

After successful PDU session setup, PCF may trigger PDU session release when Validation condition is not satisfied.

When the AF wants to apply the Background Data Transfer Policy to a future session, then the AF provides, to the NEF, the Background Data Transfer reference ID together with the External Identifier or External Group Identifier of the UE(s) that are to be subject to the policy. The NEF stores the Background Data Transfer Reference ID in the UDR as Application Data for the UE(s). A PCF that serves the UE(s) (i.e. the PCF that serves the UE for AM Policies) may retrieve the Background Data Transfer Reference ID by retrieving the UE's Application Data from the UDR or by subscribing to notifications of changes to the UEs' application data in the UDR. Furthermore, the PCF retrieves the specific Background Data Transfer Policy based on the received Background Data Transfer Reference ID from the UDR.

When the PCF determines to send the Background Data Transfer Policy information to the UE as part of a URSP rule, the PCF will store the policy in the UDR as part of the UE's Policy Set Entry and will use the associated S-NSSAI and DNN to store the Background Data Transfer Reference ID in the UE's PDU Session policy control subscription information (see clause 6.2.1.3). The PCF uses local policies to decide if and when the Background Data Transfer Policy information is going to be sent to the UE as Validation Criteria in the RSD part of the URSP rule (see clause 6.6.2.1). The UE uses Validation Criteria to determine whether or not a PDU Session should be established. The Time Window and Location Criteria are not required to be checked again during the lifetime of the PDU Session.

The PCF may, based on operator configuration, trigger the UE Configuration Update procedure when the policy is selected, or the PCF may wait until receiving a notification from the AMF that the UE has entered the Tracking Area or Presence Area where the policy applies, and/or the PCF may wait until the time window when the policy applies is approaching. The UE's support of the Validation Criteria in a URSP rule is optional.

NOTE 5:	If a non-supporting UE receives Validation Criteria, it ignores the URSP rule.

When the PDU Session is established, the PCF that serves the PDU session will use the Background Data Transfer Reference ID in the UE's PDU Session policy control subscription information (see clause 6.2.1.3) to retrieve the corresponding background data transfer policy (i.e. Time Window and/or Location Criteria) from the UDR and derives the PCC rules for the background data transfer according to this transfer policy.

NOTE 6:	The AF will typically contact the PCF for the individual UEs to request sponsored connectivity for the background data transfer.

NOTE 7:	A transfer policy is only valid until the end of its time window. The removal of outdated transfer policies from the UDR is up to implementation.

When the PCF knows that the network performance in the area of interest goes below the criteria set by the operator from the NWDAF as described in TS 23.288 [24], the PCF retrieves all the background transfer policies from the UDR. For each of the affected background transfer policies the PCF notifies the background transfer reference ID, the updated content that the PCF will apply due to change of conditions and optionally time, network area information of affected background transfer policy to each AF that subscribed to be notified. The PCF also provides a new candidate list of background transfer policies to the AF, through the NEF.

If the PCF provided updated content for the previously negotiated background policy transfer in the notification, the PCF updates with this content the background transfer policy stored in the UDR for the corresponding background transfer reference ID.

When the AF receives the notification, the AF may select one of the background transfer policies included in the candidate list then inform the PCF about the selected background transfer policy. The PCF updates the background transfer policy stored in the UDR for the corresponding background transfer reference ID.

As a consecuence of the update in the UDR of the background transfer policy, if there is a change in the negotiated Time Window, the PCF identifies the UEs for which the background transfer policy was already applied and updates URSP rules with the new Validation Criteria as described in section 4.16.12.2 in 23.502 [3].

NOTE x: A PCF can subscribe to notifications on changes in background transfer policy in UDR. Upon reception of such notification the PCF has also to identify the UEs for which the background transfer policy was already applied and update URSP rules with the new Validation Criteria as described in section 4.16.12.2 in 23.502 [3].

* * * End of Changes * * * *