3GPP TSG-SA2 Meeting #135	S2-1909799

# Split, Croatia, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0086 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding reference architecture and reference points to support network data analytics services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The reference points between NWDAF and PCF/NSSF (N23/N34) had been defined in Release15. In R16, NWDAF need to interact with more NFs (e.g., NEF, UDF, NRF, AMF, SMF) for the data collection and provision of analytics to more consumers. These addition of reference points and related descriptions are missing in current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: 1. addition of reference architecture for 5G system to support network data analytics services in reference point representation. 2. addition of reference points to support network data analytics services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of referece points to support network data analytics services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 4.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

4.2	Non-roaming architecture

As depicted in Figure 4.2-1, the 5G System architecture allows NWDAF to collect data from any NF. The NWDAF belongs to the same PLMN where the network function that notifies the data is located.

Figure 4.2-1: Data Collection architecture from any NF

The Nnf interface is defined for the NWDAF to request subscription to data delivery for a particular context, to cancel subscription to data delivery and to request a specific report of data for a particular context.

The 5G System architecture allows NWDAF to retrieve the management data from OAM by invoking the existing OAM services.

As depicted in Figure 4.2-2, the 5G System architecture allows any NF to request network analytics information from NWDAF. The NWDAF belongs to the same PLMN where the network function that consumes the analytics information is located.

Figure 4.2-2: Network Data Analytics Exposure architecture

The Nnwdaf interface is defined for the network functions, to request subscription to network analytics delivery for a particular context, to cancel subscription to network analytics delivery and to request a specific report of network analytics for a particular context.

Figure 4.2-3 depicts reference architecture for 5G system to support network data analytics services in reference point representation.

Figure 4.2-3: non-roaming reference architecture for 5G system to support 
network data analytics services in reference point representation

NOTE:	The reference points to support network data analytics services are listed in clause 4.X.

* * * Next change * * * *

4.X	Reference points to support network data analytics services

The following reference points show the interactions that exist between NWDAF and other NFs.

N59:	Reference point between the NWDAF and an AF.

N60:	Reference point between the NWDAF and the NEF.

N61:	Reference point between the NWDAF and the UDM.

N62:	Reference point between the NWDAF and the NRF.

N63:	Reference point between the NWDAF and the AMF.

N64:	Reference point between the NWDAF and the SMF.

NOTE:	The N23 and N34 reference points are listed in clause 4.2.7 of TS23.501 [2].

* * * End of changes * * * *