SA WG2 Meeting #S2-135	S2-1910429

14 - 18 October, 2019, Split Croatia                                         (Revision of S2-1909400)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR |  | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GS bridge model interpretation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in S2-1909399, the current 5GS bridge model may have two interpretations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove text that specifies any binding relationship between ports. The internal binding of the PDU session to a UE/DS-TT side port is still valid. Change Figure 5.29.1-1 to show the bridging property. Moreover, the sentence “Identity of the 5GS Bridge and port numbers of the Ethernet in NW-TT could be preconfigured on the UPF, which is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs.” may be misleading as one could interpret that the port number are selected based on subscribed DNN, traffic classes and VLANs. However, it applies to the UPF. We modified this sentence. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS bridge model may be interpreted in more than one way. One of the interpretations leads to limited connectivity of the 5GS bridge and will not operate properly in a TSN system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 5.28.1	5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the ports on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the logical TSN bridge is per UPF. The bridge ID of the logical TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side and a PDU Session during reporting of logical TSN bridge information. The TSN AF also stores the information about ports on the UPF/NW-TT side. The UPF implements a switching functionality that forwards traffic to the appropriate egress port based on the traffic forwarding table.

All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single virtual bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5G virtual bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 1:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based virtual bridge

NOTE 2:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple logical TSN bridges.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Report the bridge information of 5GS Bridge to TSN network.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes the following:

-	Bridge ID of 5GS Bridge:

-	MAC address of the bridge;

-	bridge name;

-	number of ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	Topology of 5GS Bridge as defined in IEEE 802.1AB [97].

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q [98].

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

Identity of the 5GS Bridge and port numbers of the Ethernet in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs. Port number of Ethernet port on the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment and stored at the SMF. SMF provides the port numbers and MAC addresses of the Ethernet ports in DS-TT of the related PDU session and of NW-TTs to the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released then SMF informs TSN AF accordingly.

The AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

* * * * End of Change * * * *