SA WG2 Meeting #S2-136AH	S2-2000126

13 - 17 January, 2020, Incheon, Korea

Source:	Nokia, Nokia Shanghai Bell

Title:	PDU session enhanced for multicast to provide the basic multicast connectivity service

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_5MBS

Abstract of the contribution: This contribution proposes a solution for key issue #1 (MBS session management) and key issue #Y (Seamless switch between unicast and multicast delivery).

# 1 	Introduction

The needs for different service levels were discussed in S2-1910741. The key issue on the definition service levels was proposed in the same document and agreed. A basic service level is proposed in S2-1911365. This paper proposes a solution that enhances the existing PDU session management procedures which are then used to configure the 5G System so that the basic service level can be provided as a basic multicast connectivity service.

# 2	Proposal

The proposed changes to TR 23.757 are the following:

*** Changes to 23.757 ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 22.101: "Service aspects; Service principles".

[y]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[x]	3GPP TS 23.316: " Wireless and wireline convergence access support for the 5G System (5GS)"

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 MBS session management | 2 Service levels definition | 3 Levels of authorization for MC | 4 QoS for MC and BC | 5 BC TV and Radio services | 6 Local MBS | 7 MC-UC delivery mode switch | 8 BC-UC delivery method switch |
| Solution #X: PDU Session Enhanced for Multicast with switching between Unicast and Multicast delivery in RAN | x |  |  |  |  |  | x |  |

## 6.X	Solution #X: PDU Session Enhanced for Multicast with switching between Unicast and Multicast delivery in RAN

### 6.X.1	Functional description

This solution proposes to allocate the switching between unicast and multicast delivery in RAN and to enhance the PDU session for the support of multicast PDUs transfer between a data network and UEs and thus offer the basic service level.

switching between unicast and multicast delivery in RAN

The basic multicast connectivity service meets the requirements for the basic service level and allows the exchange of multicast PDUs between a data network and UEs in a way that does not require a content provider to explicitly request the service from the 3GPP System. In this perspective, the basic multicast connectivity service is not different from the (unicast) connectivity service provided by PDU session. However, an efficient delivery of multicast can be achieved only by a proper configuration of 5G System that can be accomplished by enhancing PDU session modification procedures as presented in this document.

Before describing the solution, it is important to discuss what functionalities are required in the system for an efficiently delivery of multicast. The efficiency of 3GPP system regarding the delivery of same data to UEs have been a topic of many study and work items and resulted in a number of enhancements that tackle this issue. These enhancements are MBMS counting in RAN, MBMS operation on Demand (MooD), and application level signalling (e.g. signalling over GC1 and V1 reference points) of UE’s serving cell identity or multicast area identities that are consequently used for MBMS bearer management by an application server (e.g. GC AS or V2X AS).

MooD and the application level signalling are in principle the same because MooD also relies on UE location reporting either as part of consumption reporting or within an HTTP request header. A decision whether to switch between MBMS and unicast delivery is solely based on cell identities not considering any other aspects of RAN operation. On the other hand, MBMS counting in RAN was the first enhancement introduced in E-UTRAN to help MCE to determine whether to suspend or resume MBMS bearers.

The MBMS counting in RAN has its own drawbacks. The counting is not accurate. CN is not informed about the MBMS bearer suspension. The MBMS bearer suspension likely results in a service interruption because UEs have to establish unicast connection with an application server. The problem of service interruption was address by MBMS bearer to be suspended notification for MBSFN transmission in Rel-12. Nonetheless, the fundamental problem with all these solutions is that the change between unicast and MBMS happens end-to-end on per-cell basis while only RAN efficiency is the biggest concern and the rest of system is not optimized.

The end-to-end switching between unicast and MBMS is also not optimal from the core network resource management perspective. For example, considering that a RAN node serves number of cells where in each cell there is at least one UE receiving a service but only in one cell the number of receiving UEs is above a threshold used by an application server or a MooD proxy to trigger MBMS delivery. The outcome of switching to MBMS delivery for that cell is that the number of content copies delivered through the system is equal to the number of cells. In addition to that, the load of application server can only be reduced partially because the application server must serve unicast request originated from the same RAN node (please note, the application server is not aware and shall not be aware if this situation) after the switch to MBMS delivery. This is illustrated on Figure 6.X.1-1. The AS requested delivery via MBMS in the cell with diagonal stripes while unicast is used to deliver content in other cells because the threshold for switching to MBMS delivery for these cells has not been met. Deployments when a RAN node is serving a considerable number of cells should not be considered uncommon especially considering CU-DU split RAN architecture in 5G.

Figure 6.X.1-1: Content delivery with unicast and MBMS

In Figure 6.X.1-2, it is assumed that the RAN is aware of users in a multicast group and of data streams to be multicast. The RAN uses this information in addition to any other information available such as UE’s location, cell load, etc. in the second stage when it decides whether a unicast bearer or a point-to-multipoint RAN bearer (particulars to be studied by RAN) should be used to deliver the multicast data. This is illustrated on Figure 2. Figure 6.X.1-2 also assumes that multicast content is delivered a single time to the RAN by a multicast distribution session associated with the multicast session. A less efficient alternative would be to deliver the content to the RAN via unicast, and the RAN can then detect, e.g. based on a multicast address, that the content can be delivered over the radio via multicast.

Figure 6.X.1-2: Multicast content delivery multicast and unicast selection at NG-RAN and AS

PDU Session Enhanced for Multicast

The proposed multicast communication service session management is an extension of the existing solution for IPTV in 3GPP TS 23.316 [x].

Enabling efficient delivery of common content to UEs using multicast capabilities should be thought of as two stages process. In the first stage, the AS enables multicast delivery, which comprises of an advertisement of multicast session (i.e. providing information about multicast group via an application layer signalling) to UEs and a transmission of data to the multicast session (i.e. sending multicast packets to the multicast group). As part of NR-RAN and 5GC configuration during the enhanced PDU Session update procedure (see section 6.X.3), 5GC is configured to receive multicast packets sent to the multicast group and to deliver multicast packets to the NG-RAN via a tunnel. The tunnel can be a unicast tunnel as currently specified for MBMS but also multicast tunnelling could be considered. The PDU Session modification is performed for each UE that joined the multicast group and thus the NG-RAN learns about the UEs to which the multicast data shall be delivered. The NG-RAN uses this information in addition to any other information available such as UE’s location, cell load, etc. in the second stage when it decides whether a unicast bearer or a point-to-multipoint RAN bearer (particulars to be studied by RAN) should be used to deliver the multicast data. This is illustrated on Figure 6.X.2-2. It should be noted that UEs in cells where the content is delivered using point-to-multipoint bearers have always availability to communicate with the AS, for example, for the purpose of a unicast repair.

#### 6.X.2.1	Multicast context and Multicast flow setup/modification via PDU Session Modification procedure

The Multicast context and Multicast flow setup/modification uses an enhanced PDU session modification procedure for unicast traffic defined in TS 23.502 [x].

Figure 6.X.2-1: PDU Session modification for multicast

- The content provider announces the availability of multicast using higher layer (e.g. application layer). The announcement includes at least multicast address of multicast group that UE can join.
The content provider may also send a request to reserve resources for the corresponding multicast session to the NEF and communicate the related multicast address. If a single ingress point for multicast data is required, the UPF and SMF to handle a multicast distribution session may then be assigned and stored in the UDR. If tunnelling is used to transport multicast data in the external network, the transport address at the UPF to receive tunnelled multicast data is assigned and communicated to the content provider; as an alternative IP multicast can be used in the external network. The NEF also stores information about the node assigned to handle the multicast distribution in the data base.

NOTE 1:	The request to reserve resources for the corresponding multicast session is optional and can be replaced by configured data based on commercial agreements. If IP multicast is used in the external network, the content provider does not require information where to send the multicast data.

2.	The UE registers in the PLMN (see subclause 4.2.2.2 of 3GPP TS 23.502 [y]) and request the establishment of a PDU session (see subclause 4.3.2.2 of 3GPP TS 23.502 [y]). The AMF obtains information from the UDM whether the UE can join multicast sessions as part of the SMF Selection Subscription data. If so, for direct discovery the AMF selects an SMF capable of handling multicast sessions based on locally configured data or a corresponding SMF capability stored in the NRF. For delegated discovery the AMF indicates within request to establish the PDU session to the SCP that support of multicast is required for the selected SMF.

3. Alternative 1: user plane signaling

3a. The UE joins the multicast group

3b.	The UPF is a multicast capable router. The reception of the join message triggers the UPF to notify the SMF. The UPF can be optimized to send the notification only when UE’s status in regard to number of multicast groups UE has joined changes, i.e. when the UE joins or leaves a group. The SMF initiates PDU session modification procedure upon the reception of the notification from the UPF.

4.	Alternative 2: control plane signaling

4a.	The UE sends the PDU Session Establishment/Modification Request either upon a request from higher layers or upon a detection by lower layers of UE joining a multicast group (i.e. detection of IGMP or MLR and detection of the change of content of these messages). The PDU Session Modification Request shall include information about multicast group, which UE wants to join, such as multicast addresses listed in the IGMP and MLR messages. This information is needed for configuration of UPF with appropriate packet filters.

4b.	The AMF invokes Nsmf_PDUSession_UpdateSMContext (SM Context ID, N1 SM container (PDU Session Modification Request with the multicast information)).

5.	The SMF checks at the UDR whether a multicast distribution session for the multicast group (address) exists in the system, i.e. whether there is a UE that already joined the multicast group. If the multicast distribution session for the multicast group does not exist, then the SMF creates it when the first UE joins the multicast group and stores in the UDR that it is handling the multicast distribution session.

NOTE 2:	In deployments where a single SMF in a network slice is supporting multicast sessions, the SMF can use local data rather than data stored in the UDR to determine whether multicast distribution session for the multicast group exist.

6.	The SMF request the AMF to transfer a message to the RAN node indicating that the UE joined the multicast session identified by the multicast address using the Namf_Communication service.

Editor’s note: Whether the N1N2 message transfer service operation or a new service operation is used is ffs. The related Session Management signaling towards UE, e.g. providing information regarding Multicast Context and Mulitcast Flows is also FFS. It is also FFS if additional dedicated signaling towards the RAN node to establish a multicast distribution session is required.

7.	The session modification request is sent to the RAN. The request is sent in the UE context using the currently standardized message enhanced with multicast related information, which include a multicast group identity (e.g. multicast address itself or an identity generated by the SMF that corresponds to the multicast group). The RAN is using the multicast group identity to determine that the session modification procedures of two or more UEs correspond to one multicast group. In other words, the RAN learns what UEs are receiving the same multicast from the multicast group identity. When the RAN receives a session modification request for previously unknown multicast group identity, the RAN is configured to serve this multicast group. The configuration may include the allocation of downlink tunnel if unicast tunnelling on N3 reference point is used or joining multicast group if multicast tunnelling on N3 is used.

8.	The RAN performs necessary access network resource modification such as configuration of broadcast bearers. The details of this procedure should be studied in the RAN.

9.	The RAN sends the session modification response that may include downlink tunnel information, see step 6.

10.	The AMF transfers the possible downlink tunnel information received in step 6 to the SMF. The SMF stores in the UDR that it requires that the multicast distribution session is distributed towards the RAN node serving the UE and the possible received downlink tunnel information if it did not previously store that information when handling a multicast service request of another UE served by the same RAN node. If another SMF is assigned to handle the multicast distribution session and the UDR has not previously requested that other SMF to send data for that multicast distribution session to that RAN node, the UDR informs that other SMF.

Editor’s note: Whether the Nsmf_PDUSession_UpdateSMContext service operation or a response to the Namf_Communication service operation is used is ffs.

11.	If the RAN node serving the UE does not yet receive the multicast distribution session. the SMF handling the multicast distribution session sends an N4 session modification request to the UPF including tunnel information if unicast tunnelling is used.

NOTE 3:	The exemplary procedure in this call flow shows that the same UPF that send the user plane event notification in step 3b, and the same SMF receiving the UE request in step 3b or 4b, is also serving the multicast distribution session. However, the information stored in the UDR related to the multicast distribution session allows the system to also support other deployments.

12.	The UPF receives multicast PDUs according to the configuration in strep 11.

13.	The UPF sends multicast PDUs in the N3/N9 tunnel associated to the muticast distribution session to the RAN. There is only one tunnel per multicast distribution session and RAN node, i.e. all associated PDU sessions share this tunnel.

14.	The RAN selects multicast od unicast radio bearers to deliver the multicast PDUs to UEs that joined the multicast group.

15. The RAN performs the transmission using the selected bearer.

### 6.X.3	Impact analysis

SMF: The SMF must handle a multicast context and the enhanced PDU session procedures.

AMF: no impacts

UPF: The UPF must support a new capability to trigger a user plane event in a response to the reception of a join message. The UPF should also act as multicast capable router but this functionality was already introduced in 3GPP TS 23.316 [x].

RAN: The RAN must support the PDU session procedures and store UEs’ association with multicast group in a context as received from the SMF. The RAN should be able to autonomously select unicast or PTM bearers that are used for multicast data transmission to UEs.

N3: A tunnel on this interface, which is configured when the first UE joins a multicast group and PDU session modification is performed, should be used to deliver multicast data from the UPF to the RAN. This tunnel is released when the last UE leaves the group.