3GPP TSG SA WG2 Meeting #136AH	S2-2000184

# Incheon, Korea, 13-18 January 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2019 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for handover between SNPN and PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR related to open issue #13 " Is seamless service continuity between SNPN and PLMN supported " and stage 1 requirements in TS 22.261: "Subject to an agreement between the operators and service providers, operator policies and the regional or national regulatory requirements, the 5G system shall support for non-public network subscribers: - access to subscribed PLMN services via the non-public network; - seamless service continuity for subscribed PLMN services between a non-public network and a PLMN; - access to selected non-public network services via a PLMN; - seamless service continuity for non-public network services between a non-public network and a PLMN." Description of service (PDU Session) continuity when UE moves between SNPN and PLMN when at the same time accessing PLMN via SNPN and vice versa is not clear in the specifications. There are 4 different scenarios that should be better described: UE handover from PLMN to SNPN when the PDU Session is anchored in PLMN. UE handover from PLMN to SNPN when the PDU Session is anchored in SNPN. UE handover from SNPN to PLMN when the PDU Session is anchored in PLMN. UE handover from SNPN to PLMN when the PDU Session is anchored in SNPN. The procedures are already supported using the existing procedures in 23.502 but there is a benefit on showing more explicitly how these mobility procedures can be used for the applicable mobility scenarios. Also 23.501 need to state that the scenarios are supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 23.501 it is clarified that service (PDU Session) continuity when UE moves between SNPN and PLMN when at the same time accessing PLMN via SNPN and vice versa is supported. Adding a reference to a new annex, in TS 23.502, with informative procedures as proposed by CR S2-2000185. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not enough guidance for implementations as missing procedures for PDU Session continuity between PLMN and SNPN and unclear if stage 1 requirements supported or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** First change **************

#### 5.30.2.x	Session continuity between SNPN and PLMN

Session continuity of PLMN services when a UE changes between SNPN 3GPP access and PLMN 3GPP access, and session continuity of SNPN services when a UE changes between PLMN 3GPP and SNPN 3GPP access, are supported by using the procedures in TS 23.502 [3] clause 4.9.2.1 and 4.9.2.2 as described in TS 23.502 [3] Annex X.

*************** End of changes ***********