3GPP TSG-SA/WG2 Meeting #136-AH	S2-2000201

# Incheon, Korea,  2020-01-13 -- 2020-01-17

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2021 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Binding for notification reselection corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is wrongly indicated that the NF service instance Id may be used for reselection of a notification target, when this Id is not suitable to be used in a discovery request. It is wrongly indicated that the service name may be used as the unique value in the Binding indication. It is not indicated how the Binding indication is used once received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is corrected that the NF service instance Id is not suitable to be used for reselection of a notification target. It is corrected that the service name is only possible to be used in combination to other id in the Binding indication, not as an isolated value, since it is not enough to perform the discovery of the corresponding alternative notification endpoints. It is indicated the usage of the Binding indication (it is used to discover alternative notification endpoints). Clarification that the Routing Binding indication is a copy of the Binding indication, instead of the ambiguos term used: “derived”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong usage of NF service instance Id. Wrong usage of service name. Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.0, 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 1959 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START CHANGES ***

#### 6.3.1.0	Principles for Binding, Selection and Reselection

Binding can be used to indicate suitable target NF producer instance(s) for NF service instance selection, reselection and routing of subsequent requests associated with a specific NF producer resource (context) and NF service. This allows the NF producer to indicate that the NF consumer, for a particular context, should be bound to an NF service instance, NF instance, NF service set or NF set depending on local policies and other criteria (e.g. at what point it is in the middle of a certain procedure, considering performance aspects etc).

Binding can also be used by the NF consumer to indicate suitable NF consumer instance(s) for notification target instance reselection and routing of subsequent notification requests associated with a specific notification subscription.

The Binding indication is provided by the recipient of the messages the binding indication relates to. The Binding Indication contains the information in Table 6.3.1.0-1.

The Routing Binding Indication may be included in Request, Subcsribe or Notification messages (see clause 7.1.2). It can be used in case of indirect communication by the SCP to route the message. The Routing Binding Indication is a copy of the information in the Binding Indication that may be used either by consumer or producer but for indirect communication may be used by SCP instead.

The NF service producer may provide a Binding Indication to the NF service consumer as part of the Direct or Indirect Communication procedures, to be used in subsequent related service requests. The level of Binding Indication provided by the NF service producer to the NF consumer indicates if the resource in the NF service producer is either bound to NF service instance, NF instance, NF Service Set or NF set as specified in Table 6.3.1.0-1. The Binding Indication may include NF Service Set ID, NF Set ID, NF instance ID, or NF service instance ID, for use by the NF consumer or SCP for NF Service Producer (re-)selection. If the resource is created in the NF Service Producer, the NF Service Producer provides resource information which includes the endpoint address of the NF service producer. For indirect communication, the NF service consumer should copy the Binding Indication into the Routing Binding Indication , that is included in subsequent requests, to be used by the SCP to discover a suitable NF service producer instance(s).

During explicit or implicit notification subscription, a Binding Indication may be provided by the NF service consumer to NF service producer. The NF service consumer may also provide a Binding Indication in response to notification requests. The level of Binding Indication provided by the NF service consumer to the NF service provider indicates if the notification endpoint is either bound to NF instance, NF Service Set or NF set as specified in Table 6.3.1.0-1. The Binding Indication may include NF Set ID, NF instance ID or NF Service Set ID. As an addition,  service name may be included to indicate that the alternative endpoints are defined at service level. The NF Service Set ID,  and service name relate to the service of the NF service consumer that will handle the notification. In case the provided notification endpoint by the NF Service Consumer to the NF Service Producer cannot be reached, the NF Service Producer may use the information in the Binding Indication to discover an alternative default notification endpoint. These notification endpoints are default notification endpoints that the NF Service Consumer may register in the NRF as part of the NF profile. The default notification endpoint may be either on NF level, or NF service level.

In case the notification endpoint included in the subscription is not reachable, the Binding Indication is used by the NF service producer to discover an alternative default notification endpoint, i.e. the URI where the notification is to be sent. These notification endpoints are default notification endpoints that the NF Service Consumer may register in the NRF as part of the NF profile. The default notification endpoint may be either on NF level, or NF service level. For indirect communication, the NF service producer  copies the Binding information into the Routing Binding Indication that is included in the Notification  request, to be used by the SCP to discover an alternative default notification endpoint.

Table 6.3.1.0-1 defines the selection and reselection behaviour of NF services consumers and SCPs depending on the binding indication provided by an NF service producer. The detailed procedures refer to clause 4.17.11 and 4.17.12 of TS 23.502 [3]

Table 6.3.1.0-1: Binding, selection and reselection

| Level of Binding indication | The NF Consumer / Notification sender / SCP selects | The NF Consumer / Notification sender / SCP can reselect e.g. when selected producer is not available | Binding information for selection and re-selection |
| --- | --- | --- | --- |
| NF Service Instance | The indicated NF Service Instance | An equivalent NF Service instance: - within the NF Service Set (if applicable) - within the NF instance - within the NF Set (if applicable) | NF Service Instance ID (NOTE y), NF Service Set ID, NF Instance ID, NF Set ID, Service name (NOTE 4) |
| NF Service Set | Any NF Service instance within the indicated NF Service Set | Any NF Service instance within an equivalent NF Service Set within the NF Set (if applicable) (Note 2) | NF Service Set ID, NF Instance ID, NF Set ID, Service name (NOTE 4) |
| NF Instance | Any equivalent NF Service instance within the NF instance. | Any equivalent NF Service instance within a different NF instance within the NF Set (if applicable) | NF Instance ID, NF Set ID, Service name (NOTE 4) |
| NF Set | Any equivalent NF Service instance within the indicated NF Set | Any equivalent NF Service instance within the NF Set | NF Set ID |
| NOTE 1: if the Binding Indication is not available, the NF Consumer/SCP routes the service request to the target based on routing information available. NOTE 2: NF Service Sets in different NFs are considered equivalent if they include same type and variant (e.g. identical NF Service Set ID) of NF Services. NOTE 3: If a Routing Binding Indication is not available, the SCP routes the service request to the target based on available routing information. NOTE 4: The service name is only applicable if the Binding Indication relates to a notification target. In addition, the service name is only applicable in combination with one of the other values for binding. NOTE y: NF service instance Id is not relevant if the Binding Indication relates to a notification target (notification endpoint is included in the subscription request). |  |  |  |

*** NEXT CHANGE ***

### 7.1.2	NF Service Consumer - NF Service Producer interactions

The end-to-end interaction between two Network Functions (Consumer and Producer) within this NF service framework follows two mechanisms, irrespective of whether Direct Communication or Indirect Communication is used:

-	"Request-response": A Control Plane NF_B (NF Service Producer) is requested by another Control Plane NF_A (NF Service Consumer) to provide a certain NF service, which either performs an action or provides information or both. NF_B provides an NF service based on the request by NF_A. In order to fulfil the request, NF_B may in turn consume NF services from other NFs. In Request-response mechanism, communication is one to one between two NFs (consumer and producer) and a one-time response from the producer to a request from the consumer is expected within a certain timeframe. The NF Service Producer may also add a Binding Indication (see clause 6.3.1.0) in the Response, which may be used by the NF Service Consumer  to select suitable NF service producer instance(s)  for subsequent requests. For indirect communication, the NF Service Consumer should copy the Binding Indication into the Routing Binding indication, that is included in subsequent requests, to be used by the SCP to discover a suitable NF service producer instance(s).

Figure 7.1.2-1: "Request-response" NF Service illustration

-	"Subscribe-Notify": A Control Plane NF_A (NF Service Consumer) subscribes to NF Service offered by another Control Plane NF_B (NF Service Producer). Multiple Control Plane NFs may subscribe to the same Control Plane NF Service. NF_B notifies the results of this NF service to the interested NF(s) that subscribed to this NF service. The subscription request shall include the notification endpoint, i.e. Notification Target Address) and a Notification Correlation ID (e.g. the notification URL) of the NF Service Consumer to which the event notification from the NF Service Producer should be sent to.

NOTE 1:	The notification endpoint URL can contain both the notification endpoint and the Notification Correlation ID.

The NF Service Consumer may add a Binding Indication (see clause 6.3.1.0) in the subscribe request, which may be used by the NF Service Producer  to discover a suitable notification endpoint. For indirect communication, the NF Service Producer should copy the Binding Indication into the Routing Binding indication, that is included in the response, to be used by the SCP to discover a suitable notification endpoint. The NF Service Producer may also add a Binding Indication (see clause 6.3.1.0) in the subscribe response, which may be used by the NF Service Consumer (or SCP) to select suitable NF service producer instance(s). In addition, the subscription request may include notification request for periodic updates or notification triggered through certain events (e.g., the information requested gets changed, reaches certain threshold etc.). The subscription for notification can be done through one of the following ways:

-	Explicit subscription: A separate request/response exchange between the NF Service Consumer and the NF Service Producer; or

-	Implicit subscription: The subscription for notification is included as part of another NF service operation of the same NF Service; or

-	Default notification endpoint: Registration of a notification endpoint for each type of notification the NF consumer is interested to receive, as a NF service parameter with the NRF during the NF and NF service Registration procedure as specified in TS 23.502 [3] clause 4.17.1.

The NF Service Consumer may also add a Binding Indication (see clause 6.3.1.0) in the response to the notification request, which may be used by the NF Service Producer  to discover a suitable notification endpoint. For indirect communication, the NF Service Producer should copy the Binding Indication into the Routing Binding indication, that is included in the response, to be used by the SCP to discover a suitable notification endpoint.

Figure 7.1.2-2: "Subscribe-Notify" NF Service illustration 1

A Control Plane NF_A may also subscribe to NF Service offered by Control Plane NF_B on behalf of Control Plane NF_C, i.e. it requests the NF Service Producer to send the event notification to another consumer(s). In this case, NF_A includes the notification endpoint, i.e. Notification Target Address) and a Notification Correlation ID, of the NF_C in the subscription request. NF_A may also additionally include the notification endpoint and a Notification Correlation ID of NF A associated with subscription change related Event ID(s), e.g. Subscription Correlation ID Change, in the subscription request, so that NF_A can receive the notification of the subscription change related event. The NF_A may add Binding Indication (see clause 6.3.1.0) in the subscribe request.

Figure 7.1.2-3: "Subscribe-Notify" NF Service illustration 2

Routing of the messages for the NF interaction mechanisms above may be direct, as shown in the figures 7.1.2-1 to 7.1.2-3, or indirect. In case of Indirect Communication, an SCP is employed by the NF service consumer. The SCP routes messages between NF service consumers and NF service producers and may do discovery and associated selection of the NF service producer on behalf of a NF service consumer. Figure 7.1.2-4 shows the principle for a request-response interaction and figure 7.1.2-5 shows an example of a subscribe-notify interaction.

Figure 7.1.2-4: Request response using Indirect Communication

Figure 7.1.2-5: Subscribe-Notify using Indirect Communication

NOTE:	The subscribe request and notify request can be routed by different SCPs.

*** END CHANGES ***