SA WG2 Meeting #136AH	S2-2000219

13 - 17 January, 2020, Incheon, South Korea	(revision of S2-200xxxx)

Source:	Intel

Title:	UE-to-Network Relay KI update

Document for:	Agreement

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes to update the KI of UE-to-Network Relay.

# 1.	Discussion

Following aspects are missing from KI#3:

- How to authorize a UE to be a UE-to-Network Relay;

- How to authorize a UE to access 5GC via a UE-to-Network Relay.

* * * * Start of the 1st Change * * * *

## 5.3	Key Issue #3: Support of UE-to-Network Relay

### 5.3.1	General description

According to TS 22.261 [3] and TS 22.278 [2], support for UE-to-Network Relay needs to be studied. In addition, the Rel-16 5G architectural design (e.g. flow-based QoS communication over PC5/Uu interface) shall be taken into consideration as well.

The case that UE may be able to access to network via the direct or indirect Uu path illustrated in figure 5.3.1-1 needs to be considered, where path #1 is direct Uu path that may not exist, as well as path #2 and path #3 are indirect Uu paths via different UE-to-Network Relays.

Figure 5.3.1-1: Example scenario of direct or indirect Uu path between UE and Network

Therefore, 5G ProSe needs to support UE-to-Network Relay. In particular, the following aspects need to be studied:

-	How to authorize a UE to be a UE-to-Network Relay and how to authorize a UE to access 5GC via a UE-to-Network Relay.

-	How to establish a connection between Remote UE and a UE-to-Network Relay to support connectivity to the network for the Remote UE.

-	How to support end-to-end requirements between Remote UE and the network via a UE-to-Network Relay, including QoS (such as data rate, reliability, latency) and the handling of PDU Session related attributes (e.g. S-NSSAI, DNN, PDU Session Type and SSC mode).

-	How to transfer data between the Remote UE and the network over the UE-to-Network Relay.

NOTE 1: Security and privacy aspects will be handled by SA WG3.

-	How to (re)select a UE-to-Network Relay for communication path selection between two indirect Uu paths (i.e. path #2 and path #3 in figure 5.3.1-1).

-	How to perform communication path selection between a direct Uu path (i.e. path #1 in figure 5.3.1-1) and an indirect Uu path (i.e. path #2 or path #3 in figure 5.3.1-1) via a UE-to-Network Relay.

-	How to perform communication path switch between a direct Uu path and an indirect Uu path via a UE-to-Network Relay, or between two indirect Uu paths via different UE-to-Network Relays. And how to guarantee service continuity during these communication path switch procedures.

NOTE 2:	Support of non-unicast mode communication (i.e. one-to-many communication/broadcast or multicast) between network and UE-to-Network Relay UE and between UE-to-Network Relay and Remote UE(s) depends on the result of FS_5MBS work.

Two cases can be considered regarding support of UE-to-Network Relay, i.e., UE-to-Network Relay served by gNB as shown in Figure 5.3.1-2 and UE-to-Network Relay served by ng-eNB as shown in Figure 5.3.1-3.

Figure 5.3.1-2: UE-to-Network Relay served by gNB

Figure 5.3.1-3: UE-to-Network Relay served by ng-eNB

NOTE 3:	Whether to support the case that a UE-to-Network Relay is served by ng-eNB depends on solution to be identified in this study and RAN decision.

* * * * End of Changes * * * *