3GPP TSG SA WG2 Meeting #136-AH	S2-2000269

13th – 17th January 2020, Incheon, KR

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1973 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing Non-UE N2 Message Services Operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2020-01-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Section 5.2.2 describes AMF services and AMF Service operations. However, Non-Ue Message Service operations (NonUeN2MessageTransfer, NonUeN2InfoSubscribe, NonUeN2InfoUnSubscribe, NonUeN2InfoNotify) belonging to Namf_Communication Service are missing. NonUeN2MessageTransfer Service operation are used by: - Peer AMF. See Configuration Transfer procedure (Clause 5.26 of 3GPP TS 23.501) - CBCF/PWS-IWF. See Public Warning System Service (Clause 9A of 3GPP TS 23.041) - LMF. See Obtaining Non-UE Associated Network Assistance Data (Clause 6.11.3 of 3GPP TS 23.273) - Peer AMF. RIM (RAN Information Management) Information Transfer (Clause 5.36 of 3GPP TS 23.501) NonUeN2InfoSubscribe/NonUeN2InfoUnSubscribe/ NonUeN2InfoNotify Service operation are used by: - CBCF/PWS-IWF. See Public Warning System Service (Clause 9A of 3GPP TS 23.041) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Non-Ue Message Services operations (NonUeN2MessageTransfer, NonUeN2InfoSubscribe, NonUeN2InfoUnSubscribe, NonUeN2InfoNotify) are included as part of AMF services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1, 5.2.2.17, 5.2.2.18, 5.2.2.19, 5.2.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | These changes are also applicable for Rel-15 specification. |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

#### 5.2.2.1	General

The following table shows the AMF Services and AMF Service Operations.

Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) |
| --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Request/ Response | Peer AMF |
|  | CreateUEContext | Request/ Response | Peer AMF |
|  | ReleaseUEContext | Request/ Response | Peer AMF |
|  | RegistrationCompleteNotify | Subscribe / Notify | Peer AMF |
|  | N1MessageNotify | Subscribe / Notify | SMF, SMSF, PCF, LMF, Peer AMF |
|  | N1MessageSubscribe |  | SMF, SMSF, PCF |
|  | N1MessageUnSubscribe |  | SMF, SMSF, PCF |
|  | N1N2MessageTransfer | Request/ Response | SMF, SMSF, PCF, LMF |
|  | N1N2TransferFailureNotification | Subscribe / Notify | SMF, SMSF, PCF, LMF |
|  | N2InfoSubscribe | Subscribe / Notify | NOTE 1 |
|  | N2InfoUnSubscribe |  | NOTE 1 |
|  | N2InfoNotify |  | AMF, LMF |
|  | EBIAssignment | Request/Response | SMF |
|  | AMFStatusChangeSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeUnSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeNotify | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | NonUeN2MessageTransfer | Request/ Response | Peer AMF, CBCF, PWS-IWF, LMF |
|  | NonUeN2InfoSubscribe | Subscribe / Notify | CBCF, PWS-IWF, LMF |
|  | NonUeN2InfoUnSubscribe |  | CBCF, PWS-IWF, LMF |
|  | NonUeN2InfoNotify |  | CBCF, PWS-IWF, LMF |
| Namf_EventExposure | Subscribe | Subscribe / Notify | NEF, SMF, UDM, NWDAF |
|  | Unsubscribe | Subscribe / Notify | NEF, SMF, UDM, NWDAF |
|  | Notify | Subscribe / Notify | NEF, SMF, UDM, NWDAF |
| Namf_MT | EnableUEReachability | Request/Response | SMSF |
|  | ProvideDomainSelectionInfo | Request/Response | UDM |
| Namf_Location | ProvidePositioningInfo | Request/Response | GMLC |
|  | EventNotify | Subscribe / Notify | GMLC |
|  | ProvideLocationInfo | Request/Response | UDM |
|  | CancelLocation | Request/Response | GMLC |
| NOTE 1: In this Release of the specification no known consumer is identified to use this service operation. |  |  |  |

* * * 2nd Change * * * *

##### 5.2.2.2.16	Namf_Communication_AMFStatusChangeNotify service operation

Service operation name: Namf_Communication_AMFStatusChangeNotify

Description: Report AMF Status change (e.g. AMF unavailable) notification to subscribed NFs.

Input, Required: GUAMI(s).

Input, Optional: Target AMF(s) Name associated with the indicated GUAMI.

Output, Required: None.

Output, Optional: None.

See clause 5.21.2.2, TS 23.501 [2] for the example usage of this service operation. The GUAMI(s) is used to identify the AMF. For network deployment without UDSF case, the target AMF Name which is to serve the user of the indicated GUAMI is also included.

##### 5.2.2.2.17	Namf_Communication_ NonUeN2MessageTransfer service operation

Service operation name: Namf_Communication_NonUeN2MessageTransfer.

Description: The NF Service Consumer requests to transfer a non-UE specific message to NG-RAN node(s) via N2.

Input, Required: N2 Message Container.

Input, Optional: TAI List, RAT Selector (ng-eNB or gNB), Global RAN Node List.

Output, Required: Result indication.

Output, Optional: Unknown Tracking Area List.

If the result of the service operation fails, the AMF shall set the corresponding cause value in the result indication which can be used by the NF consumer for further action.

##### 5.2.2.2.18	Namf_Communication_ NonUeN2InfoSubscribe service operation

Service operation name: Namf_Communication_ NonUeN2InfoSubscribe.

Description: The NF Service Consumer invokes this service operation to subscribe to the AMF for notifying non UE specific N2 information of a specific type.

Input, Required: N2 information type to be subscribed, Notification Callback URI.

Input, Optional: Global RAN Node List, Access Type List, NF Id.

Output, Required: Subscription Id.

Output, Optional: None.

##### 5.2.2.2.19	Namf_Communication_ NonUeN2InfoUnSubscribe service operation

Service operation name: Namf_Communication_ NonUeN2InfoUnSubscribe.

Description: The NF Service Consumer invokes this service operation to unsubscribe to the AMF to stop notifying N2 information of a specific type.

Input, Required: None.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

##### 5.2.2.2.20	Namf_Communication_ NonUeN2InfoNotify service operation

Service operation name: Namf_Communication_ NonUeN2InfoNotify.

Description: The AMF uses this service operation to notify a particular event towards the NF Service Consumer that has subscribed for the specific information.

Input, Required: Notification Subscription Id, N2 information.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

* * * End Of Change * * * *