SA WG2 Meeting #S2-136AH	S2-2000544

13 - 17 January, 2020, Incheon, South Korea	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2064 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Item#4: Clarification on the PSFP and Qbv for TSC traffic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-1-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the existing specification, the TSN AF can receive 3 kind of configuration (Qbv, PSFP, Forwarding rule) from CNC as defined in 23.501 clause 5.28.2 5GS Bridge configuration. But how the TSN AF use these information to identify the PDU session, create the TSN Qos information is not clear. In the Discussion paper, it clarify how TSN AF uses these configuraiotn to derive the TSN Qos information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TSN AF use the PSFP and Qbv to derive the TSN Qos information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | how the TSN AF use configuration (Qbv, PSFP, Forwarding rule) from CNC to identify the PDU session, create the TSN Qos information is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.28.2	5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session. The QoS parameters mapping for TSN is described in TS 23.503 [45] clause 6.1.3.23.

The configuration information of 5GS Bridge as defined in IEEE 802.1Qbv [96] and in IEEE 802.1Q [98], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT:

-	Egress ports of 5GS Bridge, e.g., ports on DS-TT and NW-TT;

-	Traffic classes and their priorities.

NOTE:	In this Release of the specification, only support simplified IEEE 802.1Qbv [96], Annex Q.2 for 5GS.

The configuration information of 5GS Bridge as defined in IEEE 802.1Q [98], includes the following:

-	Chassis ID of 5GS Bridge;

-	Traffic forwarding information as defined in IEEE 802.1Q [98] clause 8.8.1:

-	Destination MAC address and VLAN ID of TSN stream;

-	Port number in the Port MAP as defined in IEEE 802.1Q [98] clause 8.8.1.

-	Configuration information per stream according to IEEE 802.1Q [98] clause 8.6.5.1:

-	Ingress port number of 5GS Bridge, i.e., ports on DS-TT/NW-TT;

-	Stream priority.

The SMF report the MAC address of the DS-TT port of the related PDU Session to AF via PCF as the MAC address of the PDU Session. The association between the MAC address used by the PDU Session, 5GS Bridge ID and port number on DS-TT is maintained at AF and further used to assist to bind the TSN traffic with the UE's PDU session.

In the case of provisioning traffic routing information based the information above, the TSN AF determines the MAC address used by the PDU Session for the TSN traffic based on ingress port number or egress port number, and requests to PCF influence traffic routing for TSN traffic as defined in clause 5.6.7 with the MAC address of the PDU Session, traffic class, destination MAC address and VLAN ID.

With the IEEE 802.1Q [98] clause 8.8.1 and IEEE 802.1Q [98] clause 8.6.5.1, the TSN AF identifies the Ingress port, Egress port and corresponding PDU session for one stream. The TSN AF further use the Configuration information of scheduled traffic in IEEE 802.1Qbv [96] to derive the TSN Qos information for each PDU session in the Egress port.

NOTE:	How the TSN AF derives the TSN Qos information from CNC configuration is implementation.

END OF CHANGES