3GPP TSG-SA WG2 Meeting #136-AH	S2-2000664

# Incheon, Korea, January 13-17 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2085 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TSN stream aggregation and QoS parameter mapping guidelines |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel--16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It addresses the following FASMO items: #22 TSC Burst Size calculation and maximum flow bitrate has not been stated in TS 23.501, new Annex I. #25 Correction to Annex I – It states that the AF aggregates the TSN stream(s) but this is incorrect. Factually, AF creates one set of TSN QoS container for multiple TSN streams to enable aggregation of TSN streams [to single QoS flow] “. The aggregation of TSN streams into a single QoS flow actually happens in the UPF based on rules configured by the SMF. - Current text assumes the AF can aggregate TSN streams into a single QoS flow as long as the periodicities align but this doesn’t always work, let us take the following example: 1 - Periodicities are all 30 msec. (adjusting a few orders of magnitude to be a bit more realistic) 2 – TSN Stream-1 starts at 1pm, TSN-Stream-2 starts at 1 pm + 10 msec, and TSN-Stream 3 starts at 1pm+15 msec. With only one BAT, per QoS Flow, it is not possible to aggregate these TSN streams (on the one QoS Flow). The periodicities are off-set. If we had instead the following: TSN Stream-1 starts at 1pm, TSN-Stream-2 starts at 1 pm + 30 msec, and TSN-Stream 3 starts at 1pm+60 msec. Thus it should be clear that the BAT is also compatible. Furthermore, AF is actually not doing the aggregation itself rather it is enabling the aggregation of TSN streams by combining the parameters. In other words, “one set of parameters and one container are being calculated by the AF for multiple TSN streams to enable aggregation of TSN streams [to single QoS flow] “ So, it should be corrected and clarified to be more accurate. - TSN Time Sync Annex still has an Editor’s note that should be removed |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the AF enables aggregation, also add that compatible BAT is needed in order to aggregate TSN streams. - Fix TSN Time Sync Annex to remove the EN and replace with the correct TS reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompatible TSN streams could be aggregated leading to incorrect and suboptimal behavior for QoS flows delivery. Also, EN will remain in the TS (frozen stage 2 for Rel-16) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.1, 5.27.2, H.2, I.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 5.27.1	TSN Time Synchronization

#### 5.27.1.1	General

For supporting TSN time synchronization, the 5GS is integrated with the external network as a TSN bridge as described in clauses 4.4.8 and 5.28.1. It shall be modelled as an IEEE 802.1AS [104] compliant entity according to TS 22.104 [105]. For TSN Synchronization, the entire E2E 5G system can be considered as an IEEE 802.1AS [104] "time-aware system". Only the TSN Translators (TTs) at the edges of the 5G system need to support the IEEE 802.1AS [104] operations. UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock) which shall serve to keep these network elements synchronized. The TTs located at the edge of 5G system fulfil all functions related to IEEE 802.1AS [104], e.g. (g)PTP support, timestamping, Best Master Clock Algorithm (BMCA), rateRatio. Figure 5.27.1-1 illustrates the 5G and TSN clock distribution model via 5GS.

Figure 5.27.1-1: 5G system is modelled as IEEE 802.1AS compliant time aware system for supporting TSN time synchronization

Figure 5.27.1-1 depicts the two synchronizations systems considered: the 5GS synchronization and the TSN domain synchronization.

-	5GS synchronization: Used for NG RAN synchronization. 5G RAN synchronization is specified in TS 38.331 [28].

-	TSN domain synchronization: Provides synchronization service to TSN network. This process follows IEEE 802.1AS [104].

The two synchronization processes can be considered independent from each other and the gNB only needs to be synchronized to the 5G GM clock.

To enable TSN synchronization, the 5GS calculates and adds the measured residence time between the TTs into the Correction Field (CF) of the synchronization packets of the TSN domain.

Next Change

### 5.27.2	TSC Assistance Information (TSCAI)

TSC assistance information describes TSC traffic characteristics for use in the 5G System. The knowledge of TSN traffic pattern is useful for the gNB to allow it to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. TSC assistance information, as defined in Table 5.27.2-1, is provided from SMF to 5G-AN, e.g. upon QoS Flow establishment. The TSCAI parameters are set according to corresponding parameters obtained from the AF. The AF gets per TSN stream traffic characteristics according to IEEE 802.1Q [98] clause 8.6.5.1 for 5GS logical bridge to identify the PDU session for the UL traffic.

The determination of the TSCAI by the 5GC is based on information received from the TSN AF via PCF. The TSN AF is responsible for obtaining PSFP (IEEE 802.1Q [98]) parameters and use them to calculate traffic pattern parameters (such as burst arrival time with reference to the ingress port, periodicity, and flow direction) and responsible of forwarding these parameters to the SMF (via PCF). TSN AF may enable aggregation of TSN streams if the TSN streams belong to the same traffic class, terminate in the same egress port and have the same periodicity and compatible Burst arrival time. One set of parameters and one container are being calculated by the AF for multiple TSN streams to enable aggregation of TSN streams to the same QoS Flow. TSN Streams with different periodicities and incompatible Burst Arrival Times should be mapped to different QoS Flows.

Annex I describe how the traffic pattern information is determined.

NOTE 1:	Further details of aggregation of TSN streams are left for implementation.

In this case, TSN AF creates one TSN QoS container for the aggregated TSN streams. 5GC derives TSCAI on a per QoS Flow basis and send it to 5G-AN. The Burst Arrival Time and Periodicity component of the TSCAI that the SMF signals to the 5G-AN are specified with respect to the 5G clock. The 5GC is responsible for mapping the Burst Arrival Time and Periodicity from a TSN clock (that the TSN stream is based on) to the 5G clock based on the time offset and cumulative rateRatio between TSN time and 5GS time as measured and reported by the UPF. 5GC sets the DL TSCAI Burst Arrival Time as the sum of TSN QoS Burst Arrival Time and CN PDB. 5GC sets the UL TSCAI Burst Arrival Time as the sum of TSN QoS Burst Arrival Time and UE-DS-TT Residence Time. 5GC sets the Periodicity as the TSN QoS Periodicity.

NOTE 2:	In order to get Burst Arrival Time, Periodicity on a per TSN stream basis, support for IEEE 802.1Q [98] (as stated in clause 4.4.8.2) Per-Stream Filtering and Policing (PSFP) with stream gate operation is a prerequisite.

In the case of drift between TSN time and 5G time, the UPF updates the offset to SMF. In the case of change of cumulative rateRatio between TSN time and 5G time, the UPF updates the cumulative rateRatio to SMF. The SMF may then trigger a PDU Session Modification as defined in TS 23.502 [3] clause 4.3.3 in order to update the TSCAI parameter to the NG-RAN without requiring AN or N1 specific signalling exchange with the UE.

NOTE 3:	In order to prevent frequent updates from the UPF, the UPF sends the offset or the cumulative rateRatio only when the difference between the current measurement and the previously reported measurement is larger than a threshold.

Table 5.27.2-1: TSC Assistance Information

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two bursts. |
| Burst Arrival time | The arrival time of the data burst at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). |

Next Change

Annex H (normative):
TSN usage guidelines

# H.1	General

This Annex provides guidelines on the use of certain specific IEEE parameters and protocol messages in case of TSN as described in clause 5.27.

# H.2	Signalling of ingress time for time synchronization

The ingress time is provided from the NW-TT/UPF to the DS-TT/UE as part of a gPTP Sync or Follow_up message using the Suffix field defined in clause 13.4 of IEEE 1588-2008 [107]. The structure of the Suffix field follows the recommendation of clause 14.3 of IEEE 1588-2008 [107], with an organizationId specific to 3GPP, an organizationSubType referring to an ingress timestamp, and data field that can carry the originTimestamp datatype of the Sync message defined in clause 13.6.1 of IEEE 1588-2008 [107]. TS 24.535[xx] specifies the fields in the gPTP Sync message.

Next Change

Annex I (normative):
TSN usage guidelines

# I.1	Determination of traffic pattern information using PSFP

As described in clause 5.27.2, the calculation of the TSCAI relies upon mapping of information for the TSN stream(s) based upon certain information.

The traffic pattern parameter determination based on PSFP (IEEE P802.1Q [98]) is as follows:

-	Periodicity of a TSN stream is set equal to PSFPAdminCycleTime if there is only one PSFPGateControlEntry with a PSFPgateStates value set to Open in the PSFPGateControlList. If there is more than one PSFPGateControlEntry with a PSFPgateStates value set to Open in the PSFPGateControlList, then the Periodicity of the TSN Stream is set equal to sum of the timeIntervalValues from the first gate open instance to a next gate open instance in the PSFPGate Control List.

-	Burst Arrival time of a TSN stream at the ingress port is determined based on the following conditions:

-	The Burst Arrival Time of a TSN Stream should be set to PSFPAdminBaseTime plus the sum of the timeIntervalValues for which the PSFPgateStatesValue is Closed in the PSFPgateControlList until the first gate open time (i.e. until PSFPgateStatesValue set to Open is found). If the PSFPgateStatesValue is Open for the first timeIntervalValue, then the Burst Arrival time is set to PSFPAdminBaseTime.

NOTE:	TSN Streams that have the same periodicity and compatible Burst Arrival Times as determined above may be aggregated into a single QoS Flow. TSN Streams with different periodicities and incompatible Burst Arrival Times should be mapped to different QoS Flows.

-	Burst Size of a TSN stream at the ingress port is determined based on the following conditions:

-	The Burst Size may be determined from TSN Stream gate control operations in the TSC stream gate control list specified in IEEE 802.1Qci. If in the PSFPAdminControlList, IntervalOctetMax is provided for a PSFPGateControlEntry entry with an “open” PSFPgateStatesValue, the Burst Size may be set to the IntervalOctetMax for that control list entry. If IntervalOctetMax is not provided, the Burst Size may be set to the TimeInterval of the PSFPGateControlEntry entry with the an “open” PSFPgateStatesValue multiplied by the port speed.

-	When multiple compatible TSN Streams are aggregated within a TSC QoS Flow, the Burst Size may be set to the sum of the Burst Sizes for each TSN stream as determined above.

-	Flow direction of a TSN stream is determined based on the following conditions:

-	If the ingress port PSFP information is targeted for a DS-TT port, the Flow direction is UL. If the ingress port PSFP information is targeted for a NW-TT port, the Flow direction is DL. For TSN streams aggregated to a single QoS Flow, the flow direction of any TSN stream in the group is used for the aggregated flow.

End Change