SA WG2 Meeting #136-AH	S2-191xxxx

13 - 17 January 2020, Incheon , KR

Source:	Nokia, Nokia Shanghai Bell

Title:	eNPN- update of Solution 1 Standalone NPN supporting SPs

Document for:	Discussion/Approval

Agenda Item:	6.x.x

Work Item / Release:	FS_eNPN

Abstract of the contribution: This contribution proposes update to solution #1 Standalone NPN supporting SPs

# 1		Discussion

Reason for updates:

- SP should include UDM and AUSF at the minimum.

- Storage of subscription and credentials are in the UDM/UDR.

- Rel-16 UE with SNPN access mode should be supported by this architecture in a backward compatible manner.

- Unclear why SPID is required?

We proposed to capture the following solution in TR 23.700-07.

*** Start Change ***

## 6.1	Solution #1: Standalone non-public network supporting service providers

### 6.1.1	Introduction

The solution addresses key issue #1 (Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN).

Figure 6.1.1-1: Standalone Non-public network supporting service providers

NOTE:	The entity separate from the SNPN is referred to as service provider. It can be either a PLMN or an SNPN. In the case service provider is an SNPN, SNPN owns a 5GC including at least AUSF and UDM. In case the service provider is a PLMN, PLMN owns 5GC including at least AUSF and UDM.

The following are the main principles of the solution:

-	In figure 6.1.1-1, the standalone non-public network (SNPN) takes the role of a visited network and the service provider is the home network.

-	The Service Provider owns the subscription of the UE(s) and Standalone NPN provides access and connectivity for the UE.

-	For each subscription with a service provider, the UE is configured with an "Equivalent home network" (equivalent service provider) list that assists the UE in network selection.

### 6.1.2	Functional Description

Figure 6.1.2-1: Standalone Non-public network supporting service continuity

The solution has the following assumptions:

-	The service provider has an agreement with the standalone non-public network.

-	A standalone non-public network has control plane and optionally user plane interfaces with one or more service providers. At the minimum, 5GC within SNPN interfaces with the AUSF, UDM within the Service Provider. Presence of other interfaces depends on the NFs present in the SNPN and this is left up to deployment choice.

-	SNPN access mode in the UE as defined in Rel-16 is not required to be supported because the network selection is controlled via a configured list which includes both PLMN ID and combination of PLMN ID and NID. However, Rel-16 UE(s) with SNPN access mode is expected to be supported by this architecture in a backward compatible manner.

Editor’s note: support for cell re-selection without SNPN AM in the UE is FFS.

The solution is based on the following principles:

-	The service provider identifier may be a PLMN ID or PLMN ID+NID.

Editor’s note: need for SPID in addition to PLMN ID, PLMN ID + NID for this architecture to be scalable for all deployments is FFS.

-	When the service provider of the UE is a PLMN, the UE subscription data and credentials are stored at the UDM/UDR of the PLMN. This is represented in Figure 6.1.2-1, where UE#A has subscription with SP#1 and the UE is authorized to services provided by SP#1.

-	The stand-alone non-public network can provide local services to the UE using the local breakout architecture or provide access to the service provider services using the home-routed roaming architecture.

-	When the service provider of the UE is a PLMN, the standalone non-public network selects the session management function and PDU Session Anchor (PSA) in the PLMN to support PDU sessions that require service continuity using the home-routed roaming architecture.

-	Mobility in the absence of the N14 interface between SNPN and PLMN is handled by the "Existing PDU Session" indication in the PDU Session Establishment Request.

-	When the service provider of the UE is a SNPN, the UE subscription data and credentials are maintained by the SNPN. This is represented in Figure 6.1.2-1, where UE#B has subscription with SP#2 and the UE is authorized to services provided by SP#2.

-	The NG-RAN nodes in the standalone non-public network broadcasts the following:

-	List of combined PLMN IDs and NIDs identifying the standalone non-public networks to which the NG-RAN provides access.

-	The UE shall be configured with the following:

-	Subscriber identifier and credentials for the subscribed service provider (which is identified by PLMN ID or a combination of PLMN ID and NID).

-	An "Equivalent Home Network list" which is modelled based on the EHPLMN list defined in TS 23.122 [5]. This list can include both PLMN ID and combination of PLMN ID and NID ranked in priority order. Alternatively, the UE may be configured with an "Operator Controlled Network Selector with Access Technology" list which is modelled based on the Operator Controlled PLMN Selector with Access Technology list defined in TS 23.122 [5].

### 6.1.3	Procedures

The UE configuration is described in clause 6.1.2.

The principles for standalone non-public network discovery and selection with support for service providers is as given below:

-	UE has subscription with a service provider identified by its SPID (For example - identified as SP#X).

-	SP#X has an agreement with standalone non-public network that is identified by its NID (For example - identified as NPN#A), allowing the UEs that have subscription with SP#X to access the services of SP#X via NPN#A.

-	The UE has a stored "Equivalent Home Network" list or "Operator Controlled Network Selector with Access Technology" list that contains both SP#X and NPN#A.

-	The radio access network of NPN#A broadcasts its own ID i.e., NPN#A.

-	The UE which has subscription with SP #X selects NPN #A since NPN #A is configured in UE's "Equivalent Home Network" list or "Operator Controlled Network Selector with Access Technology" list.

-	Once the network selection is complete, the UE registers with the network by providing the UE identity and credential corresponding to the SPID. It is assumed that the authentication request is routed to UDM in the service provider's administrative domain.

### 6.X.4	Impacts on existing entities and interfaces

Editor's note:	This clause lists impacts to existing entities and interfaces.