3GPP TSG-WG SA2 Meeting #136-AH 	S2-2000753

# Incheon, Korea, January 13 – 17, 2020	(revision of S2-191xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0109 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE mobility analytics type by NWDAF service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-12-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - If AF is in trusted domain, the NWDAF can retrieve service data from AF directly not via NEF, hence, the UE ID in Table 6.7.2.2-2 could also be SUPI or internal UE ID, and the collected UE mobility can be applicable to all UE applications if the application Id is omitted; - When an NF service consumer requests to retrieve UE mobility information by using NWDAF services, if target of analyticis reporting is a single UE or a group of UEs, Analytics Filter Information is unnecessary to include a list of TA or Cells, if it is included, the analytics request is applicable to any UE within the list of TA/Cells. However, currently, UE mobility analytics type is not applicable to any UE case. - For output analytics result, it’s possible that not all the indicated ratio UEs within the group ID or the list of TA/Cells own the same analytics mobility result, i.e. a list of (predicted) time slots. For example, within the group or the list of TA/Cells, UE 1, UE 3 and UE 5 own the same list of (predicted) time slots, and UE 2, UE 10 and UE 12 own the same list of (predicted) time slots. However, no such possibility described in TS 23.288 yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct that UE ID in Table 6.7.2.2-2 could also be SUPI or internal UE ID; - Calrify that UE mobility analytics type is applicable to any UE if Analytics Filter Information contains a list of TA/Cells; - Clarify that it’s possible that not all the indicated ratio UEs within the group ID or the list of TA/Cells own the same analytics mobility result. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirement of UE mobility |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.1; 6.7.2.2; 6.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM, and to perform data analytics to provide UE mobility statistics or predictions.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs or any UE.

-	Analytics Filter Information containing optional list of TA or Cells, optional maximum number of results

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

If a list of TA or Cells are indicated as Analytics Filter Information, the target of analytics reporting is applicable to any UE.

# * * * * Second change * * * *

6.7.2.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information as defined in the Table 6.7.2.2-1;

Table 6.7.2.2-1: UE Mobility information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar mobility behavior. The UE whose mobility behavior is unlike other UEs with the same TAC may be an abnormal one. |
| Frequent mobility re-registration | AMF | A UE (e.g. a stationary UE) may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple re-registrations if the cells belong to different registration areas. The number of re-registrations N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.7.2.2-2;

Table 6.7.2.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI, SUPI or internal/external UE ID |
| Application ID (Optional) | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical area that the UE enters |
| >Timestamp | A time stamp when UE enters this area |

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

# * * * * Third change * * * *

6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID(s) or any UE | Identifies a UE or more than one UE or a group of UEs or any UE, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI(s) or any UE. (NOTE) |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot (average and variance) |
| > UE location (1..max) | Observed location statistics |
| >> UE location | TA or cells which the UE stays |
| >> Ratio | Percentage of UEs in the group (in case of an UE group) |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID(s) or any UE | Identifies an UE or more than one UE or a group of UEs or any UE, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI(s) or any UE. (NOTE) |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location prediction during the Analytics target period |
| >> UE location | TA or cells where the UE or UE group may move into |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in case of an UE group) |

NOTE:	When target of analytics reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. When target of analytics reporting is a group of UEs, UE group ID will be included if all the indicated Ratio UEs own the same analytics mobility result, otherwise, more than one list of (predicted) time slots will be provided to the NF service consumer(s), the UE IDs within the indicated Ratio of the group which own the same analytics mobility result will also be included. When target of analytics reporting is any UE, no UE ID will be included if all the indicated Ratio UEs own the same analytics mobility result, otherwise, more than one list of (predicted) time slots will be provided to the NF service consumer(s), the UE IDs within the group which own the same analytics mobility result will also be included.

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of results provided as input parameter.

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

# * * * * End of changes * * * *