3GPP TSG-SA2 Meeting #136AH	S2-2000777

# 13-17 Jan. 2020 Incheon, South Korea

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2096 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple TSN domains (FASMO#6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FASMO#6: Does UPF only need to generate gPTP packets on N6 or also towards UEs (see NOTE 2 in clause 5.27.1.3 in TS 23.501 and S2-1911359? In NOTE 2, it only describes the UPF/NW-TT propagates the 5G clock using the 802.1AS profile to TSN networks connecting to UPF/NW-TT (i.e. UL direction) but it does not describe how the TSN devices connected behind the UE/DS-TT (i.e. DL direction). However, since gPTP packets for TSN clock distribution have multicast MAC destination addresses to ensure those gPTP packets are sent to all egress ports in IEEE 802.1AS, it shoule be applied to all TT outputs in DS-TTs and NW-TTs even under the NOTE 2 condition where TSN time synchronization is based on 5G clock. Furthermore, in clause 5.27.1.2.1, it describes the 5G internal system clock shall be made available to DS-TT by the UE. Therefore, it implies under NOTE 2 condition, DS-TT can forward the gPTP messages received from the UE and NW-TT can forward the gPTP messages from UPF to TSN networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the NOTE 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description for the NOTE 2 leading to confusion and likely wrong implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

#### 5.27.1.3	Support for multiple TSN working domains

Each TSN working clock domain sends its own gPTP messages. The related Ethernet frames carry the gPTP multicast Ethernet destination MAC address and the gPTP message carries a specific PTP "domainNumber" that indicates the time domain they are referring to. The NW-TT makes ingress timestamping (TSi) for the gPTP event messages of all domains.

A UE receives gPTP messages and forwards them all to the DS-TT. The DS-TT receives the original TSN clock timing information and the corresponding TSi via gPTP messages for one or more working domains. The DS-TT then makes egress timestamping (TSe) for the gPTP event messages for every external TSN working domain. Ingress and egress time stamping is based on the 5G system clock at NW-TT and DS-TT.

NOTE 1:	An end-station can select TSN timing information of interest based on the "domainNumber" in the gPTP message.

The process described in "Distribution of TSN clock and time-stamping" is thus repeated for each TSN working domain between a UE and the UPF it is connected to. When a UE is connected to multiple TSN working domains via the same UPF, then all gPTP messages from these domains are forwarded to the UE via the same PDU session.

NOTE 2:	If all TSN working domains can be made synchronous and the synchronization can be provided by the 5G clock, all the TT output ports at the UPF side (NW-TT) and at the UE side (DS-TT) towards the connected TSN networks propagate the 5G clock using the 802.1AS profile (i.e. the 5G system as an IEEE 802.1AS [104] compliant time-aware system). The time synchronization of 5G clock is made available to the NW-TT by the UPF and to the DS-TT by the UE.

**** END OF CHANGES ****