3GPP TSG-WG SA2 Meeting #136AH 	S2-2000958

# Incheon, South Korea, January 13 – 17, 2020	(revision of S2-2000413)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0075 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of PC5 Link Identifier description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently for unicast communicaton the recently introduced PC5 Link Identifier is used by the UE V2X layer to internally identify different unicast links (see clauses 5.2.1 and 6.3.3).The V2X layer only passes the PC5 Link Identifier for the UE AS. As an alternative option, this is also possible that for unicast, the V2X layer can still pass the Layer 2 IDs to AS layer along the V2X packets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description about using Layer 2 IDs for unicast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | restricted parameters for unicast from V2X layer to AS layer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3.3	Unicast mode V2X communication over PC5 reference point

6.3.3.1	Layer-2 link establishment over PC5 reference point

To perform unicast mode of V2X communication over PC5 reference point, the UE is configured with the related information as described in clause 5.1.2.1.

Figure 6.3.3.1-1 shows the layer-2 link establishment procedure for unicast mode of V2X communication over PC5 reference point.

Figure 6.3.3.1-1: Layer-2 link establishment procedure

1.	The UE(s) determine the destination Layer-2 ID for signalling reception for PC5 unicast link establishment as specified in clause 5.6.1.4. The destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.2.1.

2.	The V2X application layer in UE-1 provides application information for PC5 unicast communication. The application information includes the V2X service type(s) (e.g. PSID(s) or ITS-AID(s)) of the V2X application and the initiating UE's Application Layer ID. The target UE's Application Layer ID may be included in the application information.

The V2X application layer in UE-1 may provide V2X Application Requirements for this unicast communication. UE-1 determines the PC5 QoS parameters and PFI as specified in clause 5.4.1.4.

If UE-1 decides to reuse the existing PC5 unicast link as specified in clause 5.2.1.4, the UE triggers Layer-2 link modification procedure as specified in clause 6.3.3.4.

3.	UE-1 sends a Direct Communication Request message to initiate the unicast layer-2 link establishment procedure. The Direct Communication Request message includes:

-	Source User Info: the initiating UE's Application Layer ID (i.e. UE-1's Application Layer ID).

-	If the V2X application layer provided the target UE's Application Layer ID in step 2, the following information is included:

-	Target User Info: the target UE's Application Layer ID (i.e. UE-2's Application Layer ID).

-	V2X Service Info: the information about V2X Service(s) requesting Layer-2 link establishment (e.g. PSID(s) or ITS-AID(s)).

-	Security Information: the information for the establishment of security.

NOTE 1:	The Security Information and the necessary protection of the Source User Info and Target User Info are defined by SA WG3.

The source Layer-2 ID and destination Layer-2 ID used to send the Direct Communication Request message are determined as specified in clauses 5.6.1.1 and 5.6.1.4. The destination Layer-2 ID may be broadcast or unicast Layer-2 ID. When unicast Layer-2 ID is used, the Target User Info shall be included in the Direct Communication Request message.

UE-1 sends the Direct Communication Request message via PC5 broadcast or unicast using the source Layer-2 ID and the destination Layer-2 ID.

4.	Security with UE-1 is established as below:

4a.	If the Target User Info is included in the Direct Communication Request message, the target UE, i.e. UE-2, responds by establishing the security with UE-1.

4b.	If the Target User Info is not included in the Direct Communication Request message, the UEs that are interested in using the announced V2X Service(s) over a PC5 unicast link with UE-1 responds by establishing the security with UE-1.

NOTE 2:	The signalling for the Security Procedure is defined by SA WG3.

When the security protection is enabled, UE-1 sends the following information to the target UE:

-	If IP communication is used:

-	IP Address Configuration: For IP communication, IP address configuration is required for this link and indicates one of the following values:

-	"IPv6 Router" if IPv6 address allocation mechanism is supported by the initiating UE, i.e., acting as an IPv6 Router; or

-	"IPv6 address allocation not supported" if IPv6 address allocation mechanism is not supported by the initiating UE.

-	Link Local IPv6 Address: a link-local IPv6 address formed locally based on RFC 4862 [21] if UE-1 does not support the IPv6 IP address allocation mechanism, i.e. the IP Address Configuration indicates "IPv6 address allocation not supported".

-	QoS Info: the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

5.	A Direct Communication Accept message is sent to UE-1 by the target UE(s) that has successfully established security with UE-1:

5a.	(UE oriented Layer-2 link establishment) If the Target User Info is included in the Direct Communication Request message, the target UE, i.e. UE-2 responds with a Direct Communication Accept message if the Application Layer ID for UE-2 matches.

5b.	(V2X Service oriented Layer-2 link establishment) If the Target User Info is not included in the Direct Communication Request message, the UEs that are interested in using the announced V2X Service(s) respond to the request by sending a Direct Communication Accept message (UE-2 and UE-4 in Figure 6.3.3.1-1).

The Direct Communication Accept message includes:

-	Source User Info: Application Layer ID of the UE sending the Direct Communication Accept message.

-	QoS Info: the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters requested by UE-1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

-	If IP communication is used:

-	IP Address Configuration: For IP communication, IP address configuration is required for this link and indicates one of the following values:

-	"IPv6 Router" if IPv6 address allocation mechanism is supported by the target UE, i.e., acting as an IPv6 Router; or

-	"IPv6 address allocation not supported" if IPv6 address allocation mechanism is not supported by the target UE.

-	Link Local IPv6 Address: a link-local IPv6 address formed locally based on RFC 4862 [21] if the target UE does not support the IPv6 IP address allocation mechanism, i.e. the IP Address Configuration indicates "IPv6 address allocation not supported", and UE-1 included a link-local IPv6 address in the Direct Communication Request message. The target UE shall include a non-conflicting link-local IPv6 address.

If both UEs (i.e. the initiating UE and the target UE) selected to use link-local IPv6 address, they shall disable the duplicate address detection defined in RFC 4862 [21].

NOTE 3:	When either the initiating UE or the target UE indicates the support of IPv6 router, corresponding address configuration procedure would be carried out after the establishment of the layer 2 link, and the link-local IPv6 addresses are ignored.

The source Layer-2 ID used to send the Direct Communication Accept message is determined as specified in clauses 5.6.1.1 and 5.6.1.4. The destination Layer-2 ID is set to the source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the Direct Communication Accept message from peer UE, UE-1 obtains the peer UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

The V2X layer of the UE that established PC5 unicast link passes the PC5 Link Identifier assigned for the unicast link and  the PC5 unicast link related information down to the AS layer. The PC5 unicast link related information includes Layer-2 ID information (i.e. source Layer-2 ID and destination Layer-2 ID). This enables the AS layer to maintain the PC5 Link Identifier together with the PC5 unicast link related information.

6.	V2X service data is transmitted over the established unicast link as below:

When the V2X layer provides the V2X service data to AS layer, the V2X layer selects one set of parameters to be sent as follows.

-	The PC5 Link Identifier and PFI, or,

-	Source Layer-2 ID, destination Layer-2 ID, cast type and PFI

NOTE 4:	It is up to UE implementation to select which set of parameters to be used.	 It is expected that the UE always selects the same set of parameters for all PC5 unicast links.

UE-1 sends the V2X service data using the source Layer-2 ID (i.e. UE-1's Layer-2 ID for this unicast link) and the destination Layer-2 ID (i.e. the peer UE's Layer-2 ID for this unicast link).

NOTE 5:	PC5 unicast link is bi-directional, therefore the peer UE of UE-1 can send the V2X service data to UE-1 over the unicast link with UE-1.

# * * * * End of changes * * * *