SA WG2 Meeting #136AH	S2-2001698

Incheon, South Korea, January 13 – 17, 2020	(e-mail revision 5 of S2-2001530)

Source:	Huawei, HiSilicon

Title:	New Solution external authentication and authorization

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	[FS_eNPN] / [Rel-17]

Abstract: This contribution introduces a solution to Key Issue #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN.

# 1. Introduction/Discussion

This contribution proposes the address the KI #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN, particularly on the following aspects:

-	How to identify the separate entity providing the subscription;

-	Network selection enhancements, including UEs with multiple subscriptions;

-	e.g. how does the UE discover and select an SNPN which provides authentication in an external entity;

-	Architecture enhancements needed to support multiple SPs e.g.:

-	What are the interfaces exposed and/or used by SNPN and the separate entity;

-	What is the architecture and solution for a UE accessing a separate entity via SNPN access network;

-	How to exchange authentication signalling between the SNPN and the separate entity, including:

-	Authentication by the PLMN, based on PLMN identities and credentials, for access to the SNPN;

-	Authentication via SNPN to separate entity based on non-3GPP identities (e.g. non-IMSI) and credentials;

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-07.

# * * * * First change * * * *

## 6.Y	Solution Y: External Authentication and Authorization

### 6.Y.1	Introduction

This solution addresses the key issue #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN. This solution allows the UE using subscription or credentials owned by an entity separate from the desired SNPN to register and request connectivity at the desired SNPN.

### 6.Y.2	Functional Description

#### 6.Y.2.1	Architecture and Concept

Figure 6.Y.2.1-1: Architecture in case separate entity that owns the subscription is deployed by the vertical

Figure 6.Y.2.1-2: Architecture in case separate entity that owns the subscription is deployed by the PLMN

NOTE 1:	For the sake of simplicity, the figures above do not show all NFs and interfaces.

SP: Service Provider, who provides the AAA-S and the services to the end user. It can be the verticals or PLMN operators.

AAA-Server (AAA-S): entity separate from the desired SNPN but owns UE’s desired SNPN subscription. It is also responsible to authenticate/authorize the UE based on the UE’s SNPN subscription. It is owned by the SP. If the SP is PLMN, the AAA-S can be the AUSF in PLMN.

AAA-P: it acts as the relay for interactions between AUSF and the AAA-S of SP (vertical).

Editor's note:	The interfaces of AAA-P need to be clarified.

AUSF: The AUSF undertakes any AAA protocol interworking with the AAA protocol supported by the AAA-S. The AUSF uses towards the AAA-P or the AAA-S an AAA protocol message of the same protocol supported by the AAA-S.

Nx: Reference point between AUSF and AAA-S of the vertical.

Ny: Service-based interface between AUSF of desired SNPN and AUSF/UDM of PLMN.

Editor's note:	Whether Ny is a new service-based interface or Nausf is used is FFS.

SP-ID: SP Identity, it is used to uniquely identify a SP. SP-ID can be domain name in case SP is the vertical or the PLMN ID in case SP is PLMN operator. The AUSF can address the AAA-S with the SP-ID.

External Subscription (E-Sub): SP subscription (credential/identity), which is stored in AAA-S of the SP. It can be PLMN identities and credentials in case the SP is the PLMN, or be non-3GPP identities (e.g. non-IMSI) and credentials in case the SP is not PLMN. E-Sub is used to authenticate and authorize the UE on the following aspects:

- Access to the services of the SP, and/or

- Access to the desired SNPN that provides connectivity to services of the SP.

Editor's note:	It is FFS whether E-Sub is only used to authenticate Access to the desired SNPN.

Primary Subscription (P-Sub): it is the credential/identity stored at the UDM of the desired SNPN. P-Sub is used to authenticate and authorize the UE for access to the desired SNPN that provides connectivity to services of the SP.

External Authentication and Authorization (EAA): Authentication/Authorization executed by the AAA-S based on UE’s E-Sub.

#### 6.Y.2.2	EAA for the UEs with P-Sub and E-Sub

This clause applies when the UE has both the P-Sub (subscription stored at desired SNPN) and the E-Sub (subscription stored at entity separate from the desired SNPN):

- SNPN supports the P-Sub which contains PLMN ID+NID information for network selection as defined in clause 5.30.2.4 of TS 23.501 [x];

- UE discovers and selects the desired SNPN;

- UE performs registration at the SNPN using SUPI;

- Based on P-Sub data the SNPN triggers the EAA for the UE. The EAA using E-Sub is performed between the UE and the AAA-S of SP via SPNP;

- SNPN and UE establishes the PDU Session to access to SP services;

Editor's note:	It is FFS whether and how to reuse the NSSAA to perform the EAA.

Editor's note:	This clause will be re-visited when SA2 decides whether the E-Sub is only used to authenticate for Access to the desired SNPN.

#### 6.Y.2.3	EAA for the UEs with only E-Sub

This clause applies when the UE is only provisioned with the E-Sub (subscription stored at entity separate from the desired SNPN):

- UE is provisioned with E-Sub that contains the network configuration, which is used for SNPN discovery and selection; the network configuration contains the following information for each desired SNPN:

  - PLMN ID+NID; and

  - Priority; and optionally

  - Subscribed SP ID list;

- SNPN broadcasts the following information:

  - PLMN ID+NID; and

  - support of EAA, and optionally

  - Supported SP-ID list;

Editor's note: Whether a domain name SP ID SP-ID can be broadcast in SIB is FFS.

- UE discovers and selects the desired SNPN based on network configuration and the broadcast information;

Editor's note: Interaction between network configuration for SNPN discovery and SNPN broadcast of supported SP ID list during SNPN selection is FFS.

- UE using E-Sub performs registration to the SNPN.

- Based on local policy the SNPN triggers the EAA for the UE. The EAA is performed between the UE and the AAA-S of SP via SPNP.

- SNPN and UE establishes the PDU Session to access to SP services;

Editor's note:	The further details of the network selection are FFS.

### 6.Y.3	Procedures

#### 6.Y.3.1	EAA for the UEs with P-Sub and E-Sub

In this case, E-Sub is used to authenticate and authorize the UE for access to the services of the SP, P-Sub is used to authenticate and authorize the UE for access to the desired SNPN that provides connectivity to services of the SP. UE selects and registers to the desired SNPN using the P-Sub. After the registration, the AMF triggers the EAA based on P-Sub data. The SNPN only allows the UE to access to the services of the SP for which the SNPN has successful EAA.

Figure 6.Y.3.1-1: EAA of UEs with P-Sub and E-Sub

1.	Steps 1-14a in clause 4.2.2.2.2 of TS 23.502 [y] are performed.

2.	AMF retrieves the P-Sub data from UDM using Nudm_SDM_Get service operation. The P-Sub data includes the indication, SP-ID, AAA-S address for EAA of each SP.

3.	AMF decides to start the EAA based on P-Sub data after registration.

4.	Steps 14c-24 in clause 4.2.2.2.2 of TS 23.502 [y] are performed.

5.	The EAA using E-Sub is performed between the UE and the AAA-S of SP via SPNP. In order to implement this EAA procedure, Network Slice-Specific Authentication and Authorization procedure defined in clause 4.2.9 of TS 23.502 [y] can be used, by changing the Authentication and Authorization from per slice to per service provider, i.e., replacing the S-NSSAI with the SP-ID in the related steps of Network Slice-Specific Authentication and Authorization procedure.

Editor's note:	This clause will be re-visited when SA2 decides whether the E-Sub is only used to authenticate for Access to the desired SNPN.

#### 6.Y.3.2	EAA for the UEs with only E-Sub

In this case, E-Sub is used to authenticate and authorize the UE for access to the services of the SP, as well as for access to the desired SNPN that provides connectivity to services of the SP. For EAA of UEs with only E-Sub, refer to clause 6.Y.3.2. In this case, UE selects and registers to the desired SNPN using network configuration in E-Sub and broadcast system information. Instead of Authentication/Security at AUSM/UDM of SNPN during the registration, the AMF triggers the EAA for UEs using E-Sub based on local policy. The SNPN establishes security context and decides whether to accept the registration based on EAA results.

Figure 6.X.3.2-1: EAA of UEs with only E-Sub

1.	Steps 1-7 in clause 4.2.2.2.2 of TS 23.502 [y] are performed with the following changes:

In Step 1, the UE includes the support for EAA in the AN message and “EAA” Registration Type value in Registration Request message.

Editor's note:	Need for an explicit indication of EAA support and need for a new Registration type are FFS.

In Step 2, the RAN selects an AMF that supports for EAA.

2.	The AMF decides to trigger EAA based on UE request and local policy, and selects an AUSF that supports for exchange of EAA signalling between UE and AAA-S.

3.	The EAA is performed between the UE and the AAA-S of SP via SPNP and the AMF receives the EAA results. Besides that, AAA-S provides the anchor key to AUSF, AUSF further provides it to AMF, and the AMF generates the security context with that anchor key.

4.	Steps 21-22 in clause 4.2.2.2.2 of TS 23.502 [y] are performed.

Editor's note:	Details of this EAA procedure are FFS.

### 6.Y.4	Impacts on existing entities and interfaces

Editor's note:	This clause lists impacts to existing entities and interfaces.

# * * * * End of changes * * * *