SA WG2 Meeting #136	S2-1910943

# Nov 18th-22nd, 2019 ; Reno, US	(revision of S2-1910357)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0038 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Line ID uniqueness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, (Huawei, HiSilicon)? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The LS from BBF in S2-1908655 (BBF Liase-310) has provided information regarding the usage of Line ID and its uniqueness. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on this LS, on the need to ensure unicity of the identifier used in 5GC for ULI and, in case of FN RG, used for SUCI / SUPI, and in order to simplify the responsibilities between 3GPP on one side and BBF / Cable Labs on the other side, the CR introduces: - The definition of a new Global Line Identifier (GLI) and of Global Cable identifier (GCI) which are made up of - A Line ID (BBF access) or HFC indetifier (Cable access) and - an identifier of the access region added to ensure unicity of the GLI/GCI. - The GLI (and GCI) are to be defined by BBF (by Cable Labs) - The definition of 2 new SUPI types: GLI based SUPI and GCI based SUPI both taking the form of a NAI whose user part is the GCI or GLI and whose domain part is an identifier of the operator managing the 5GC - Clarify the notion of pseudonym The definition of the format of these idenfiers will be specified in TS 23.003 and in relevant BBF and CableLabs specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct specification of Line ID and Cable HFC_Identifier support in W-5GAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1, 4.7.2, 4.7.3, 4.7.x (new), 4.7.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev3 - Reworded the Cover sheet, - The GLI ( GCI) and Line ID are defined by BBF or by Cable lbas - The definition of 2 new SUPI types: GLI based SUPI and GCI based SUPI both taking the form of a NAI whose user part is the GCI or GLi and domain part is an identifier of the operator managing the 5GC - Clarify the notion of pseudonym - Typos - It is understood that in 4.7.4 the two sentences highlighted in blue are not aligned there are change on change in this Tdoc |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.7.1	General

As described in TS 23.501 [2], each subscriber in the 5G System shall be allocated one 5G Subscription Permanent Identifier (SUPI) for use within the 3GPP system. As described in TS 23.501 [2], each FN-RG or 5G-RG accessing the 5G System shall be assigned a Permanent Equipment Identifier (PEI).

The clauses below describe specific aspects for supporting 5G-RG and FN-RG.

### 4.7.2	SUPI and SUCI for 5G-BRG support

The SUPI for an 5G-BRG shall contain an IMSI, as described in TS 23.501 [2], clause 5.9.2.

The SUCI provided by the 5G-BRG to the network contains the concealed SUPI, as described in TS 33.501 [11].

### 4.7.3	SUPI and SUCI for FN-BRG support

A new SUPI type is introduced to support FN-BRG: GLI based SUPI. A SUPI of type GLI based SUPI takes the form of a NAI whose user part is the GLI (Global Line identifier defined in clause 4.7.x) and whose domain part is an identifier of the operator managing the 5GC.

The SUPI for an FN-BRG subscription shall, based on operator configuration, either contain an IMSI or a GLI based SUPI.

The SUCI provided by the W-AGF to the 5GC for FN-BRG always corresponds to a GLI based SUPI. This SUCI acts as pseudonym of the SUPI and the UDM performs a mapping to the actual SUPI that, depending on operator configuration, contains either an IMSI or the same GLI based SUPI than in the SUCI.

As described in TS 23.003 [14], the SUCI also contains an identifier of the Home network i.e. the identifier of the operator managing the 5GC.

Editor's note:	Additional details regarding SUCI, including whether or not a Line ID in SUCI is concealed, is to be determined by SA WG3. The text above about SUCI for wireline access may need to be updated based on SA WG3 outcome.

### 4.7.4	SUPI and SUCI for 5G-CRG and FN-CRG support

A new SUPI type is introduced to support FN-CRG: GCI based SUPI. A SUPI of type GCI based SUPI takes the form of a NAI whose user part is the GCI (Global Cable identifier defined in clause 4.7.Y) and whose domain part is an identifier of the operator managing the 5GC.

The SUPI for a 5G-CRG/FN-CRG subscription shall contain either an IMSI, as described in TS 23.501 [2], clause 5.9.2, or a GCI based SUPI  ..

Editor's note:	The use of non-IMSI based SUPI and associated credentials and authentication method needs to be specified by SA WG3.

The SUCI provided by the 5G-CRG to the network contains the concealed SUPI, as described in TS 33.501 [11].

The SUCI provided to the network for FN-CRG support always corresponds to a GCI based SUPI, the This SUCI acts as pseudonym of the SUPI and the UDM performs a mapping to the SUPI that, depending on operator configuration, contains either an IMSI or the same GLI based SUPI than in the SUCI.

As described in TS 23.003 [14], the SUCI also contains an identifier of the Home network for both IMSI and GCI based SUPI. When SUCI is derived from a GCI based SUPI (taking the form of a NAI), the username shall contain the HFC_Identifier and the realm part shall identify the home network of the operator administrating the HFC_Identifier value.

Editor's note:	Additional details regarding SUCI, including whether or not a HFC_Identifier in SUCI is concealed, is to be determined by SA WG3. The text above about SUCI for wireline access may need to be updated based on SA WG3 outcome.

### 4.7.5	Line ID

The Line ID is defined in BBF Specifications, see BBF WT-456 [9].

### 4.7.x	Global Line Identifier

For usage with 5GC, a Global Line Identifier (GLI) is specified in order to define a globally unique identifier of the line connecting the RG to the network. In this release an RG is associated with a unique GLI.

For FN-BRG, the GLI is used to build a SUCI. For FN-BRG the GLI may be used to build a SUPI. See clause 4.7.3. For all types of RG, the GLI is used as User Location Information on wireline access.

The GLI contains an identifier of the access region and the Line ID value. The identifier of the access region ensures the unicity of the GLI while the Line ID may not be unique in some deployments. The identifier of the access region and line ID are be administered by the W-AGF operator (e.g. by the provider of access network when different from the 5GC operator).

The Global Line Identifier is a variable length identifier encoded as defined in TS 23.003 and in BBF WT-456 [9].

### 4.7.y	Global Cable Identifier

For usage with 5GC, a Global Cable Identifier (GCI) is specified in order to define a globally unique identifier of the line connecting the CRG to the network. In this release an RG is associated with an unique GCI.

For FN CRG, the GCI is used to build a SUCI. For FN CRG the GCI may be used to build a SUPI. See clause 4.7.3. For all types of CRG the GCI is used as User Location Information on wireline access.

The GCI contains an identifier of the access region and the HFC_Identifier value. The identifier of the access region ensures the unicity of the GCI while the HFC_Identifier may not be unique in some deployments. The identifier of the access region and HFC_Identifier are administered by the W-AGF operator (e.g. by the provider of Access network when different from the 5GC operator).

The Global Cable Idenfier is a variable length identifier encoded as defined in TS 23.003 and by Cablelabs in [x].

# * * * * End of changes * * * *