SA WG2 Meeting #S2-136	S2-1911152

18 - 22 November, 2019, Reno, Nevada, USA	(revision of S2-1909646)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1831 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of emergency services for Rel-16 UE not supporting CAG in CAG cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the LS from RAN2 (R2-1911827), Rel-16 UEs not supporting the CAG feature can camp on a CAG cell as an acceptable cell to obtain limited service. It means that unlike Rel-15 UE not supporting CAG, Rel-16 UE not supporting CAG will not be prevented from camping on an acceptable CAG cell by CellReservedForOtherUse indicator. In our point of view, as the type of UEs which can use emergency services is determined by the regulator and network operator and the CAG cell is hosted by PLMN, the support of emergency services for Rel-16 UE not supporting CAG in CAG cells should not be excluded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support of emergency services for Rel-16 UE not supporting CAG in CAG cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Emergency services for Rel-16 UE not supporting CAG remains unsupported in CAG cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

5.30.3.5	Support of emergency services in CAG cells

Emergency Services are supported in CAG cells, for UEs supporting CAG, whether normally registered or emergency registered as described in clause 5.16.4 and TS 23.502 [3] clause 4.13.4.

A UE supporting CAG may camp on an acceptable CAG cell in limited service state as specified in TS 23.122 [17] and TS 38.304 [50].

A UE not supporting CAG may camp on a CAG cell as an acceptable cell to obtain limited service.

NOTE:	Support for Emergency services requires the cell to only be connected to AMFs that supports emergency services.

During handover to a CAG cell, if the UE is not authorized to access the target CAG cell and has emergency services, the target NG-RAN node only accepts the emergency PDU sessions and the target AMF releases the non-emergency PDU connections that were not accepted by the NG-RAN node. Upon completion of handover the UE behave as emergency registered.

* * * * End of Changes * * * *