3GPP TSG-SA WG2 Meeting #136	S2-1912105

# Reno, USA, 18 – 22 November 2019 	(revision of S2-1911552)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0059 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Policy Control Request Triggers for wireline access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT3 had LS (S2-1909972, C3-194377) to SA2 about questions on: - Q1: Whether access network information reporting is supported over wireline access? - Q2: Is there any other policy control request trigger which is not applicable when 5G-RG or FN-RG connects to the 5GC via wireline access? - Q3: Is this limitation also applicable to the UE policy when 5G-RG or FN-RG connects to the 5GC via wireline access? Is the PLMN change applicable to the UE policy in this case? To conclude the questions, the main concern is about the “Policy Control Request Triggers” regarding AM policy, UE policy and SM policy. Therefore, in this CR, we propose to further describe the PCRTs applicable for RG accessing via W-5GAN to the “Policy Control Request Triggers for AMF and SMF” as defined in clauses 6.1.2.5 and 6.1.3.5 in TS23.503. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of subclause about “Policy Control Request Triggers relevant for AMF and wireline access” - Addition of subclause about “Policy Control Request Triggers relevant for SMF and wireline access” - Deletion of reference to PCRTs defined in 23.503 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.X, 9.5.X, 8.4.3, 9.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

9.1.X	Policy Control Request Triggers relevant for SMF and wireline access type

The Policy Control Request Triggers relevant for SMF and wireline access define the conditions when the SMF shall interact again with PCF after a PDU Session established via W-5GAN. PCR triggers defined in Table 6.1.3.5 in TS23.503 [4] are supported forW-5GAN scenario with the following not supporting ones:

- PLMN change

- Location change (serving area)

- Location change (serving CN node in 5GS)

- Location change (serving CN node in EPC)

- Change of UE presence in Presence Reporting Area

- 3GPP PS Data Off status change

- GFBR of the QoS Flow can no longer (or can again) be guaranteed

- UE resumed form suspend data

- Manageable Ethernet Port detected

- Port Management Information Container available

Additionally, the new triggers defined in clause 9.7 for IPTV service are also applied for a 5G-RG connected via W-5GAN scenario.

# * * * * Second change * * * *

9.5.X	Policy Control Request Triggers relevant for AMF and wireline access type

The Policy Control Request Triggers relevant for AMF and wireline access type define the conditions when the AMF shall interact again with PCF after the AM Policy Association. PCR triggers defined in Table 6.1.2.5 in TS23.503 [4] are supported for W-5GAN scenario with the following not supporting ones: with the following deltas

- Location change (tracking area)

- Change of UE presence in Presence Reporting Area

- RFSP index change

- UE-AMBR change

- PLMN change

Additionally, the following PCR triggers are added regarding the wireline access type:

Table 9.5.X-1: Policy Control Request Triggers relevant for AMF and wireline access type

| Policy Control Request Trigger | Description | Condition for reporting |
| --- | --- | --- |
| Access Type change (NOTE 1) | The Access Type and the RAT Type has changed | PCF (AM Policy) |
| RG-TMBR | The Subscribed RG-TMBR has changed | PCF (AM Policy) |
| NOTE 1: The RAT type is reported for 3GPP access, or when the 5G-RG or FN-RG registers over wireline access (i.e. W-AGF). |  |  |

The UE Policy related PCR triggers like location change, PRA change and PLMN change are not applicable for wireline access.

# * * * * Third change * * * *

8.4.3	Npcf_AMPolicyControl

8.4.3.1	Npcf_AMPolicyControl_Create service operation

The input data listed in clause 5.2.5.2.2 in TS 23.502 [3] apply when an AM Policy Association is created for a 5G-RG, except for the handling of RFSP information that applies only in case a 5G RG is registered over 3GPP access.

The input information when the UE registers via W-5GAN includes the Access type set to non-3GPP access, the User Location Information including the Global Line ID or the HFC node Id.

The output information when the UE registers via W-5GAN is defined in clause 9.5 and the Policy Control Request triggers applicable for RG access via W-5GAN are defined in clause 9.5.X-1.

8.4.3.2	Npcf_AMPolicyControl_Update service operation

The input data listed in clause 5.2.5.2.5 in TS 23.502 [3] apply when an AM Policy Association is updated for a 5G-RG or for a FN-RG, except for the notification of UE location change (if an RG registers only on Wireline access), PRA changes or RFSP index change.

PCRT on UE location change apply when a 5G RG registers on a second access (5G RG using Hybrid access).

The output information when the UE registers via W-5GAN is defined in clause 9.5 and the Policy Control Request triggers applicable when RG accesses via W-5GAN are defined in clause 9.5.X.

The Access type change trigger requests the AMF to report a new Access Type and RAT Type to the PCF. When the UE simultaneous connects over both 3GPP and wireline non-3GPP access type, the AMF reports the list of Access Type and RAT combinations available in the UE access and mobility context in the Npcf_AMPolicyControl_Update service operation.

# * * * * Forth change * * * *

9.5.1	Access and mobility related policy information

This clause specifies the delta related to Access and Mobility related policy information defined in TS 23.503 [4] clause 6.1.2.1 for 5G-RG and FN-RG.

The access and mobility policy control encompass the management of service area restrictions for a 5G-BRG or a 5G-CRG connecting to 5GC via W-5GAN or simultaneously via NG-RAN and W-5GAN.

The management of service area restrictions enables the PCF of the serving PLMN to modify the service area restrictions based on operator defined policies at any time, either by expanding a list of allowed Line IDs or the HFC Node ID or by reducing the list of non-allowed Line IDs or HFC Node IDs. Operator defined policies in the PCF may depend on input data such as time of day, or UE context information provided by other NFs, etc.

The AMF reports the subscribed service area restrictions in NG-RAN received from UDM during 5G-RG Registration in NG-RAN procedure when local policies in the AMF indicate that Access and Mobility Control is enable within the PLMN. The AMF may request update of the service area restrictions applicable to NG-RAN when the policy control request triggers listed in clause 6.1.2.5 in TS 23.503 [4], are met.

The AMF reports the subscribed service area restrictions in W-5GAN received from UDM during 5G-RG Registration in W-5GAN procedure when local policies in the AMF indicate that Access and Mobility Control is enable within the PLMN. The AMF may request update of the service area restrictions applicable to W-5GAN when the policy control request triggers listed in clause 9.5.X are met.

The AMF receives the modified service area restrictions from the PCF and then use them as described in clause 4.3.3.3.

No mobility events, such a change of UE location or change of AMF applies when provisioning the service area restrictions for a 5G-BRG or a 5G-CRG when connected via W-5GAN.

The PCF may provide the service area restrictions applicable to a 5G-RG connected to 5GC via W-5GAN or via NG-RAN or simultaneously connected to 5GC via W-5GAN and NG-RAN to AMF. The PCF may provide the service area restrictions applicable to a FN-CRG to the AMF.

The Service Area Restrictions provided to AMF for a 5G-RG connected via NG-RAN is according to the information listed in listed in TS 23.503 [4] clause 6.5.

The Service Area Restrictions provided to AMF for a 5G-RG connected via W-5GAN is according to the information listed in Table 9.5-1.

For a 5G-RG simultaneously connected to 5GC via W-5GAN and NG-RAN the PCF provides Service Area Restrictions for both W-5GAN and NG-RAN.

The Service Area Restrictions provided to AMF for a FN-CRG connected via W-5GAN is according to the information listed in Table 9.5-1.

Table 9.5-1: Access and mobility related policy control information for 5G-RG and FN-CRG accessing via W-5GAN

| Information name | Description | Category | PCF permitted to modify in a AM context in the AMF | Scope |
| --- | --- | --- | --- | --- |
| Service Area Restrictions for a 5G-BRG | This part defines the service area restrictions applicable for a 5G-BRG |  |  |  |
| List of allowed Line IDs | List of allowed Line IDs (NOTE 2) (NOTE 3). | Conditional (NOTE 1) | Yes | AM context |
| List of non-allowed Line IDs | List of non-allowed Line IDs (NOTE 3). | Conditional (NOTE 1) | Yes | AM context |
| Service Area Restrictions for a 5G-CRG or for a FN-CRG | This part defines the service area restrictions applicable for a 5G-CRG or for a FN-CRG |  |  |  |
| List of allowed HFC Node IDs | List of allowed HFC Node IDs (NOTE 2) | Conditional (NOTE 1) | Yes | AM context |
| List of non-allowed HFC Node IDs | List of non-allowed HFC Node IDs (NOTE 2). | Conditional (NOTE 1) | Yes | AM context |
| NOTE 1: If service area restrictions is enable. NOTE 2: Either the list of allowed (Line IDs or HFC Node IDs) or the list of non-allowed (Line IDs or HFC Node IDs) are provided by the PCF. |  |  |  |  |

The list of allowed Line IDs or the list of allowed HFC Node IDs indicates the locations where the 5G-RG is allowed to be registered, see clause 4.3.3.3 for the description on how AMF uses this information.

# * * * * Fifth change * * * *

# * * * * End of changes * * * *