SA WG2 Meeting #136 	S2-1911202

# 18 - 22 Nov 2019, Reno, USA

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1890 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UCMF service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2019-9-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current UCMF service, for resolve, assign, notify operation, the TAC or vendor information is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding TAC or Vendor information as input for UCMF service resolve, assign, notify operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resolve, assign, notify operation cannot diffrentiate manufacturers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.18.3.1, 5.2.18.3.2, 5.2.18.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

##### 5.2.18.3.1	Nucmf_UECapabilityManagement Resolve service operation

Service Operation name: Nucmf_UECapabilityManagement_Resolve

Description: Consumer NF gets the UE radio access capability corresponding to a specific UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned).

Inputs, Required: UE Radio Capability ID, Type Allocation Code or Vendor ID.

Inputs, Optional: None.

Outputs, Required: UE radio access capability.

Outputs, Optional: None.

##### 5.2.18.3.2	Nucmf_UECapabilityManagement_Assign service operation

Service or service operation name: Nucmf_UECapabilityManagement_Assign

Description: The NF consumer sends the UE radio access capability in the UCMF and obtains a PLMN-assigned UE Radio Capability ID in return.

Inputs, Required: UE radio access capability, Type Allocation Code.

Inputs, Optional: None.

Outputs, Required: UE Radio Capability ID.

Outputs, Optional: None.

The AMF sends the UE radio access capability other than NB-IoT radio access capability to the UCMF.

**** Next Change ****

##### 5.2.18.3.5	Nucmf_UECapabilityManagement_Notify service operation

Service Operation name: Nucmf_UECapabilityManagement_Notify

Description: Producer NF provides notifications about creation or deletion of UCMF dictionary entries to subscribed consumer NF.

Inputs, Required: Notification Type ("creation", "deletion"), Type Allocation Code or Vendor ID.

Inputs, Optional: If Notification Type is set to "creation": One or more UCMF dictionary entries, each UCMF dictionary entry consisting of a UE Radio Capability ID and the corresponding UE radio access capability. If Notification Type is set to "deletion": One or more UE Radio Capability IDs.

Outputs, Required: None.

Outputs, Optional: None.

**** End of Changes ****