3GPP TSG-WG SA2 Meeting #136 	S2-1911523

# Reno, NV, USA, November 18 – 22, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.060 | CR | 2047 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR TS 23.060 Support N7 interface for GERAN/UTRAN access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, China Unicom, Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current release, both Gx interface and N7 interface have been defined for policy and charging control of GERAN/UTRAN/E-UTRAN and E-UTRAN/NR combination respectively. While operators may want to simplify the network deployment and reduce the operational complexity of PGW-C/GGSN/SMF and the interface between PGW-C/GGSN/SMF and PCF/PCRF. With such consideration, the PGW needs to support N7 interface to interact with PCF, and support the functionalities required for GERAN/UTRAN access, e.g. the mapping from EPS QoS information to R99 QoS information, as defined in Annex E. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An annex is added to describe the support of N7 interface for GERAN/UTRAN access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PGW/GGSN has to support both Gx and N7 interfaces for policy control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex ZZ (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. For a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

…

[121]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[122]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[123]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[x]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[y]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[z]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

# * * * * Second change * * * *

Annex ZZ (normative):
Support of N7 interface for GERAN/UTRAN access

In some network deployment with GERAN/UTRAN access, the SGSN may select either GGSN or PGW as defined in clause 5.3.7.1. When dynamic PCC is deployed, the GGSN or PGW may support the N7 interface, as defined in the TS 23.501[x], TS 23.502 [y] and 23.503 [z], to interact with PCF for policy and charging control.

In this case, apart from the functionalities defined in the current specification, the selected GGSN or PGW additionally performs:

- Mapping between 5GS QoS information and EPS QoS information as defined in TS 23.502 [y] Annex C, and mapping between EPS QoS information and the GERAN/UTRAN related QoS information as defiend in Annnex E.

- Creation of unique PDU Session ID for each PDN connection of the UE.

In addition, the PGW may also support N40 interface to interact with CHF for charging management.

# * * * * End of changes * * * *