3GPP TSG-WG SA2 Meeting #136 	S2-1911736

# Reno, NV, USA, November 18 – 22, 2019	(revision of S2-191xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0074 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of Cancellation of a Deferred 5GC-MT-LR procedure for TS 23.273 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is agreed that external AF can send the cancellation request towards core network to cancel a specific deferred location request it previously subscribes to the 5GS. It is clear in the specification that UE and LMF would release all the resource for the location request after receiving such cancellation request from external AF. However, it is not clear whether the GMLC or AMF should release the resource for the location request after receiving such cancellation request from external AF. Rev1 addition: It is proposing the GMLC and AMF also release the resource for location service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GMLC releases all the resource for the location request after receiving the cancellation acknowledgement from UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete MT-LR procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3.3	Cancellation of Reporting of Location Events by an AF or External LCS Client

Figure 6.3.3-1 summarizes a procedure to enable an AF or External LCS Client to cancel a deferred 5GC-MT-LR procedure for periodic, or triggered location. It is assumed that a deferred 5GC-MT-LR for periodic or triggered location events has already been requested according to the procedure in clause 6.3.1 up until at least step 20.

Figure 6.3.3-1: Cancellation of a Deferred 5GC-MT-LR for periodic or triggered location events by an AF or External LCS Client

1.	The external LCS client or AF (via an NEF) send a request to cancel the periodic or triggered location to the (H-)GMLC.

The H-GMLC may itself initiate the cancellation procedure, e.g. when it is notified that the UE's privacy setting stored in the UDM was changed. For every outstanding Deferred Location Request against that UE, the H-GMLC shall perform a new privacy check based on the updated privacy setting stored in the UDM. If the privacy check passes, i.e. the LCS Client is still allowed to position the target UE, the handling of the outstanding Deferred Location Request shall be continued. Otherwise, if the privacy check does not pass, i.e. the Location estimate of the target UE is not allowed to be provided to the LCS Client, the H-GMLC shall initiate a cancellation procedure

2.	The (H-)GMLC queries the UDM to determine the serving AMF address as in step 3 of clause 6.3.1.

3.	For a roaming UE, the H-GMLC obtains a V-GMLC address if not received at step 2 and invokes an Ngmlc_CancelLocation service operation to forward the cancellation request to the V-GMLC. The (H-)GMLC also includes the contact address for the (H-)GMLC and the LDR reference number in the request and the latest LMF identification received in step 20 or step 29 in clause 6.3.1 if either step has occurred and included an LMF identification.

4.	The (H-)GMLC or V-GMLC invokes the Namf_Location_CancelLocation service operation to forward the cancellation request to the serving AMF and includes the (H-)GMLC contact address, LDR reference number and LMF identification if available. AMF releasess all resources for the location request.

5. If an LMF identification was included in step 4, the AMF forwards the cancelation request to the indicated LMF by invoking an Nlmf_CancelLocation service operation and includes the (H-)GMLC contact address and LDR reference number. The LMF then releases all resources for the location request.

6.	If the UE is not currently reachable (e.g. is using eDRX or PSM), the AMF waits for the UE to become reachable.

7.	Once the UE is reachable, if the UE is then in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.4 of TS 23.502 [19] to establish a signalling connection with the UE.

8.	The AMF sends the cancelation request to the target UE and includes the (H-)GMLC contact address and the LDR reference number. The UE then releases all resources for the location request.

9.	The UE returns an acknowledgment to the AMF.

10. The AMF returns the acknowledgment to the V-GMLC or (H-)GMLC.

11. For a roaming UE, the V-GMLC returns the acknowledgment to the H-GMLC, the H-GMLC then releases all resources for the location request.

# * * * * End of changes * * * *