SA WG2 Meeting #136	S2-1912604

18 - 22 November, 2019, Reno USA

Source:	CableLabs, Charter Communications

Title:	Key Issues to address handling of active data sessions

Document for:	Discussion / Approval

Agenda Item:	8.4

Work Item / Release:	FS_MUSIM / Rel-17

Abstract: The contribution proposes adding a KI to address handling of data services while the MUSIM device moves in and out of the coverage of the user preferred network

# 1	Background

None of the key issues in 3GPP TR 23.761 V0.1.0 explicitly handle uncoordinated scenarios (where the MUSIMs belong to different network operators) dealing with active data session handling when the device moves in and out of the network coverage of the user preferred network (based on user preferences).

Note: The contribution does not propose service continuity or PDU session transfer between the networks associated with multiple USIMs. The contribution strictly considers data service handling and does not consider voice service handling.

Consider a scenario with:

USIM A: Preferred network for data services as per user preference

USIM B: Non-preferred network for data services as per user preference

| Assumption(s) | Scenario | Impact to user |
| --- | --- | --- |
| Active data session on the network associated with USIM A | MUSIM device moves outside the coverage of the USIM A network, but is within coverage of the USIM B network | User is unable to re-initiate any data session on the USIM B following loss of the USIM A network |
| MUSIM device outside the USIM A network coverage Active data session on the network associated with USIM B | MUSIM device moves inside the USIM A network coverage, while still being within the USIM B network coverage | Data services continue on USIM B and user preference is not realized |

Such scenarios are key for non-public networks that would have limited network coverage. Example of non-public network include private networks such as enterprise, health care, industrial, law enforcement, etc.

We propose to include additional key issue mentioned below to address data service handling in uncoordinated scenarios:

- Key Issue 5: Handling of fast data service re-initiation in uncoordinated scenario

# 2	Proposal

We propose to include the below mentioned “Key Issue” to the existing key issues defined in 3GPP TR 23.761 V0.1.0.

* * * * Start of Change * * * *

## 5.5	Key Issue 5: Handling of data service re-initiation in uncoordinated scenario

### 5.5.1	Description

Consider a MUSIM device with active data services on the network associated with USIM A (preferred network as per user preference), if the MUSIM device moves outside the USIM A network coverage, then MUSIM device should allow the user to initiate data services on USIM B immediately to avoid any service disruption.

Similarly consider a MUSIM device outside the network coverage of USIM A (preferred network as per user preference) and with active data services on the network associated with USIM B. If the MUSIM device moves inside the coverage of the USIM A network, then MUSIM device should re-initiate data services with USIM A to ensure compliance with user preference.

The present key issue shall study how to minimize service disruption for data services when:

- A MUSIM device moves outside the coverage area of the network associated with the preferred USIM (based on per user preferences) and is within a coverage area of the network of another USIM provisioned for data services

The present key issue shall also study how to re-initiate data services on a user preferred USIM when:

- A MUSIM device moves inside the coverage area of the network associated with the preferred USIM (based on per user preferences) while still within the coverage area of another USIM provisioned for data services

* * * * End of Change * * * *