3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002139

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0135 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of emergency usage of data analytics report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE moving trajectory is very important and useful information for epidemic prevention and therefore it is proposed to allow emergency centres to comsume UE moving trajectory for emergency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of emergency usage of data analytics report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support of emergency epidemic prevention . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4(new), 6.7.2.1, 6.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2	Non-roaming architecture

As depicted in Figure 4.2-1, the 5G System architecture allows NWDAF to collect data from any NF. The NWDAF belongs to the same PLMN where 5GC NF  that notifies the data is located.

Figure 4.2-1: Data Collection architecture from any NF

The Nnf interface is defined for the NWDAF to request subscription to data delivery for a particular context, to cancel subscription to data delivery and to request a specific report of data for a particular context.

The 5G System architecture allows NWDAF to retrieve the management data from OAM by invoking OAM services.

As depicted in Figure 4.2-2, the 5G System architecture allows any NF to request network analytics information from NWDAF. The NWDAF belongs to the same PLMN where 5GC NF that consumes the analytics information is located.

Figure 4.2-2: Network Data Analytics Exposure architecture

The Nnwdaf interface is defined for 5GC NFs, to request subscription to network analytics delivery for a particular context, to cancel subscription to network analytics delivery and to request a specific report of network analytics for a particular context.

4.3	Roaming architecture

The interactions between the NWDAF and the other 5GC NFs are only considered in the same PLMN case.

Roaming architecture does not apply in this release of the specification.

4.4	Emergency supporting architecture

The 5G System architecture allows NWDAF that could be emergency specific to collect data from any NF.

The 5G System architecture allows emergency centre(s) to request network analytics information from NWDAF, which could be emergency specific.

# * * * * Second change * * * *

6.7.2	UE mobility analytics

6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM, and to perform data analytics to provide UE mobility statistics or predictions.

The 5G System architecture allows emergency centre(s) to request UE mobility statistics or predictions from NWDAF, which could be emergency specific.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytics Filter Information containing optional list of TA or Cells, optional maximum number of results

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

# * * * * Third change * * * *

6.7.2.4	Procedures

The NWDAF can provide UE mobility related analytics, in the form of statistics or predictions or both, to another NF. If the NF is an AF, and when the AF is untrusted, the AF will request analytics via the NEF, and the NEF will then convey the request to NWDAF.

Figure 6.7.2.4-1: UE mobility analytics provided to an NF

1.	The NF sends a request to the NWDAF for analytics on a specific UE or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF can request statistics or predictions or both. The type of analytics is set to UE mobility information. The NF provides the UE id or Internal Group ID in the Target of Analytics Reporting.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for notification of location changes. This step may be skipped when e.g. the NWDAF already has the requested analytics available.

NOTE:	The NWDAF determines the AMF serving the UE or the group of UEs as described in clause 6.2.2.1.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provide requested UE mobility analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for UE mobility analytics provided by NWDAF are defined in clause 6.7.2.3.

5-6. If at step 1, the NF has subscribed to receive notifications for UE mobility analytics, after receiving event notification from the AMFs subscribed by NWDAF in step 2, the NWDAF may generate new analytics and provide them to the NF.

If the consumer NF is an emergency centre, it requests UE mobility statistics or predictions from NWDAF for the purpose of emergency.

# * * * * End of changes * * * *