SA WG2 Meeting #137E	S2-2001924

# Feb 24 - 28, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2146 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on steering for CIoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - When the UE registers with AMF, UE includes its Preferred Network Behaviour for 5GC and EPC. But there could be some UEs only support 5GS CIoT, so it should be clarified UE includes its Preferred Network Behaviour for EPC if the UE supports S1 mode and supports any of the EPC Preferred Network Behaviour. - In current spec, if after redirection the UE cannot find a cell supporting connectivity, the UE may re-enable the disabled N1/S1 mode and then perform Registration, Attach or TAU. In this case, the source CN may reject the UE again, which will lead to the result that the UE will not get the service from the network for a very long time, especially considering static or low mobility UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifes UE includes its Preferred Network Behaviour for EPC if the UE supports S1 mode and supports any of the EPC Preferred Network Behaviour when UE registers with AMF. - Clarifies that to avoid AMF or MME rejects the UE again, the UE does not include its Preferred Network Behaviour for the CN type to which the UE cannot find a cell supporting connectivity during Registration, Attach or TAU procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It implies UE supporting 5GS CIoT will also support EPS CIoT. - The source CN will reject the UE again and again, which will result that the UE may not get the service if being redirected to a CN type to which the UE cannot find a cell supporting connectivity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.31.3	Selection, steering and redirection between EPS and 5GS

The UE selects the core network type (EPC or 5GC) based on the broadcast indications for both EPC and 5GC, and the UE's EPC and 5GC Preferred Network Behaviour. For a network that supports NB-IoT, it shall broadcast an indication of whether N3 data transfer is supported or not in system information.

When the UE performs the registration procedure it includes its Preferred Network Behaviour for 5GC and EPC if the UE supports S1 mode and supports any of the EPC Preferred Network Behaviour  in the Registration Request message and the AMF replies with the 5G Supported Network Behaviour in the Registration Accept message.

If the UE supports any of the CIoT 5GS Optimisations included in 5GC Preferred Network Behaviour, then when the UE performs an Attach or TAU procedure and the UE includes its EPC Preferred Network Behaviour then the UE shall also include its 5GC Preferred Network Behaviour.

In the networks that support CIoT features in both EPC and 5GC, the operator may steer the UEs from a specific CN type due to operator policy, e.g., due to roaming agreements, Preferred and Supported Network Behaviour, load redistribution, etc. Operator policies in EPC and 5GC are assumed to avoid steering UEs back and forth between EPC and 5GC.

To redirect a UE from 5GC to EPC, when the UE requests Registration, the AMF sends a Registration Reject with an EMM cause value indicating the UE should not use 5GC. The UE disables N1 mode and re-enables S1 mode, if itwas disabled. The UE then performs either an Attach or TAU with EPC as described in clause 5.17.2.

To redirect a UE from EPC to 5GC, when the UE requests an Attach or TAU procedure, the MME sends a reject message with an EMM cause indicating the UE should not use EPC. The UE disables S1 mode and re-enables N1 mode, if it was disabled. The UE then performs a Registration with 5GC as described in clause 5.17.2.

When determining whether to redirect the UE the AMF/MME takes into account the UE support of S1/N1 mode (respectively) and the UEs (5G) Preferred Network Behaviour and the (5G) Supported Network Behaviour of the network the UE is being redirected towards.

If after redirection the UE cannot find a cell supporting connectivity, the UE may re-enable the disabled N1/S1 mode and then perform Registration, Attach or TAU. To avoid AMF or MME rejects the UE again, the UE does not include its Preferred Network Behaviour for the CN type to which the UE cannot find a cell supporting connectivity during Registration, Attach or TAU procedure.

END OF CHANGES