SA WG2 Meeting #S2-138e	S2-2002778

20 - 24 April, 2020, Electronic Meeting	(revision of S2-xxxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2192 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NAS non delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI15 |  |  |  |  |  | Date: |  |  | 10/04/2020 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the LS in from RAN3, S2-2002658/R3-201362, The text in clause 5.3.3.2.5 seems causes confusions in RAN3. In case of NAS non-delivery in RRC inactive state, the reason for non-delivery may be different, e.g. due to paging failure or due to mobility as resume to a new NG-RAN node. The way that CN sending NAS-PDU to NG-RAN node is also different, i.e. via DL NAS transport, NAS-PDU as part of N2 message, or NAS-PDU as part of SMF related IE in a N2 message For MT-SMS, N2 DL NAS transport is used. NG-RAN shall provide NAS non delivery notification first in case of delivery failure, before the NG-RAN initiated the UE context release procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 4.13.3.7 to clarify NAS non delievery notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current text makes RAN3 unclearly how to apply different procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 2245 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.3.7	MT SMS over NAS in CM-CONNECTED state via 3GPP access

MT SMS in CM-CONNECTED procedure is specified by reusing the MT SMS in CM-IDLE state with the following modification:

-	There is no need for the AMF to perform Paging of the UE and can immediate continue with a message to SMSF via N20 to allow the SMSF to start forward the MT SMS.

-	If the delivery of the NAS PDU containing the SMS fails e.g. if the UE is in RRC Inactive and NG-RAN paging was not successful, the NG-RAN shall provide NAS non-delivery notification to the AMF, then initiate the UE context release in the AN procedure. The AMF provides an indication of non-delivery to the SMSF.

# * * * * Second change * * * *

# * * * * End of changes * * * *