3GPP SA WG2 Meeting #138E	S2-2003106

April 20-24, 2020                                                                                             was S2-2001806

# Source:	Juniper

# Title:	IEEE Compatible Ethernet Switching Services

# Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: IEEE Compatible Ethernet Switching Services

## Acronym: 5G-Switching

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-17.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| Vertical_LAN | SA2 | 820017 | 5GS Enhanced support of Vertical and LAN Services |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  | N/A |  |

Dependency on non-3GPP (draft) specification: None

## 3	Justification

Current 5G Ethernet Switching solutions (with or without 5G-LAN) specified in TS 23.501 have many documented limitations. TEI17 proposal S2-2001806 attempted to remove the limitations and to take advantages of IETF IP/E-VPN solutions that provide the same services on the wireline side. The feedback for that proposal was that more study is needed.

Recently it was agreed that supporting multiple ports on TSN NW-TT is a R16 FASMO issue. However, even after it is clarified in R16 that multiple ports will be allowed on TSN NW-TT, effectively only one port could be supported until a single-N6 limitation in 5.8.2.5.3 of 23.501 is removed.

The single-N6 limitation would have been removed by the TEI17 S2-2001806. Given the concern with that proposal and the need for multiple-N6 due to the R16 TSN FASMO issue, it is proposed here to reduce the scope of S2-2001806, hence this revised S2-2003106 and its draftCR companion S2-2003149.

This proposal meets TEI criteria in that it is a straightforward enhancement with significant benefits, and it is expected to take 0.5 TU.

## 4	Objective

The objective is to specify Ethernet Switching (with or without the 5G-LAN) in a more complete way. The following is a summary, and the S2-2003149 is the corresponding proposed CR.

For generality, even Ethernet Switching without 5G-LAN, as specified in 5.8.2.5.3, is considered as just a special case of 5G-LAN, in that all Ethernet PDU sessions and relevant N6 interfaces for the Network Instance belong to a default 5G-VN group.

- When members of an Ethernet 5G-VN are anchored on the same UPF or if no UPF-UPF tunnelling is used, a UPF behaves as an Ethernet switch for the VN, with the PDU sessions and N6 interfaces treated as logical switch interfaces. The switch is IEEE-compatible, including running Spanning Tree Protocols or using other methods to block traffic if needed to prevent looping. The other methods are outside the scope of this specification.

- When UE members of a 5G-VN are anchored on different UPFs and N19 tunnelling is used, the tunnels are also treated as logical interfaces of the switch for the VN.

Note: This is similar to #1, except that if there are more than two UPFs for the VN, Spanning Tree Protocols or other methods must be used to prevent looping among UPFs (e.g. with full-mesh of N19 tunnels, traffic received from an N19 tunnel is not sent back to another N19 tunnel).

In essence, traffic forwarding is autonomous based on the built-in switching functionalities of UPF without the need of SMF programmed PDRs/FARs. This is implied in 5.8.2.5.3 though it is made explicit in this proposal.

Note: even when these switching-based options are used for a 5G-VN, PDRs/FARs can still be used on top of UPF autonomous forwarding. For example, traffic of different flows to the same destination could be forwarded towards the same member (based on MAC address lookup) but with different QoS treatment (based on PDR/FAR where the PDR contains additional information identifying the flows and the FAR contains additional forwarding treatment).

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Updated clause 5.8.2.5.3. A new clause 5.8.2.13.4 that specifies the optional alternative, plus some minor relevant changes | TSG#88 | This will be handled as “TEI17”. |

## 6	Work item Rapporteur(s)

Zhaohui (Jeffrey) Zhang, Juniper Networks, zzhang@juniper.net

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

N/A

## 9	Supporting Individual Members

Supporting IM name

Juniper

Orange

China Mobile

KPN

Spreadtrum