SA WG2 Meeting #138E	S2-20003073r01

# 20 - 24 Aprial, 2020, Electronic												 (revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0459 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Reporting and Credit management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the agreement addressed in the DP “TS 23.203 Endorsement” (S2-1903636), any reference to 23.203 shall be replaced by the referenced content. However, when the replacement for 6.1.3.4 is implemented by S2-1904118, some content is missed, e.g. credit re-authentication. For EPC, the credit re-authorisation and the triggers are defined in clause 6.1.3 and clause A.4.3.1.1 of TS 23.203. For 5GC, the credit re-authorisation and the triggers are defined in TS clause 5.2.1.4 of TS 32.255. Converged charging introduced for 5G includes quota management and usage reporting. Usage reporting is commonly applied for both online charging and offline charging, then it is not distinct to credit management (quota management). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 6.1.3.4 to align with credit management defined in TS 32.255 and refer to TS 32.255. Update 6.1.3.3 and Annex D to align with usage reporting defined in TS 32.255 and refer to TS 32.255. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SMF doesn’t know how to request credit re-authorization to CHF. In consistant with SA5 for the definition of credit control, usage reporting and Sponsored data connectivity charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.3, 6.1.3.4, D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.1.3.3	Reporting

Reporting refers to the differentiated PDU Session resource usage information (measured at the UPF) being reported by the SMF to the CHF.

The SMF shall report usage information for each charging key value.

For service data flow charging, for the case of sponsored data connectivity, usage shall be reported for each charging key, Sponsor Identity and Application Service Provider Identity combination if Sponsor Identity and Application Service Provider Identifier have been provided in the PCC rules with offline charging method.

NOTE 1:	Online charging method for PCC Rules that include Sponsor Identity and Application Service Provider Identity is not supported in this document. Online charging for sponsored data connectivity can be based on charging key as described in Annex D.

The SMF shall report usage information for each charging key/service identifier combination if service identifier level reporting is requested in the PCC rule.

NOTE 2:	For reporting purposes when charging is performed by the SMF:

a)	the charging key value identifies a service data flow if the charging key value is unique for that particular service data flow, and

b)	if the service identifier level reporting is present then the service identifier value of the PCC rule together with the charging key identify the service data flow.

The SMF may receive events trigger information from the CHF, which shall cause the SMF to perform a usage reporting to CHF when the event occurs, as specified in TS 32.255 [21].

Charging information shall be reported based on the result from the service data flow detection and measurement on a per PDU Session basis.

A report may contain multiple containers, each container associated with a charging key, charging key and Sponsor Identity (in case of sponsored connectivity) or charging key/service identifier.

NEXT CHANGE

#### 6.1.3.4	Credit management

The credit management applies only for service data flow with online charging method  and shall operate on a per charging key basis. The SMF should initiate one charging session with the CHF for each PDU Session subject to charging, in order to perform credit management within the charging session.

NOTE 1:	Independent credit control for an individual service/application may be achieved by assigning a unique charging key value in the corresponding PCC rule.

The SMF shall request a credit for each charging key occurring in a PCC rule. It shall be up to operator configuration whether the SMF shall request credit in conjunction with the PCC rule being activated or when the first packet corresponding to the service is detected. The CHF may either grant or deny the request for credit. The CHF shall strictly control the rating decisions.

NOTE 2:	The term 'credit' as used here does not imply actual monetary credit, but an abstract measure of resources available to the user. The relationship between this abstract measure, actual money, and actual network resources or data transfer, is controlled by the CHF.

During PDU Session establishment and modification, the SMF shall request credit using the information after applying policy enforcement action (e.g. upgraded or downgraded QoS information), if applicable, even though the SMF has not signalled this information to the AMF or RAN.

The events trigger information which may be received from the CHF, causing the SMF to perform a usage reporting and credit request to CHF when the event occurs are specified in TS 32.255 [21].

The CHF may subscribe to Change of UE presence in Presence Reporting Area at any time during the life time of the charging session as described in TS 32.255 [21].

If the PCF set the Out of credit event trigger (see clause 6.1.3.5), the SMF shall inform the PCF about the PCC rules for which credit is no longer available together with the applied termination action.

NEXT CHANGE

# D.2	Reporting for sponsored data connectivity

There are two deployment scenarios for usage reporting for sponsored data connectivity. The Sponsor Identifier and Application Service Provider Identifier are provided for sponsored services to the PCF from the AF over the Rx/N5 interface.

In the first scenario the PCF assigns a service specific Charging Key for a sponsored IP flow. The Charging key is used by the SMF to generate separate usage records for reporting and optional credit management. Correlation of reported usage from multiple users per sponsor and/or application service provider is then performed using the charging key.

In a second scenario the Sponsor Identifier and Application Service Provider Identity is included in PCC rules from the PCF to the SMF with offline charging method, as defined in clause 6.3.1. For this scenario the same Charging Key may be used both for IP flows that are sponsored and for flows that are not sponsored. Usage reported by the SMF includes the Sponsor Identity and the Application Service Provider Identity. Correlation of reported usage from multiple users per sponsor and/or application service provider can then be based on Sponsor Identity and Application Service Provider Identity instead of the Charging Key.. Online charging method for PCC rules that include a Sponsor Identity and an Application Service Provider Identity is not supported.

END OF CHANGES