3GPP TSG-SA/WG2 Meeting #138E	S2-2003740

# Elbonia, 1 – 5 June, 2020

Source:	ZTE, Intel

Title:	New KI: Service continuity across network slices

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes a new key issue to study the mechanism to support service continuity across network slices to align with RAN’s study for Enhancement on Network Slicing.

# 1	Introduction

Based on the SID RP-193254 [1], the following objective can be found

- Study necessity and mechanisms to support service continuity, including [RAN3]

  - For intra-RAT handover service interruption, e.g. target gNB doesn’t support the UE’s ongoing slice, study slice re-mapping, fallback, and data forwarding procedures. Coordination with SA2 is needed.

There is no corresponding key issue covers the service continuity part in SA2 study. This contribution proposes a new key issue to study the mechanism to support service continuity across network slice

# 2	Proposal

It is proposed to agree the following changes in TR 23.700-40.

************* Start Changes *************

## 5.X	Key Issue #n: Support for service continuity across different network slices

### 5.X.1	General Description

An S-NSSAI may not be supported homogenously within 5G network of a PLMN. When the UE moves around, intra-RAT handover may be triggered, if the target AMF or gNB does not support an S-NSSAI associated to one of the UE’s PDU Sessions, the S-NSSAI will be rejected by the target AMF or gNB according to Rel-15 and Rel-16 5GS design, which will interrupt the service continuity and cause bad user experience.

This key issue will study:

-	whether and how to support service continuity for intra-RAT handover (i.e. Xn based handover and N2 based handover) when the target gNB doesn’t support the UE’s ongoing PDU Session associated to a network slice.

NOTE: The solution should cover both LBO and Home-routed roaming scenarios.

Editor’s note:	Coordination with RAN3 is needed

*************** End Changes ***************