3GPP TSG-WG SA2 Meeting #139E e-meeting	S2-2004156

Elbonia, June 1 – 12, 2020

Source:	China Mobile

Title:	Key Issue #16, New Sol: UP optimization for edge computing

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This paper proposes a new solution for Key Issue #16.

1	Discussion

“Key Issue #16: New Sol: UP optimization for edge computing” was added to TR 23.700-91. This p-CR proposes a solution for what kind of analytics could be exposed to AF (edge computing) to improve UP optimization and support edge computing operations efficiently. The solution proposes that MEC subscribes to NWDAF estimated service experience for an application via NEF, and adjusts the service parameter strategy based on the estimated service experience result provided by NWDAF to match it with the network conditions to achieve the better user experience.

2	Proposal

This contribution proposes to implement the following new text to TR 23.700-91.

* * * Start of Change * * *

6.x	Solution #X for Key Issue #16: optimization for edge computing

6.x.1	Description

This clause describes a solution to optimization for edge computing NWDAF subscribes directly to UPF for the collection of UE data

6.x.2 Procedure

1.  Consumer MEC sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = group of UEs in area serviced by the MEC , Analytics Filter information = (Application ID, Analytics target period S-NSSAI, DNN, Area of Interest)) to NWDAF via NEF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = group of UEs in the area serviced by the MEC)

2b. NWDAF subscribes the network data from 5GC NF(s) by invoking Nnf_EventExposure_Subscribe service operation

2c. With these data, the NWDAF trains evaluation model and estimates the Service experience for the application in the area serviced by the MEC

3. The NWDAF provides the data analytics and predicted result, i.e. the estimated Service Experience to the MEC via NEF by means of Nnwdaf_AnalyticsSubscription_Notify.

4.  MEC conducts analysis based on the analysis and prediction of the Service experience data provided by NWDAF and determines the adjustment strategy of service parameters based on the analysis results. The service parameters of video business may be the video bit rate, frame rate, codec format, compression quality, screen size, etc. In this way, the adjustment strategy of service parameters can better match the network conditions and achieve the best user experience.

6.x.3	Impacts on services, entities and interfaces

MEC:

-	Requires to support service interface between NWDAF and MEC via NEF

-	Requires to support the adjustment strategy of service parameters based on the analysis results provided by NWDAF

NWADAF:

-	Requires to support service interface between NWDAF and MEC via NEF