SA WG2 Meeting #139E e-meeting	S2-2004330r03

June 1 – 12, 2020, Elbonia	(revision of S2-2004107 merge S2-2004106)

Source:	vivo

Title:	KI #7, New Sol: Mode switch between multicast and unicast

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: Add new Solution for mode switch between multicast and unicast.

# 1. Introduction

In TR 23.757, the key issue #7 about reliable delivery mode switching between unicast and multicast are agreed as follows:

This key issue aims at providing support for dynamic delivery mode switching in the 5GS. Depending on the number of devices receiving a specific content, their location, and RAN considerations, it may be necessary to support reliable and efficient delivery mode switching between unicast and multicast modes, i.e., to be able to dynamically transfer a unicast session to multicast delivery mode and vice versa. In addition, when a UE is receiving a multicast session, it may move across NG-RAN nodes and it is possible that the UE moves from a NG-RAN node that supports MBS to one that does not support MBS, or vice versa.

The following aspects will be studied:

-	Triggers for delivery mode switching in 5GS.

-	How delivery mode switching between unicast and multicast modes is performed in the 5GS (including the UE) while supporting service continuity.

NOTE:	During the study of this key issue, RAN WGs, SA4 and SA6 will be involved, if needed.

In order to support mode switch, it is necessary to discuss the triggers, NF functions and signalling procedures.

The triggers for mode switching maybe include but not limited to the following factors:

- # of devices receiving the specific content.

- # of devices in a location area.

- Measurement of multicast signalling strength.

The first factor may be collected and monitored by AF or by 5GC. The second factor may be collected and monitored by 5GC or by RAN node. The last factor is monitored by UE.

When the AF transmits the service data using IP multicast address, the network may use individual delivery method or shared delivery method to transmit the service data.

There are some condition for the 5GS to determine mode switch based on, e.g., target RAN node capability during mobility. The UE may choose to stay in multicast mode with delivery method change between "shared" and "individual".

# 2. Proposal

It is proposed to include following new solution for mode switch between unicast and multicast into TR 23.757.

FIRST CHANGE (NEW TEXT)

## 6.X	Solution #X: Mode switch between multicast and unicast

### 6.X.1	Functional description

#### 6.X.1.1 General

This solution addresses KI#7 "Reliable delivery mode switching between unicast and multicast". The architecture of this solution is based on architecture alternative-1 (see Annex A.1).

The content delivery mode may be unicast instead of multicast due to the number of the user is too limited or the multicast signalling is weak. When users successively join into the service, the number of user may be above a threshold for the service and trigger mode switch for the service, e.g. from unicast mode to multicast mode or vice versa. When users successively leave the service, it also may trigger mode switch for the service, e.g. from shared mode to dedicated mode.

When the service data is transferred using multicast mode, individual or shared delivery method may be used. There are some conditions for the 5GS to determine mode switch based on, e.g., target RAN node capability during mobility,. The UE may choose to stay in multicast mode with delivery method change between "shared" and "individual".

Editor’s Note: If the UE chooses to stay in multicast mode with delivery method change between "shared" and "individual", how the RAN and UE are expected to handle the MBS packet is FFS. The impact on RAN depends on RAN WG study.

Editor’s Note: The solution assumes that SMF1 and SMF2 are the same. Mode switching involving SMF1 and SMF2 is FFS.

#### 6.X.1.2 Potential triggers for mode switch

The triggers for mode switch maybe include but not limited to the following factors:

- Number of devices receiving the specific content.

- Number of devices in a specific location area related to the specific content.

- Measurement of multicast signalling strength.

The first factor may be collected and monitored by AF or by 5GC. The second factor may be collected and monitored by 5GC or by RAN node. The last factor is monitored by UE.

### 6.X.2	Procedures

#### 6.X.2.3 Multicast to unicast mode switch triggered by SMF

Figure 6.X.2.3-1 shows the procedure of switch from multicast mode to unicast mode triggered by SMF, which has a precondition that the AF has used IP multicast address to deliver multicast service data to users.

Figure 6.X.2.3-1: Multicast to unicast mode switch triggered by SMF

1.	The UE has on-going group communications with the AF and is receiving service data by multicast mode. The SMF decides to perform switch from multicast mode to unicast mode based on, e.g. the service configuration provisioned by the AF, the number of user for the multicast service, or the number of user in a location area. The SMF identifies the users need to perform mode switch.

The following steps 2~6 are performed for each identified user.

2.	The SMF may get the AMF ID from UDM for the identified user if needed.

3.	The SMF generates a N1 MB message (TMGI/IP multicast address) for mode switch instruction and invokes Namf_Communication_N1N2MessageTransfer (SUPI, N1 MB message) service operation towards the AMF.

4.	The AMF forwards the N1 MB message to the user.

5.	The UE decides to use unicast mode. The UE may initiate PDU Session Establishment / Modification procedure or Service Request procedure to activate the unicast path with the AF.

6.	The UE requests to receive data with unicast mode from the AF over application layer, e.g., GC1 interface. The AF may update the service to trigger the UE to activate a unicast path. The AF sends the service data to the UE with unicast mode.

7.	The UE starts receiving service data via unicast mode with break-before-make style or make-before-break style.

8.	The AF, after a specific time, may determine to stop service data delivery using IP multicast address, e.g. when all user registered in the service have changed to use unicast mode.

#### 6.X.2.4 Multicast to unicast mode switch triggered by RAN

Figure 6.X.2.4-1 shows the procedure of switch from multicast mode to unicast mode triggered by RAN, which has a precondition that the AF has used IP multicast address to deliver multicast service data to users.

Figure 6.X.2.4-1: Multicast to unicast mode switch triggered by RAN

1.	The UE has on-going group communications with the AF and is receiving service data by multicast mode. The RAN node decides to perform mode switch based on, e.g., the number of interesting user for the multicast service.

2.	The UEs are in RRC_CONNECTED state, e.g. for counting. The RAN node sends an AN message (List of TMGI) to the UE.

Editor's Note: The need for Step 2 is FFS.

3.	The UE decides to use unicast mode. The UE may initiate PDU Session Establishment / Modification procedure or Service Request procedure to activate the unicast path with the AF.

4.	The UE requests to receive data with unicast mode from the AF over application layer, e.g., GC1 interface. The AF sends the service data to the UE with unicast mode.

5.	The UE starts receiving service data via unicast mode with break-before-make style or make-before-break style.

6.	The AF, after a specific time, may determine to stop service data delivery using IP multicast address, e.g. when all user registered in the service have changed to use unicast mode.

#### 6.X.2.8 Individual to shared delivery method change triggered by SMF

Figure 6.X.2.8-1 shows the procedure of mode switch with delivery method change from "individual" to "shared" triggered by SMF.

Figure 6.X.2.8-1: Individual to shared delivery method triggered by SMF

1.	The UE has on-going group communications with the AF and is receiving service data by multicast mode. The 5GS is using individual delivery method to deliver multicast service data. The SMF decides to perform mode switch based on, e.g., the service configuration update provisioned by the AF, the number of user for the multicast service, or the number of user in a location area. The SMF identifies the member users that need to perform mode switch.

The following steps 2~6 are performed for each identified user.

2.	The SMF may get the AMF ID from UDM for the identified user if needed.

3.	The SMF generates a N1 MB message (TMGI/IP multicast address) for mode switch instruction and invokes Namf_Communication_N1N2MessageTransfer (SUPI, N1 MB message) service operation towards the AMF. The N1 MB message may indicate delivery method.

4.	The AMF forwards the N1 MB message to the user.

5.	The UE decides to switch mode and chose to stay in multicast mode with delivery method changed. If the transmission resource information for the TMGI cannot be detected, e.g. in the SIB, the UE performs multicast operation to update the multicast session for transmission resource reservation for the shared delivery method, e.g. initiates User Multicast Session Join / User Multicast Context Update procedure in solution 4 as described in clause 6.4. The 5GS will update the PDU Session to de-activate the individual leg

6.	The UE starts receiving multicast data using multicast radio technology with break-before-make style or make-before-break style.

#### 6.X.2.9 Individual to shared delivery method change triggered by RAN

Figure 6.X.2.9-1 shows the procedure of mode switch with delivery method change from "individual" to "shared" triggered by RAN, which has a precondition that the serving RAN node knows the mapping between TMGI and the individual path.

Figure 6.X.2.9-1: Individual to shared delivery method triggered by RAN

1.	The UE has on-going group communications with the AF and is receiving service data by multicast mode. The 5GS is using individual delivery method to deliver multicast service data. The serving RAN node decides to perform mode switch for some multicast groups based on, e.g., the number of individual legs for a multicast group.

Editor's note:	How the RAN node detects multiple individual legs for a multicast group is FFS.

2.	The RAN node may trigger multicast operation to reserve shared CN tunnel resources for the multicast groups if not exists, e.g. performs steps 2~15 of User Multicast Session Join via AN in solution 4 as described in clause 6.4.

The following steps 3~5 are performed for each UE.

3.	The serving RAN node sends an AN message (List of TMGI) to the UE receiving the multicast service data for mode switch.

4.	The UE starts receiving multicast data using shared delivery method with break-before-make style or make-before-break style.

5.	The UE de-activate the individual leg, e.g. initiates User Multicast Session Leave procedure in solution 4 as described in clause 6.4.

#### 6.X.2.11 Shared to individual delivery method change triggered by SMF

Figure 6.X.2.11-1 shows the procedure of mode switch with delivery method change from "shared" to "individual" triggered by SMF.

Figure 6.X.2.11-1: Shared to individual delivery method triggered by SMF

1.	The UE has on-going group communications with the AF and is receiving service data by multicast mode. The 5GS is using shared delivery method to deliver multicast service data. The SMF decides to perform mode switch based on, e.g., the service configuration update provisioned by the AF, the number of user for the multicast service, or the number of user in a location area. The SMF identifies the member users need to perform mode switch.

The following steps 2~6 are performed for each identified user.

2.	The SMF may get the AMF ID from UDM for the identified user if needed.

3.	The SMF generates a N1 MB message (TMGI/IP multicast address) for mode switch instruction and invokes Namf_Communication_N1N2MessageTransfer (SUPI, N1 MB message) service operation towards the AMF. The N1 MB message may indicate delivery method.

4.	The AMF forwards the N1 MB message to the user.

5.	The UE decides to switch mode and chose to stay in multicast mode with delivery method changed. The UE performs multicast operation to update the multicast session for transmission resource reservation for the shared delivery method, e.g. initiates User Multicast Session Join / User Multicast Context Update procedure in solution 4 as described in clause 6.4. The 5GS will update the PDU Session to activate the individual leg

6.	The UE starts receiving multicast data via PDU Session with break-before-make style or make-before-break style. The SMF may command to stop multicast service data forwarding via shared delivery method based on, e.g., the number of UE that has initiated User Multicast Session Join / User Multicast Context Update procedure.

#### 6.X.2.12 Shared to individual delivery method change triggered by RAN

Figure 6.X.2.12-1 shows the procedure of mode switch with delivery method change from "shared" to "individual" triggered by RAN.

Figure 6.X.2.12-1: Shared to individual delivery method triggered by RAN

1.	The UE has on-going group communications with the AF and is receiving service data by multicast mode. The 5GS is using shared delivery method to deliver multicast service data. The serving RAN node decides to perform mode switch based on, e.g. the number of interesting user for the multicast service.

The following steps 2~4 are performed for each UE.

2.	The UEs are in RRC_CONNECTED state, e.g. for counting. The serving RAN node sends an AN message (List of TMGI) to the UE for mode switch.

3.	The UE decides to stay in multicast mode with delivery method changed. The UE performs multicast operation to update the multicast session for transmission resource reservation for the individual delivery method, e.g. initiates User Multicast Session Join via CN / User Multicast Context Update procedure in solution 4 as described in clause 6.4. The 5GS will update the PDU Session to activate the individual leg

4.	The UE starts receiving multicast data using individual delivery method with break-before-make style or make-before-break style.

### 6.X.3	Impacts Analysis

Editor's note: Impacts are FFS.

UE:

1. Support receiving mode switch command using NAS or AS message.

RAN node:

1. Support sending AS mode switch command message.

SMF:

1. Support sending NAS mode switch command message.

END OF CHANGES