3GPP TSG-WG SA2 Meeting #139E e-meeting	S2-2004156

Elbonia, June 1 – 12, 2020

Source:	China Mobile

Title:	Key Issue #16, New Sol: UP optimization for edge computing

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This paper proposes a new solution for Key Issue #16.

1	Discussion

“Key Issue #16: New Sol: UP optimization for edge computing” was added to TR 23.700-91. This p-CR proposes a solution for what kind of analytics could be exposed to AF (edge computing) to improve UP optimization and support edge computing operations efficiently. The solution proposes that MEC subscribes to NWDAF estimated service experience for an application via NEF, and adjusts the service parameter strategy based on the estimated service experience result provided by NWDAF to match it with the network conditions to achieve the better user experience.

2	Proposal

This contribution proposes to implement the following new text to TR 23.700-91.

* * * Start of Change * * *

6.x	Solution #X: optimization for edge computing

6.x.1	Description

6.x.1.1	General

This is a solution for the Key Issue#16: UP optimization for edge computing.

6.x.1.2 Procedure

1.  The consumer MEC sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = group of UEs in area serviced by the MEC , Analytics Filter information = (Application ID, Analytics target period S-NSSAI, DNN, Area of Interest)) to NWDAF via NEF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = group of UEs in the area serviced by the MEC).

2b. NWDAF subscribes the network data from 5GC NF(s) by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF estimates the Service experience for the application for the MEC.

3. The NWDAF provides the data analytics to the MEC via NEF by means of Nnwdaf_AnalyticsSubscription_Notify.

Note : The above steps can reuse the procedure defined in clause 6.4.4, TS 23.288, with the MEC acted as a consumer.

4.  Based on statistics or prediction of observed service experience provided by NWDAF, MEC determines to adjust service parameters. E.g., service parameters of video for adjustment may be bit rate, frame rate, codec format, compression parameter, screen size, etc. In this way, the service parameters adjusted can better match the network conditions and achieve the better user experience.

6.x.2	Impacts on services, entities and interfaces

MEC:

-	Support the adjustment of service parameters based on analysis provided by NWDAF;