SA WG2 Meeting #S2-140E	S2-2004799

19 August - 01 September, 2020, Electronic, Elbonia

3GPP TSG-CT WG3 Meeting #110e	C3-203609

# E-Meeting, 02nd – 11th June 2020

Title:	Reply LS on PAP/CHAP and other point-to-point protocols usage in 5GS

Response to:	LS C1-202933 on PAP/CHAP and other point-to-point protocols usage in 5GS from CT1

Release:	Release 16

Work Item:	5GProtoc16

Source:	CT3

To:	CT1

Cc:	SA2, SA3, CT4

Contact person:	Waqar Zia

wzia@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT3 thanks CT1 for the LS on PAP/CHAP and other point-to-point protocols usage in 5GS.

CT1’s question:

The following point-to-point protocols are defined in TS 24.008 as (e)PCO parameters which can be exchanged between the UE and the network during GPRS and EPS session management procedure:

- C021           Link Control Protocol (LCP)

- C023           Password Authentication Protocol (PAP)

- 8021            IP Control Protocol (IPCP)

- C223           Challenge Handshake Authentication Protocol (CHAP)

Are above point-to-point protocols supported in 5GS (i.e. Rel-15 and/or Rel-16) as (e)PCO parameters or to be supported in the near future (i.e. Rel-17)?

CT3’s Answer: CT3 has reviewed its current specifications related to the question above and the summary of the current status is in the following:

TS 23.401 cl.4.4.3.3 describes PPP support for P-GW:

“The network does not support PPP bearer type in this version of the specification. Pre-Release 8 PPP functionality of a GGSN may be implemented in the PDN GW.”

Since PPP bearer type was deprecated in Rel-8, there is no support in TS 29.061 and 29.561 describing the interworking with external DN using PPP in 5G.

For the authentication, if PCO includes PAP/CHAP, SMF doesn’t currently support using RADIUS/Diameter protocol including the corresponding PAP/CHAP information elements directly for interaction with external DN-AAA server, in the current release 15 and 16, but a combo-SMF+PGW node can support Radius/Diameter interface through its PGW component.

# 2	Actions

To CT1

ACTION: 	CT3 asks CT1 to take the above answer into account.

# 3	Dates of next TSG CT WG 3 meetings

3GPP TSG CT3#111e	19th – 28th August 2020	E-Meeting

3GPP TSG CT3#112e	TBD	E-Meeting