3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005365

Elbonia, August 19 – September 01, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Update Solution#38 to support simultaneous change of UL CL and local PSA

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This contribution proposes to update solution#38 to support the relocation of EASs using forwarding tunnel from target UL CL to source UL CL after simultaneous change of UL CL and local PSA as described in clause 4.3.5.7 of TS 23.502.

# 1. Text Proposal

It is proposed to capture the following changes in the TR 23.748.

# * * * * First change * * * *

## 6.38	Solution #38: EAS change with reducing packet loss in uplink

### 6.38.1	Description

The proposal solution addresses the KI#2: Edge relocation, in particular on potential improvements of the coordination of change of the Edge Application Server and (local) PSA to support seamless change, e.g. preventing or reducing packet loss.

The procedure of "Change of additional PDU Session Anchor for IPv6 multi-homing or UL CL" was defined in clause 4.3.5.6 of TS 23.502 [3]. It can happen after e.g. Xn based handover and UL CL has changed. During the procedure, the on-fly uplink packets, from UL CL to source PSA and source EAS , which are to be sent to N6 or are being transmitted over N6, may be lost if the source EAS stops to handle packets during EAS relocation before all packets are received by the source EAS.

NOTE 1:	This solution assumes the UE related contexts, including application layer contexts and transporting layer contexts (e.g. TCP), if any, can be transferred from old EAS to new EAS by EAS relocation mechanisms, to ensure the packets can be handled seamlessly by the new EAS to avoid service interruption. The solution assumes the packet with original EAS IP address can be handled by the new EAS after the relocation of the EAS server application, this can be achieved e.g. with use of Anycast EAS IP address or by design of EAS to allow it to handle IP address targeting to a different EAS of same service, but details are out of scope of 3GPP.

NOTE 2:	This solution also applies to relocation of those EASs using forwarding tunnel from target UL CL to source UL CL after simultaneous change of UL CL/L-PSA as described in clause 4.3.5.7 of TS 23.502 [3]. To avoid impact on service continuity, this solution proposes to enhance the procedure to support preventing or reducing packet loss with the following mechanism: UL packets are buffered in the target PSA until the target EAS is ready to handle the incoming application traffic.

Furthermore, this solution proposes to perform one of two options to enable in-order packet delivery:

Option 1:

Establish a forwarding tunnel between source PSA and target PSA to avoid loss of uplink packets that are transferring between (target) UL CL and source PSA. SMF may also instruct (target) UL CL to send an End Maker between (target) UL CL - source PSA - target PSA to ensure in-order UL transferring on target PSA.

This option can ensure packet lossless and in-order delivery within 5GS.

Option 2:

Introduce an AF-defined Flow End Maker between (target) UL CL and source local DN/EAS to trigger EAS relocation. With that, it can be guaranteed that no uplink packets sending over the old path before the EAS relocation starts. As other user plane packet, the Flow End Marker should be a user plane packet with EAS IP as the destination IP address.

This option can ensure packet lossless between UE and EAS, including N6 interface.

### 6.38.2	Procedures

#### 6.38.2.1 Procedures for preventing packet loss in uplink with change of additional PSA

Based on the procedure specified in clause 4.3.5.6 of TS 23.502 [3], Figure 6.38.2.1-1 illustrates the procedure of edge relocation with preventing packet loss and in-order packet delivery in uplink.

Figure 6.38.2.1-1: Procedure of preventing packet loss in uplink

If seamless Edge relocation is needed, the AF influence traffic routing is enhanced to indicate the 'seamless Edge relocation' requirement (i.e. including a 'seamless Edge relocation' indication) to PCF. The PCF generates a PCC rule(s) based on the AF request and the PCC rule(s) sent to the SMF includes the 'seamless Edge relocation' indication.

For Option 1, if seamless Edge relocation is needed, the SMF binds the application flow to a dedicated QoS Flow.

1.	The SMF decides to change the local PSA of a PDU Session with UL CL.

2.	The SMF sends an early notification to the AF after target PSA (i.e. PSA2) is selected and waits for a notification response from the AF. The AF may reply in positive to the notification of step 2a by indicating that buffering of uplink traffic in the PSA2 is required if the 'seamless Edge relocation' indication was not set and the AF requests the same treatment on demand.

For Option 2, the response includes Flow End Marker information, which contains an AF-defined Flow End Marker packet.

3.	The SMF configures the PSA2 as specified in step 2 in clause 4.3.5.6 of TS 23.502 [3], and indicates the PSA2 to buffer uplink traffic. PSA1 (i.e. source PSA) keeps receiving downlink traffic from EAS1 and send it to the UE until it is released in step 9.

For Option 1, the SMF also obtains the forwarding tunnel info from PSA2.

4.	For Option 1, the SMF sends an N4 Session Modification Request to the PSA1 to establish forwarding tunnel towards PSA2. The PSA1 starts to forward the received uplink packets to the PSA2 via the tunnel.

5.	The SMF sends an N4 Session Modification Request to the UL CL to update the UL CL rules regarding to the traffic flows that the SMF tries to move from PSA1to PSA2. The N4 Session Modification Request message contains the identifications of traffic filter that needs to be updated and the tunnel ID of PSA2.

For Option 1, if an 'seamless Edge relocation' indication is included in the PCC rule, the SMF also indicates the UL CL to send End Marker per QoS Flow to the PSA1 before it starts to forward the traffic to the PSA2 to ensure the packets are in-order delivered from PSA2 to EAS2.

For Option 2, the SMF provides the Flow End Marker received in step 2 to the UL CL and indicates the UL CL to send it to the PSA1 before it starts to forward the traffic to the PSA2.

6.	For Option 1, if an 'seamless Edge relocation' indication is included in the PCC rule, the End Marker is sent from UL CL to the PSA1 then PSA2. When the End Marker is received, the PSA2 knows that no more uplink packet from the old path for the QoS Flow.

7.	For Option 2, the Flow End Marker is sent from UL CL to PSA1 then routed to source Local DN/EAS as other user plane packets. When the Flow End Marker is received, the source EAS knows that no more uplink packets from the UE. The source EAS stops to handle packets and EAS relocation can be started.

8.	The SMF sends a Late Notification to the AF. When EAS relocation is completed, the AF sends a notification response to the SMF.

9.	The SMF updates the PSA2 by indicating the PSA2 to send the buffered uplink packets. The SMF releases PSA1.

For Option 1, the forwarded packets received before the End Marker packet will be sent to EAS before those received directly from UL CL.

#### 6.38.2.2 Procedures for preventing packet loss in uplink with simultaneous change of UL CL and additional PSA

Based on the procedure specified in clause 4.3.5.7 of TS 23.502 [3], Figure 6.38.2.2-1 illustrates the procedure of edge relocation with preventing packet loss and in-order packet delivery in uplink.

SMF

UPF

(PSA2)

EAS2

EAS1

AF

UE

UL/DL Data

2a. DNAI change notification

(R)AN

UPF

(UL CL1)

Option 1

1. SMF determines to change the ULCL and local PSA

2b. Positive response

3. SMF establishes PSA2

UL Buffering

7a. N4 Session Modification Request

7b. N4 Session Modification Response

8a. N4 Session Modification Request

8b. N4 Session Modification Response

Option 1

9a. End Marker

9b. End Marker

10a. Flow End Marker

10b. Flow End Marker

Option 2

UL Data

EAS relocation

11b. Response for Late Notification

12. SMF updates PSA2 and releases UL CL 1 and PSA1

Buffered UL Data

UL/DL Data

11a. Late Notification

UPF

(PSA0)

UPF

(PSA1)

UPF

(UL CL2)

4. SMF establishes UL CL2 and tunnel between UL CL1 and UL CL2

5. SMF updates PSA0 for downlink traffic

6. SMF updates (R)AN for uplink traffic

UL Data

UL Data (Option 1)

)

Figure 6.38.2.2-1: Procedure of preventing packet loss in uplink

If seamless Edge relocation is needed, the AF influence traffic routing is enhanced to indicate the 'seamless Edge relocation' requirement (i.e. including a 'seamless Edge relocation' indication) to PCF. The PCF generates a PCC rule(s) based on the AF request and the PCC rule(s) sent to the SMF includes the 'seamless Edge relocation' indication.

For Option 1, if seamless Edge relocation is needed, the SMF binds the application flow to a dedicated QoS Flow.

1.	The SMF decides to change the UL CL and local PSA of a PDU Session.

2.	The SMF sends an early notification to the AF after target PSA (i.e. PSA2) is selected and waits for a notification response from the AF. The AF may reply in positive to the notification of step 2a by indicating that buffering of uplink traffic in the PSA2 is required if the 'seamless Edge relocation' indication was not set and the AF requests the same treatment on demand.

For Option 2, the response includes Flow End Marker information, which contains an AF-defined Flow End Marker packet.

3.	The SMF configures the PSA2 as specified in step 2 in clause 4.3.5.7 of TS 23.502 [3], and indicates the PSA2 to buffer uplink traffic. PSA1 (i.e. source PSA) keeps receiving downlink traffic from EAS1 and send it to the UE until it is released in step 12.

For Option 1, the SMF also obtains the forwarding tunnel info from PSA2.

4.	The SMF establishes Target UL CL (i.e. UL CL2) and an N9 forwarding tunnel between the Source UL CL (i.e. UL CL1) and UL CL2 as specified in step 3 in clause 4.3.5.7 of TS 23.502 [3].

5.  The SMF updates PSA0 for downlink traffic as specified in step 4 in clause 4.3.5.7 of TS 23.502 [3].

6.  The SMF updates RAN as specified in step 6 in clause 4.3.5.7 of TS 23.502 [3].

7.  For Option 1, the SMF sends an N4 Session Modification Request to the PSA1 to establish forwarding tunnel towards PSA2. The PSA1 starts to forward the received uplink packets to the PSA2 via the tunnel.

8.	The SMF sends an N4 Session Modification Request to the UL CL2 to update the UL CL rules regarding to the traffic flows that the SMF tries to move from PSA1 to PSA2. The N4 Session Modification Request message contains the identifications of traffic filter that needs to be updated and the tunnel ID of PSA2.

For Option 1, if an 'seamless Edge relocation' indication is included in the PCC rule, the SMF also indicates the UL CL2 to send End Marker per QoS Flow to the PSA1 before it starts to forward the traffic to the PSA2 to ensure the packets are in-order delivered from PSA2 to EAS2.

For Option 2, the SMF provides the Flow End Marker received in step 2 to the UL CL2 and indicates the UL CL2 to send it to the PSA1 before it starts to forward the traffic to the PSA2.

9.	For Option 1, if a 'seamless Edge relocation' indication is included in the PCC rule, the End Marker is sent from UL CL2 to the PSA1 then PSA2. When the End Marker is received, the PSA2 knows that no more uplink packet from the old path for the QoS Flow.

10.	For Option 2, the Flow End Marker is sent from UL CL2 to PSA1 via ULCL1 then routed to source Local DN/EAS as other user plane packets. When the Flow End Marker is received, the source EAS knows that no more uplink packets from the UE. The source EAS stops to handle packets and EAS relocation can be started.

11. The SMF sends a Late Notification to the AF. When EAS relocation is completed, the AF sends a notification response to the SMF.

12.	The SMF updates the PSA2 by indicating the PSA2 to send the buffered uplink packets. The SMF releases PSA1.

For Option 1, the forwarded packets received before the End Marker packet will be sent to EAS before those received directly from UL CL2.

### 6.38.3	Impacts on services, entities and interfaces

SMF:

-	Configure the target PSA to buffer uplink packets and to forward them when EAS relocation is completed.

-	For Option 1, configure the source PSA to establish an N9 tunnel between the target PSA to forward uplink traffic, and optionally, configure the (target) UL CL to send End Marker when it stops to forward uplink packets to the source PSA and configure the target PSA to send the packets to the target EAS in-order based on the End Marker.

-	For Option 2, forward Flow End Marker received in PCC rules to the source PSA.

UPF acting as a (target) UL CL:

-	For Option 1, optionally send the End Marker per QoS Flow to the source PSA then target PSA before it starts to forward the traffic to the target PSA.

-	For Option 2, forward Flow End Marker received from SMF to the source PSA then source Local DN/EAS before it starts to forward the traffic to the target PSA.

UPF acting as a source PSA:

-	For Option 1, establish an N9 tunnel between the target PSA, and forward uplink packets from (target) UL CL to the target PSA.

UPF acting as a target PSA:

-	Buffer uplink packets until EAS relocation is completed.

-	For Option 1, optionally send the packets to the target EAS in-order based on the End Marker.

AF:

-	Optionally, include a 'seamless Edge relocation' indication in the AF request sent to the PCF.

-	Optionally, indicating that buffering of uplink traffic in the target PSA is required in Early Notification response to SMF.

-	For Option 2, include Flow End Marker information in positive AF response sent to the SMF.

PCF:

-	Optionally, include the 'seamless Edge relocation' indication in the PCC rule sent to the SMF.

# * * * * End of changes * * * *