SA WG2 Meeting #140E	S2-2005845

19 August - 02 September 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #1: Evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution gives the evaluations for the solutions of KI#1 and proposes the conclusion for KI#1.

# 1		Discussion

This contribution proposes conclusions and way forward for Key Issue #1 for Uplink Time Synchronization.

# 2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

## 7.X	Key Issue #1: Uplink Time Synchronization

Solution #1 is the only solution addressing KI#1 Uplink Time Syncthonization.

# * * * * Second change * * * *

## 8.X	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, it is recommended to select solution 1 as the basis for normative work. Following are some principles to be considered for the way forward:

- DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

- In case of synchronizing TSN end stations behind 5G System (NW-TT), all gPTP messages are transmitted using the user-plane resources in 5GS and follows the principle that one PDU session is established per DS-TT port for a UPF.

- In case of synchronizing TSN end stations behind other UE(s), all gPTP messages are transmitted using the user-plane resources in 5GS and follows the principle that more than one PDU sessions are used. The 5GS ensures that the gPTtP messages from source UE are transmitted only to other UE(s) and not sent back (“avoids play back”) to the source UE.

- The NW-TT may run the BMCA to determine the port roles for the DS-TT ports connected to the same 5GS TSN bridge and expose the BMCA outcome to the TSN AF.