3GPP TSG-SA WG2 Meeting #140e 	S2-2005975

19 August - 01 September 2020, Electronic meeting        (revision of S2-2004949)

Source:	Ericsson, Qualcomm Incorporated, Huawei, Tencent

Title:	KI#1, Way Forward: Distributed Anchor connectivity model

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes a way forward for KI#1 and Distributed Anchor Connectivity model

# 1	Introduction

This contribution evaluates the different solutions for KI#1 and Distributed Anchor Connectivity Model and proposes a way forward.

# 2	Discussion

Removed to avoid misalignment with the changes part.

# 3	Proposal

************* Start Changes *************

# 7	Overall Evaluation

## 7.x	Evaluation of Solutions for Key Issue #1

### 7.x.y	Evaluation of Solutions for Key Issue #1 for Distributed Anchors

Many solutions propose using DNS for EAS discovery for distributed anchor connectivity model.

A DNS based mechanism for EAS discovery supports the different EAS deployments described in KI#1. It has no impact on the Application itself and keeps UE unaware of the application deployment (at edge or at central DN) and application ownership (e.g. the EAS is owned by the MNO or by a third party) aspects. The TR includes DNS based solutions for the other connectivity models as well, and so, selecting DNS, applications can be developed to rely on DNS for EAS discovery and be agnostic of the operator connectivity model chosen.

Distributed anchor part of Solutions #2, #4, #5 and #10 describe DNS based working solutions for EAS discovery for distributed anchor connectivity model. These solutions are using DNS state of the art: many Authoritative (DNS) Name servers already today return different responses based on the perceived topological location of the user, either using the source IP address of the DNS query or ECS when received according to RFC 7871 [7].

A pre-requisite for solutions in the distributed anchor point scenario is to anchor the UE according to subscription policies.

The goal is to provide to DNS addressing information related to the UE topological location. These solutions show that in 5GC that can be achieved by:

A.) Providing a DNS for the PDU Session that is near the PSA, the DNS request can be solved to an Application server which is closest to that PSA. Solution alternatives:

-   The 5GC(SMF) provides a DNS that is closest to the PSA

-   The 5GC(SMF) provides an Anycast DNS address

B.) Including in the DNS Query an ECS that is representative of the UE location /N6 interface, DNS can provide an EAS that is closest to the PSA. Solution alternatives:

-    The 5GC(SMF) provides a DNS that supports RFC 7871 [7] and adds an ECS that is representative of the UE location /N6 interface, e.g. based on the user IP address after (an optional) NAT.

The above summary plus a selection of procedures from solutions #2, #4, #5 and #10 are to be promoted into normative phase. Procedures from these solutions that illustrate recommended solution versions are: 6.2.3.1, 6.5.2.6, 6.5.2.7, 6.10.2.1 and 6.10.2.2.

These solutions solve KI#1 under certain conditions that are to be documented together with the solutions in normative phase:

- These solutions require the corresponding geographical resolution support by the Authoritative (DNS) nameserver.

- These solutions are guaranteed to work if the operator provided DNS settings are used by the UE for the DNS Query. Guidelines should be captured to cover scenarios where the OS, user or applications may override the operator-provided DNS settings.

- When UE DNS Queries are sent to another DNS Resolver instead (that depends on the UE Application client, Browser and/or OS configuration) if that is centrally deployed and it does not support RFC 7871 [7], the selected AS might not be closest to the User PSA. Complementary application layer mechanisms may be needed to reselect AS based on the source IP address of the user application traffic.

- UE IP address is subject to privacy restrictions and shall not be sent to authoritative DNS resolvers outside the network operator

Solution #12 enhances 5GC to dynamically re-anchor a PDU session at the Edge (e.g. re-anchoring the PDU Session from central to distributed anchor connectivity model triggered by the user DNS for the Application name resolution). Once the session has a distributed anchor, Solution #10 could be used for EAS discovery.

Solution #12 has the DNS (LDNSR) in the SMF but also mentions other possibilities such as an external LDNSR.

NOTE 1: The solution can be adopted to support external LDNSR or LDNSR in UPF whereby the principle of the solution can be maintained.

NOTE 2: this solution could make sense to be defined as a variant for this connectivity model of LDNSR functionality, and if so, that may imply standardisation in normative phase.

Solution #14 is an application layer solution to discover an Edge AS with no impact on 5GC when used with distributed anchor connectivity model. It is up to the application provider to adopt or not this mechanism if desired. 5GC does not preclude this option. This solution may be mentioned during normative phase as a complement to DNS based discovery for distributed anchor connectivity. The solution assumes the Service Switch is pre-configured the mapping information between the IP address range supported by PSA and EAS information based on the agreement between the MNO and service provider. The solution does not provide a description of the procedure for this connectivity model, so no procedure of this solution is recommended into normative.

Then, regarding the rest of the solutions for KI#1 and Distributed Connectivity model:

Solutions #6 should not be promoted into normative for this scenario, but it should be evaluated with the rest of solutions for session breakout connectivity model.

Solution #18 addresses the scenario where DNS might not be able to return an AS that is closest to the user location. Unless EC App flows can be differentiated on L3 level, the proposal implies buffering, mapping and remapping and resolving in UPF all first packet of flows showing a new destination IP, which will impact latency and throughput. It is also unclear how it coexists with application layer solutions for service continuity: UPF may overwrites any new target EAS selected at App layer for that app client, which could break procedures for seamless AS relocation for load balancing, resilience or to adapt to edge relocation. This solution relies on IETF standards, but reverse DNS lookup is not critical to the normal function of the internet, and so, it is not universally adopted, which questions the effectiveness of the solution. Rapid mapping/remapping of PTR, SRV and A/AAAA is not well-suited to DNS, and will also require cache flushing throughout the DNS hierarchy. This solution is not recommended into normative.

*************** Next Change ***************

# 9	Conclusions

## 9.x	Conclusions regarding solutions for Key Issue #1 for Distributed Anchors

Distributed anchor part of Solutions #2,#4, #5 and #10 describe DNS based working solutions for EAS discovery for distributed anchor connectivity model and the following selection of procedures from solutions #2, , #5 and #10 are to be promoted into normative phase: 6.2.3.1, 6.5.2.6, 6.5.2.7, 6.10.2.1 and 6.10.2.2.

NOTE:  6.5.2.6, 6.5.2.7 are included based on the understanding they no more refer to “Translate FQDN using LDN/N6 path information” and to “uses distance/closeness metrics”.

Decision for anchoring of the UE in the distributed anchor point scenario for EC shall be described as

- using subscription policy information to set proper UE policy (e.g. URSP via usage of dedicated DNN), or

- to apply proper policies at session (SMF) level if the UE policy (e.g. URSP via usage of dedicated DNN) cannot be used.

Privacy issues derived from potentially using UE IP address for ECS DNS in an external authoritative DNS in the related solutions shall be addressed during the normative phase.

Editor’s Note: Whether solution 12 is recommended into normative work for KI 1 will be defined at next meeting. The use of the solution 12 for KI#5 will be handled when KI#5 is discussed.

Solution #14 is an application layer solution to discover an Edge AS with no impact on 5GC when used with distributed anchor connectivity model. 5GC does not preclude this option. This mechanism may be informatively mentioned during normative phase as a complement to DNS based discovery for distributed anchor connectivity. The solution assumes the Service Switch is pre-configured with the mapping information between the IP address range supported by PSA (not the PSA information) and EAS information based on the agreement between the MNO and service provider.

Solutions #6 and #18 are not recommended into normative.

*************** End Changes ***************