# SA WG2 Meeting #140E (e-meeting)						S2-2004879

August 19th – September 2nd, 2020, Elbonia

# Title:	Draft LS Response on Clarification on AAA-Server address

# Response to:	S2-2004786 / C4-203452

# Release:	Release-16

# Work Item:	eNS

Source:	SA WG2

To:	CT WG4

Cc:	SA WG3

Contact Person:

Name:	Peter Hedman

Tel. Number:

E-mail Address:	peter.hedman@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2004877, S2-2004878

1. Overall Description:

SA2 thanks CT4 for the LS pointing to inconsistencies in stage 2 specifications with regards to the handling of the AAA-S address in NSSAA procedures.

SA2 has agreed on the attached CRs fixing the inconsistencies in SA2 specifications and would like to answer CT4 questions as follows:

- Whether the AAA-S address is conveyed by AMF to the AUSF/NSSAAF in step4?

[S2 Response]: There is no need for the AMF to provide the AAA-S address to the NSSAAF in step 4 of the NSSAA procedure described in section 4.2.9.2 of TS 23.502.

- In clause 5.15.3 of 23.501, there is the text "the indication whether the S-NSSAI is subject to Network Slice-Specific Authentication and Authorization and associated AAA Server Address", does it imply that the AAA-S address is stored in UDM/UDR and AMF gets the associated AAA Server Address from the UDM/UDR? Or is there any other mechanism to get the associated AAA Server Address?

[S2 Response]: There is no need for the UDM to store and provide the AAA-S address to the AMF as the AMF does not interact with the AAA-S directly. Instead, the AMF needs to discover and select an NSSAAF at the HN as defined in section 6.3.17 of TS 23.501.

Since the NSSAAF resides at the HN, the NSSAAF is capable of determining the AAA-S that needs to be used for NSSAA procedures per S-NSSAI based on local configuration at NSSAAF.

2. Actions:

To CT WG4 group.

ACTION: 	SA2 kindly requests CT4 to take into consideration the answers to the provided questions.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#141e	xx/2020	E-Meeting