3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005392

Elbonia, August 19 – September 01, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, Sol #7: Updates of Solution #7

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	eNS_Ph2 / Rel-17

Abstract: This contribution proposes updates on Roaming support and evaluation to complete solution #7: ‘Support of Network Slice SLA for Maximum Number of PDU sessions parameter’ in TR 23.700-40.

1. Discussion

This contribution proposes to add an explicit description of roaming support in solution #7: ‘Support of Network Slice SLA for Maximum Number of PDU sessions parameter’ by providing a more clear information about roaming support for maximum number of PDU sessions parameter.

This contribution also proposes to add an evaluation section to complete solution #7.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-40.

# * * * * First change * * * *

6.7	Solution #7: Support of Network Slice SLA for Maximum Number of PDU sessions parameter

6.7.1	Introduction

This is a solution to Key Issue #2, "Support of network slice related quota on the maximum number of PDU sessions". This solution assumes the following:

-	It is assumed that Unified Data Repository (UDR) obtains information about network slice related global quota on the maximum number of PDU sessions to perform the enforcement of SLA of the number of PDU sessions in a control plane procedure, and supports the functionalities of storage and retrieval of slice SLA data.

-	It is assumed that 5GC may have multiple enforcement points (e.g., PCF instances of a Network Slice) to perform the SLA enforcement of network slice related quota on the maximum number of PDU sessions.

-	It is assumed that multiple enforcement points (i.e., PCF instances) of a network Slice are controlled by a central enforcement point (i.e., a primary PCF) of the network slice. The central enforcement point (e.g., a primary PCF) obtains the global quota of the slice SLA attributes, maximum number of PDU sessions from UDR.

-	It is assumed that the support of Network Slice related quota on the maximum number of PDU sessions applies to the associated Network Slice even when supported by multiple Network Slice instances.

-	It is assumed that PCF instance makes policy decisions based on information obtained from a central enforcement point (i.e. primary PCF) and local policy in the PCF.

-	It is assumed the central enforcement point (i.e., the primary PCF of a given S-NSSAI) supports the functionalities of controlling distribution of the network slice related quota on slice SLA parameters. The functionalities include the (re)-distribution of the received global quota of each slice SLA attribute from UDR to the associated multiple enforcement points (e.g., secondary PCF instances of the associated Network Slice). The central enforcement point (i.e., the primary PCF) can be found by its multiple distributed enforcement points (i.e., secondary PCFs) based on configuration or the assistance of the UDR.

6.7.2	High-level Description

A global slice SLA information is the global quota of the maximum number of PDU sessions for a given S-NSSAI, which is stored at the UDR. A local slice SLA information is local quota of the maximum number of PDU sessions for a given S-NSSAI, which is stored at the enforcement points, e.g. PCF instances.

The proposed solution highlights a distributed management of network slice related quota at the control plane performed by 5GC NFs: PCF and UDR. It considers a primary PCF of a given S-NSSAI for controlling distribution of the network slice related local quota to PCF instances. The secondary PCF instances, also known as SM-PCF for the session management, apply/enforce the associated network slice related local quota in PDU session establishment procedure.

The mechanisms of distributed management of a network slice attribute related quota by a central enforcement point (e.g. a primary PCF of a given S-NSSAI) provides a precise control of slice SLA for maximum number of PDU sessions.

At PDU session establishment, when local quota is not to be exceeded, PCF decides to accept the PDU session establishment request.

At PDU session establishment, when the quota is to be exceeded or no local quota is granted, a PCF makes a decision to delegate the request to the associated central enforcement point (i.e., a primary PCF) of the S-NSSAI whether to accept or reject the current PDU session establishment request. The PCF decides the request according to the operator policy, and/or quota status. The operator policy of acceptance or rejection of the PDU session establishment request are based on the following information:

-	ARP in authorized default QoS;

-	DNN;

-	Internal Group Id;

-	PDU Session Type;

-	Access Type;

-	User Location Information.

6.7.3	Procedures

6.7.3.1	General

The following figure represents a high-level principles and workflow of the solution.

Figure 6.7.3-1: A high-level workflow of the solution

1.	UDR in 5GC obtains and stores from OAM global slice SLA information including a global quota on the maximum number of allowed PDU sessions, to be used to perform the SLA control on the number of PDU sessions. A primary PCF of a given S-NSSAIobtains global quota from UDR to control the global quota distribution into local quotas to the secondary PCF instances. The UDR store the S-NSSAI and its associated primary PCF.

2.	A primary PCF provides the local slice SLA information including local quotas of the maximum number of allowed PDU sessions per S-NSSAI to its secondary PCF instances (which is for SM policy control) of the related Network Slice (enforcement points). The sum of the local quota to the associated secondary PCF instances of S-NSSAI shall be less or equal to the network slice related the global quota of the maximum number of PDU sessions per S-NSSAI.

3.	When a UE requests for a PDU session establishment for the network slice, the SMF instance of the network slice interacts with the proper PCF instance (enforcement point) for the quota enforcement by re-using existing PDU session establishment procedure, i.e. as part of the SM Policy handling.

4.	Quota enforcement:

4a.	Each PCF instance (enforcement point) associated to the S-NSSAI maintains the local quota sent by its primary PCF for the associated number of established PDU sessions. Each PCF instance  generates a policy counter to track the local quota and the local quota status (i.e., per S-NSSAI the actual number of established PDU sessions and whether the quota is to be exceeded or not). Upon receiving UE request for a PDU session establishment, the PCF instance decides the acceptance or rejection of the UE request for a PDU session establishment, based on local quota status, local quota and the operator policy.

4b.	Alternative to step 4a, the PCF instance may send a delegation request of quota enforcement for the indicated S-NSSAI to its primary PCF, e.g., when the local quota is to be consumed by the PCF instance. Based on the global quota status of the S-NSSAI, the primary PCF decides the acceptance or rejection of the delegation request for the UE request for a PDU session establishment on the indicated S_NSSAI and sends a response to the delegation request with the decision. The PCF instance checks the decision of delegation request and decides the acceptance or rejection of the PDU session establishment request. If there is no local quota left and the response to the delegation request comprises rejection, the acceptance or rejection of the PDU session establishment shall be based on the operator policy.

The PCF based quota enforcement naturally supports the handling of slice SLA quota exceeding exceptions or exemptions by allowing special policies for the quota enforcement, e.g., no quota enforcement for the VIP customers or emergency services (e.g., MPS, MCX or PPDR).

The detail procedure of quota enforcement in PDU session establishment procedure is described in clause 6.7.3.2

5.	Upon PCF decision from step 4, if a UE request for a PDU session establishment violates or exceeds the local quota, the PCF sends an indication, e.g. rejection message (e.g., based on the operator policy), to the serving SMF. The PDU session establishment Reject message is sent to the UE by the serving SMF (via the serving AMF) along with the back-off timer and a suitable cause value.

6.	Controlling local quota re-distribution

6a.	PCF instances (enforcement points) may request their local quotas update for the indicated S-NSSAI (e.g., when their local quotas are (about to be) consumed) to its primary PCF. The primary PCF based on the received requests may re-calculate and provide the updated local quota of the maximum number of PDU sessions associated with the indicated S-NSSAI to one or more PCF instances. The primary PCF supports mechanisms for the (re)-distribution of local quota(s) to one or more PCF instances of associated S-NSSAI based on the global quota status of established number of PDU sessions of the indicated S-NSSAI.

6b.	Independently of step 6, one or more PCF instances (enforcement points) may report/notify the local quota status to its primary PCF (e.g., periodically or event based). Based on the reported local quota the primary PCF can track the global status of number of established PDU sessions and provides to one or more secondary PCF instances an updated local quota (i.e., quota re-distribution) if required. It enables 5GS to know about the current number of PDU sessions accessing the network slice.

The detail procedure of controlling (re)-distribution of local quota is described in clause 6.1.3.4.

7.	A PDU session release for a S-NSSAI can be triggered by a UE (or network) any time.

7a.	A UE triggers a PDU session release request to the serving SMF (via AMF). The SMF triggers the update of policy counter to the associated PCF during SM Policy Association Termination procedure.

7b. Upon receiving a PDU session termination request from the SMF, the secondary PCF of the S-NSSAI increases the remaining local quota of the S-NSSAI by one. If the secondary PCF has no local quota been granted per operator's policy, the secondary PCF may interact with the primary PCF for the quota update.

7c. The response of the PDU session release request of the S-NSSAI is sent to the UE by the serving SMF, via AMF.

The detail procedure of the PDU session release of a S-NSSAI is described in clause 6.7.3.3.

In the roaming case, the quota control can be in the VPLMN or HPLMN. For more detail, it is described in clause 6.7.3.X.

6.7.3.2	PDU Session establishment and Slice SLA Quota Update

Figure 6.7.3.2-1: UE requested PDU session establishment procedure with quota enforcement

1.	UE requested PDU session establishment procedure (steps 1-7a) according to clause 4.3.2.2.1 in TS 23.502 [6].

2.	The SMF performs an SM Policy Association Establishment/Modification, which include the corresponding S-NSSAI.

3.	The PCF instance creates and verifies the policy counter of a slice parameter, the number of successfully established PDU sessions, for a specific S-NSSAI. A policy counter per S-NSSAI indicates the local quota, local quota status, which comprises information of available maximum number of allowed PDU sessions and consumed number of PDU sessions. If the local value of a number of allowed PDU sessions for the specific S-NSSAI is still within a valid bound, there is no negative affect in PDU session establishment procedure.

4a.	Upon identification of no quota left or the secondary PCF has no local quota granted per operator's policy, the PCF instance shall decide for the delegation of quota enforcement and sends the request to the primary PCF. The delegation of quota enforcement message includes the S-NSSAI and its associated operation.

4b.	The primary PCF upon receiving the delegation message, verifies the status of the shared quota of the requested S-NSSAI and decides the acceptance or rejection based on the status of a shared quota of each requested S-NSSAI. The primary PCF manages the shared quota in the same way as the secondary PCF (i.e. by updating a policy counter).

4c.	The primary PCF sends a message to requesting PCF instance allowing such instance to be accepted or reject the PDU Session establishment. The response message includes an indication to accept or reject the PDU session establishment request for the indicated S-NSSAI.

5.	The policy counter value for the PDU session of the indicated S-NSSAI is updated (e.g., increase the associated policy counter value for the indicated S-NSSAI by one).

6.	The PCF instance send the response of quota enforcement, based on step 3 or step 4c, to the SMF instance.

NOTE:	PCF may also accept the UE request for a PDU session establishment although the quota has been exceeded. In this case, the PCF may include an indication of the quota exceed to the charging system.

Alt A: UE request for a PDU session establishment does not violate or exceed the local quota without delegation of quota enforcement, or the global quota with delegation of quota enforcement.

7.	PDU Session establishment procedure (steps 8-14) according to clause 4.3.2.2.1 in TS 23.502 [6] is executed.

Alt B: UE request for a PDU session establishment violates or exceeds the local quota without delegation of quota enforcement, or the global quota with delegation of quota enforcement

8.	UE request for a PDU session establishment is rejected.

6.7.3.3	PDU Session Release and Slice SLA Quota Update

Figure 6.7.3.3-1: UE requested PDU session release procedure with quota enforcement

UE is established for a PDU session associated with a specific S-NSSAI, which slice quota management is required.

1.	UE or network initiates a PDU session release procedure according to (steps 1-11) in clause 4.3.4.2 in TS 23.502 [6]. Then SMF invokes an SM Policy Association Termination procedure.

2.	During the SM policy association termination procedure as defined in clause 4.16.6 in TS 23.502 [6], the PCF instance of the UE decreases the policy counter value.

Optionally, if the PCF instance of the UE does not have local quota granted, e.g., as per operator's policy, the PCF instance delegates the request of policy counter update for the PDU session release of the S-NSSAI to the primary PCF. The primary PCF decreases the policy counter value of the indicated S-NSSAI and sends the response to the secondary PCF.

NOTE:	If the PCF instance has local quota granted before, during the PDU Session release procedure, the involvement of the primary PCF is not required. The quota re-distribution mechanism described in clause 6.1.3.4 can be used to recover the global quota at the primary PCF of a specific S-NSSAI.

3.	UE or network initiates a PDU session release procedure according to (steps 13-15) in clause 4.3.4.2 in TS 23.502 [6].

6.7.3.4	Controlling (re)-distribution of local quota of Slice SLA attributes

The procedure for the controlling (re)-distribution of local quota of slice SLA attribute on maximum number of allowed PDU sessions for a specific S-NSSAI is the same as per clause 6.1.3.4.

6.7.3.X	Controlling quota of Slice SLA attribute of Maximum Number of PDU sessions at Roaming

In LBO scenario for the Roaming UEs, the quota of the PDU session of a S-NSSAI can be controlled by the VPLMN.

In HR scenario, the interaction between V-PCF (i.e., SM-PCF) and the primary PCF in hPLMN shall take place.

6.7.3.X.1	Controlling quota of maximum number of PDU session for Roaming UEs by VPLMN

For the Roaming UEs, in LBO scenario, it is assumed that the local quota for the allowed number of PDU sessions for Roaming UEs per Subscribed S-NSSAI per home PLMN is available at vPLMN, e.g., as per SLA or Roaming agreement. The V-PCF, primary or secondary instance in vPLMN of the given S-NSSAI, enforces Roaming UEs’ requests for the PDU session establishment or release of the S-NSSAI and updates (i.e., increase or decrease) the policy counter of the S-NSSAI per home PLMN accordingly. The procedures of slice SLA quota enforcement and controlling for Roaming UEs for the allowed number of PDU sessions are the same as per Clauses 6.7.3.2 - 6.7.3.4 and with the additional HPLMN’s S-NSSAI information. That means for the same vPLMN only the indicated HPLMN’s S-NSSAI(s) need quota control.

6.7.3.X.2	Controlling quota of maximum number of PDU session for Roaming UEs by HPLMN

For the Roaming UEs, in HR scenario, the interaction between SM-PCF (i.e., secondary) and the H-PCF (primary) shall take place.

In this case, the procedure for a roaming UE’s request to a serving PLMN on a PDU session establishment or release of S-NSSAI is the same as the procedures for non-roaming UEs according to the steps 1-2 of clause 6.7.3.2 or the step 1 of clause 6.7.3.3, respectively. Then the interaction between SM-PCF (i.e., secondary) and the H-PCF (primary) shall take place for control and enforcement of the Roaming UEs’ requests, e.g., via N24. The response of the request to the roaming UE is handled by the serving PLMN as per steps 7-8 according to clause 6.7.3.2 for the PDU session establishment or as per step 3 according to clause 6.7.3.3 for the PDU session release, respectively.

6.7.4	Impacts on services, entities and interfaces

UDR: obtaining, storing and allowing retrieval of global slice SLA information including a global quota on the maximum number of allowed PDU sessions.

PCF: A primary PCF of a given S-NSSAI for distributing/providing the network slice related shared/local quota of the maximum number of PDU sessions and its secondary PCF to apply/enforce the network slice related local quota on the number of PDU sessions. When the quota is to be exceeded, PCF may reject or accept new PDU session establishment requests based on the quota status and operator policy.

SMF: handling of UE request for a PDU session establishment per the PCF decision.

UE: handling of back-off timer and a (new) cause value as a rejection response

6.7.X	Evaluation

The evaluation of the solution is described based on the following criteria.

- Scalability:

The solution allows a scalable solution to support a single instance or multiple instances of quota enforcement NFs without degradation in performance of the solution for slice SLA control and enforcement of the maximum number of PDU sessions parameter.

- Reusability:

The solution reusability for other Key Issues is very high. The PCF measurement based slice SLA control and enforcement is a generic solution which can apply not only for KI#2 but also for KI#1 as described in solution #1: PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter. In addition, the PCF based solution maintains the global quota status of a slice SLA attribute so that it can support the event notification of Network Slice quota of the slice SLA attribute to the consumer by covering KI#4.

In addition, the PCF based solution reuses the existing NFs and existing control plane procedures effectively and efficiently.

- Accuracy of Quota Control and Enforcement:

By introducing global quota, local quota and shared quota concept, the PCF based solution enables to control the slice SLA quota accurately. It means that the secondary PCFs control and enforce accurately UEs’ requests for a S-NSSAI locally based on its local quota. In a case of a local quota update (e.g., about to be consumed) or a delegation request (e.g., no remaining local quota) by a secondary PCF, the primary PCF controls and enforces accurately based on the shared quota and global quota status of slice SLA attribute.

By providing quota control and management mechanisms by primary/secondary PCFs, 5GS can overcome inaccurate quota control and enforcement. An example of inaccurate quota means a quota enforcement NF may accept the requests over the limit of the global quota as the quota enforcement NF do not receive the message of global quota has been consumed before the UEs’ requests have been accepted.

- Rapid Response of Quota enforcement:

The PCF based solution allows instantaneous decision for an acceptance or rejection of the UEs’ requests based on the status of local quota or shared quota or global quota of a slice SLA attribute during the PDU session establishment or release procedures. Since the rapid response of quota enforcement is an important requirement of the solution, to support this feature, the enforcement NF shall be aware of the actual number of remaining quota of a specific slice SLA attribute in a timely manner.

- Handling of Quota exceeding Policy:

The PCF based solution naturally supports the handling of slice SLA quota exceeding situation, exceptions or exemptions by allowing special policies, e.g., for handling of VIP customers or special DNNs or emergency services (e.g., MPS, MCX or PPDR).

- Signalling:

The PCF based solution has very low signaling load by efficiently utilizing the existing procedures (SM policy association establishment/modification and SM policy association termination procedure as defined in in TS23.502). If the local quotas are allocated to the secondary PCFs, the quota enforcement mechanisms by secondary PCFs do not require additional signaling for slice related quota management on maximum number of PDU sessions for a given S-NSSAI. It is also notable that the signalling due to the controlling (re)-distribution of local quota of Slice SLA attributes and the two steps enforcement scheme proposed by Solution #7 are infrequent events and can be negligible in relation to the performance of the solution.

# * * * * End of changes * * * *