3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-2005650

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Evaluation on key issue 1

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This contribution proposes evaluations on solutions addressing key issue 1.

1. Discussion

There are several solutions to address key issue 1 documented in TR 23.752 and are mostly complete. Among all the solutions, the fundamental issue discussed is how ProSe code is allocated.

Two main directions can be found in TR 23.752:

- ProSe code allocated via application layer mechanism;

- ProSe code allocated by DDNMF in the 5G network;

Application layer mechanism mean the ProSe application server allocates the code and sends to UE in the application layer. This approach has the following drawbacks:

- It cannot guarantee the codes allocated by different application do not collide. There is no “central” control node to manage the code so it is likely the code allocated by app A collide with the one by app B;

- Code format is not defined by 3GPP. When announcing UE announce the code, the format especially the length of the code matters the design the physical layer of UE. If the code is very long, longer than the allowed length of a single message, the code will be decomposed into multiple discovery message.

- Impact on ProSe service business offered by operator. ProSe code is for announcing and monitoring on PC5 interface by the ProSe-enabled UE. It is a type of network resources thus should be controlled by the 5G network. DDNMF is the network function responsible for ProSe code management, e.g. allocation and charging. If allocation is performed by application layer mechanism, the network will not be able to collect the information of UE ProSe discovery service.

Proposal 1: ProSe code is allocated by the DDNMF in the 5G network.

Regarding UE-DDNMF interaction, it is the debate on ProSe architecture. Following aspects can be used to compare the pros and cons of the CP based or UP based DDNMF architecture.

|  | CP based solution | UP based solution |
| --- | --- | --- |
| Complexity on solution in general | Does not require UE to establish user plane connection No ProSe proxy function need | User plane between UE and DNMF is a must Have to deploy ProSe proxy function |
| Compliant for 5GC arch | DDNMF is by function an operator-control AF, it allows the operator to allocate network resources and charge UE accordingly. DDNMF has interface with UDM, PCF, CHF | Very rare case for a 5GC NF to communicate with UE via user plane, usually via NAS in a container. P-CSCF is not service based. |
| Support for other KI | Solution 18 shows how UP based arch work for charging key issue. DDNMF interacts with CHF like for other service | Another NF “ADF” is needed for charging support, which is separate from DDMNF, and unclear how ADF knows the prose discovery traffic volume without interaction with DDNMF. |

It is observed that CP-based solution has the same signalling cost but less system impact.

Proposal 2: CP-based DDNMF architecture is the architecture selected as a way forward.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.752.

# * * * * First change * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.1	Evaluation on key issue #1

Following two aspects are for evaluation on all solutions addressing key issue#1:

- ProSe code allocation;

- UE-DDNMF interaction;

ProSe code is for announcing and monitoring on PC5 interface by the ProSe-enabled UE. It is a type of network resources thus should be controlled by the 5G network. DDNMF is the network function responsible for ProSe code management, e.g. allocation and charging. If allocation is performed by application layer mechanism, the network will not be able to collect the information of UE ProSe discovery service.

Proposal 1: ProSe code is allocated by the DDNMF of 5G network.

Regarding UE-DDNMF interaction, it is the debate on ProSe architecture. Following aspects can be used to compare the pros and cons of the CP based or UP based DDNMF architecture.

|  | CP based solution | UP based solution |
| --- | --- | --- |
| Complexity on solution in general | Does not require UE to establish user plane connection No ProSe proxy function need | User plane between UE and DNMF is a must Have to deploy ProSe proxy function |
| Compliant for 5GC arch | DDNMF is by function an operator-control AF, it allows the operator to allocate network resources and charge UE accordingly. DDNMF has interface with UDM, PCF, CHF | Very rare case for a 5GC NF to communicate with UE via user plane, usually via NAS in a container. P-CSCF is not service based. |
| Support for other KI | Solution 18 shows how UP based arch work for charging key issue. DDNMF interacts with CHF like for other service | Another NF “ADF” is needed for charging support, which is separate from DDMNF, and unclear how ADF knows the prose discovery traffic volume without interaction with DDNMF. |

It is observed that CP-based solution has the same signalling cost but less system impact.

Proposal 2: CP-based DDNMF architecture is the architecture selected as a way forward.

# * * * * Second change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	Interim conclusions to key issue 1

- ProSe code is allocated by the DDNMF of 5G network

- CP-based DDNMF architecture is the architecture selected as a way forward.

# * * * * End of changes * * * *