3GPP TSG-SA WG2 Meeting #140e	S2-2005714

19 August - 01 September 2020, eMeeting

Source:	Lenovo, Motorola Mobility

Title:	Conclusion on solutions introducing a data collection function for Key Issue #11

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution:.

# Discussion

Solutions 9, 15 and 35 have similarities as all introduce a separate NF for data collection. The main advantage of introducing a data collection function is that it reduces the load at the NWDAF for managing multiple Event based subscriptions and tracking an NF serving a UE.

The following Figure provides a summary of these solutions.

Figure 1 - Summary of solutions proposing a new NF for data collection coordination (Solutions 9, 15, 35)

The main difference in these solutions is on the method for a Data Producer (e.g. an AMF) to report Events (data) to a Data Consumer (e.g. NWDAF) where in Solutions 15 and 35 it is proposed that the data are provided based on service based APIs whereas in Solution 9 the data are provided with a messaging framework that is defined outside of 3GPP.

It is proposed to conclude for Key Issue #11 to introduce a new NF for data collection and use Solutions 9, 15 and 35 as a basis for normative work. It is also proposed that during normative work to be evaluated whether a Data Producer provides the Events (data) to a Data Consumer using a 3GPP based service API or by using a messaging framework defined outside the scope of 3GPP.

# Proposal

It is proposed to include the following as conclusion for Key Issue #11 the eNA TR.

********************************  START OF CHANGES **************************************

# 8	Conclusions

For Key Issue #11:

It is proposed to introduce a dedicate NF for data collection. Solutions 9, 15 and 35 are to be used as a basis for normative work. During normative work it will be evaluated whether a Data Producer provides the Events (data) to a Data Consumer using a 3GPP based service API or using a messaging framework defined outside the scope of 3GPP.

******************************** END OF CHANGES **************************************