SA WG2 Meeting #140	S2-2005742

19 August - 02 September 2020

Source:	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, NTT Docomo

Title:	KI#3B, Sol#9: Updates on Solution #9 for Time Synchronization

Document for:	Discussion/Approval

Agenda Item:	8.10

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution updates solution #9 for Time Synchronization

# 1		Discussion

This contribution updates Key Issue #3B solution #9 for Time Synchronization.

# 2		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

## 6.9	Solution #9: (g)PTP GM support by DS-TT

### 6.9.1	Introduction

Figure 6.9.1-1: DS-TT distributes 5GS time to devices attached to DS-TT using (g)PTP

This solution addresses KI#3B. As illustrated in Figure 6.9.1-1, key idea of this solution is that DS-TT acts as a (g)PTP grand master (GM) for devices attached to DS-TT. The time information used in (g)PTP is based on the time information received from the gNB as defined in TS 38.331 [11].

### 6.9.2	Functional Description

The solution is based on the following principles

-	DS-TT indicates to the network inside a Port Management Information Container (PMIC) whether DS-TT is capable of acting as (g)PTP GM and which version(s) it supports

-	IEEE 802.1AS [6] (i.e. gPTP), and/or

-	PTP over UDP/IPv4 as per IEEE 1588-2008 [9] Annex D, and/or

-	PTP over UDP over IPv6 as per IEEE 1588-2008 [9] Annex E.

-	If DS-TT has indicated that it is capable of acting as (g)PTP GM, then the network may request the DS-TT to generate (g)PTP messages to devices attached to the UE by providing (g)PTP parameters (incl. (g)PTP version, domain number(s), sending rate, etc.) and an instruction to start sending (g)PTP messages inside PMIC to the DS-TT.

NOTE:	DS-TT can be configured to support different (g)PTP profiles, e.g. SMPTE ST 2059-2 [9] or IEC/IEEE 61850-9-3 [10] by providing the (g)PTP parameters defined for those profiles to DS-TT.

### 6.9.3	Procedures

For IEEE TSN integration, i.e. if TSN AF is deployed, TSN AF controls the gPTP functionality in DS-TT using existing Port Management Information Container (PMIC) signaling between DS-TT and TSN AF.

Otherwise (i.e. for IP or Ethernet PDU Sessions not part of an IEEE TSN network), the NEF terminates the PMIC signaling instead of TSN AF. To achieve this, NEF generates PMIC signaling to control the (g)PTP client in DS-TT based on time synchronization API requests from an AF and sends PMIC to DS-TT using existing Rel-16 PCF functionality. Similarly, NEF receives PMIC from PCF using Rel-16 functionality.

It is assumed that a single NEF is handling all time synchronization API requests for a given UE.

The following figure illustrates an overview of the procedure to activate (g)PTP grandmaster functionality in DS-TT.

Figure 6.9.3-1: Procedure for synchronizing (g)PTP clients attached to DS-TT with the 5G clock

### 6.9.4	Impacts on services, entities and interfaces

DS-TT:

-	Optionally send (g)PTP capabilities; receive (g)PTP parameters.

-	Optionally generate (g)PTP messages based on network request.

NEF:

- Support Rel-16 PMIC signaling via PCF

*** End Change ***